Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, https://www.hanwhavision.com, https://hanwhavisionamerica.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

Model: BRR-X3210A4E

Alternates: BRR-P3210A4E, BRR-P6410A2E, BRB-X3210E, BRB-X1610E, BRB-X1610SE, BRB-X3210S1E

GENERAL DESCRIPTION

The BRR-X3210A4E (“BLAZE”) is a 32-channel BLAZE Appliance that shall record video and audio from network video cameras at up to 250Mbps to local hard disk storage, with BLAZE VMS Client and Server pre-installed along with 32 BLAZE Premium Licenses.

The BLAZE uses a 12th generation Intel® Core-S Pentium processor and operates on a Hardened OS, providing enhanced security and resistance to cyber threats.

The manufacturer shall be responsible for rigorous testing of the BLAZE’s reliability. The manufacturer shall provide a list of compatible hard disks that have been tested to guarantee reliable recording. The list shall be available on the manufacturer’s website.

The BLAZE shall provide a local monitoring environment for video and audio using a connected displays via Full HD HDMI and 4K HDMI outputs.

Camera search and discovery – The BLAZE shall have the following properties relating to camera registration:

The NVR shall have the capability to search the network for connected compatible cameras via ONVIF Profile S or Hanwha Vision’s native driver.

When cameras are searched or discovered, each camera will be automatically registered and current camera information displayed.

When cameras are searched or discovered, the NVR shall provide the ability to selectively register as many cameras as can be found.

Recording and Playback – The BLAZE shall have the following recording and playback capabilities:

Support recording from network cameras up to 250Mbps total bandwidth across 32 channels.

H.265, H.264, and MJPEG compression support.

Simultaneous playback capability up to 32 video channels.

Set recording schedules (continuous and scheduled recording).

Set up triggered recording based on: camera event/analytics, motion detection, and video loss detection.

Advanced forensic tools shall include Object Search, Similarity Search, Histogram analysis, and Event search.

The system shall support 'Edge Storage Backup' to automatically recover video data from camera SD cards after network disruptions.

Available actions upon reaching full HDD storage capacity: stop recording or overwrite.

Storage – The BLAZE shall have the following storage properties:

16 SATA HDD bays, supporting up to 10TB per drive, with a total maximum capacity of 160TB.

USB connection for memory/storage device for video clip backup and settings export.

Live View – The BLAZE shall provide the following live view capabilities:

Local live monitoring via HDMI.

Live streams: H.265, H.264, MJPEG.

Configure and exercise functions for connected PTZ cameras.

Capture and save snapshot images.

Remote Access – The BLAZE shall provide the following remote access capabilities:

Multicast or unicast streaming with unlimited simultaneous unicast and multicast access.

The system shall support Hanwha Vision Cloud portal integration for centralized multi-system management, Single Sign-On (SSO), and browser-based access via BLAZE Web Client.

Dynamic DNS (DDNS) support.

High Definition (HDMI) local monitor outputs for live viewing, playback, and backup functions.

ONVIF Profile S compliance.

This product is compliant with the National Defense Authorization Act (NDAA), Section 889, and does not contain any covered telecommunications equipment or services as defined by the Act.

OPERATIONS

The BLAZE shall provide local monitoring, playback, and configuration interfaces.

Live monitoring with multi-channel layouts, PTZ control, digital zoom, audio control (on/off/mute), and snapshot capture

Video search by time, date, event type, camera channel, or other filtering criteria

Playback with forward/reverse, variable speed, frame-by-frame navigation, and timeline scrubbing

Video backup and export.

Camera discovery, registration, and configuration (resolution, frame rate, codec, bitrate, image settings, day/night mode, motion detection, video analytics).

Recording schedule and profile configuration per channel (continuous or scheduled recording).

Two-way audio communication shall be supported via USB microphone input.

Storage management options (overwrite when full or stop recording).

Device network settings and user account management.

System information, status monitoring, and firmware upgrade.

System and event log access.

Storage status, capacity monitoring, and HDD health alerts.

Event configuration (video loss, motion detection, video analytics, audio detection, dynamic event support).

Event notification setup and event log search.

DETAILED SPECIFICATIONS

System

Video Management Software: BLAZE VMS

Licenses: 32 x BLAZE Premium License

Maximum Channels: Up to 32 channels

Processor: 12th Intel Core-S Pentium

Memory: DDR4 16GB

Operating System: Hardened OS

Video Output: 1 x HDMI (4K), 1 x HDMI (FHD)

USB Ports: 4ea (Front 2 x USB 2.0, Rear 2 x USB 3.0)

Audio Output: 1 x Audio output

Audio Communication: 2-Way (Audio In: USB Mic)

Keyboard & Mouse: Keyboard, Mouse

Video

Compression: H.265, H.264, MJPEG

Recording

Channel Capability: Up to 32 channels

Recording Bandwidth: Up to 250Mbps

Events and Response Actions

Triggers:

Motion detection

Video loss

Event defined by camera

Alarm input via network camera.

Response Actions:

Record

E-mail notification

Activate PTZ preset

Sound output

Playback

Number of Simultaneous Channels: Up to 32 channels

OS Drive

OS Drive: SSD 128GB (internally mounted)

Storage

Internal:

Number of HDD bays: 16

Capacity per HDD: Up to 10TB

Total maximum capacity: 160TB

External:

USB HDD/Flash drive for backup of video clips, firmware update, settings backup/restore, log export

Network

Connectivity: 1000 Base-T Ethernet, 3 x RJ-45 (1Gbps) connectors

Protocols Supported: TCP/IP, UDP, IPv4, IPv6, HTTP, HTTPS, DHCP, DNS, NTP, SNMP, UPnP, RTSP, RTP, RTCP, IGMP, SMTP, PPPoE, ARP

Security Features:

User password protection with group restrictions

IP address filtering (allowed/blocked list)

HTTPS (SSL) login authentication

User access log

802.1x authentication

Audio

Direction: Bi-directional (2-Way, Audio In: USB Mic)

Output: 1 x Audio output

Environment

Operating Temperature: 0°C to +40°C (32℉ to 104℉)

Operating Humidity: 20% ∼ 85% RH (non-condensing)

Electrical

Power Input: 100 ∼ 240 VAC ±10%; 50/60 Hz

Power Consumption: 220W (750.7 BTU)

Mechanical and Environmental

Form Factor: 3U rack

Color / Material: Black / Metal

Dimensions (Width x Height x Depth): 440 x 132 x 571.1 mm (17.32″ x 5.20″ x 22.48″)

Weight: 14.1Kg (31.1lbs), excluding HDD

Certifications & Standards

EMC

North America: FCC 47 CFR 15 Subpart B Class A, ICES-003 Issue 7 Class A

Europe: CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3, EN IEC 63000

Asia-Pacific: VCCI-CISPR 32 Class A (Japan), RCM AS/NZS CISPR 32 Class A (Australia/New Zealand)

Korea: KS C 9832 Class A, KS C 9835

Safety

Global: CB Scheme (IEC 62368-1, EN IEC 62368-1)

North America: UL 62368-1, CAN/CSA C22.2 NO. 62368-1

Korea: KC 62368-1

Security & Compliance

Security Certificate: FIPS 140-3 Level 3

Regulatory Compliance: NDAA Section 889

Environmental: RoHS (Directive 2011/65/EU)

END OF SECTION

8