MODEL : 8채널 AI Box (AIB-800)

1. 개 요

AIB-800은 8채널 AI Box 입니다.

2. 제 원

- 비디오 입력  : 8채널, CIF ~ 32MP, 프로토콜 : SUNAPI, ONVIF

- AI 분석 기능

Classified object type : Person/Vehicle

Attributes : Vehicle

(Type:car/bus/truck/motorcycle/bicycle)

Support Bestshot

Analytics events based on AI engine:

- Motion Detection*, Object detection, Virtual line*

(Crossing/Direction),

Virtual area* (Loitering/Intrusion/Enter/Exit)

* Some of the video analytics only works with

people/vehicle detection

Analytics events : Virtual area(Appear/Disappear)

Business Intelligence :

- Based on AI engine: People counting, Vehicle counting,

Queuemanagement, Heatmap

- 프로토콜     : TCP/IP, UDP/IP, RTP (UDP), RTP (TCP), RTSP, NTP, HTTP, DHCP (Server, Client),

SMTP, ICMP, IGMP, ARP, DNS, DDNS, uPnP, HTTPS, SNMP, ONVIF (Profile-S),

SUNAPI(Server, Client), SRTP

- DDNS         : Hanwha DDNS

- 최대 조작 스트림 : 80 Streams ( 10 Streams per 1CH)

- 지원언어    : English, French, German, Italian, Spanish, Russian, Turkish, Polish, Dutch,

Czech, Portuguese, Hungarian, Greek, Korean, Chinese, Japanese

- 운영체제    : Windows 11/10, macOS 10.13

- 지원 웹브라우저 : Google Chrome, Microsoft Edge, Mac Safari

- 뷰어 SW       : WebViewer, WAVE, SSM, Genetec, Milestone

- CMS 지원   : Support SDK/CGI(SUNAPI) for interworking with 3'rd party VMS

- 인터페이스

- 인디케이터  : LED(Status indicator) : - Alarm(1), Power(1), Network(1)

- 이더넷      : RJ-45 1ea (1Gbps)

- 알람        : (1) In(8EA, Terminal Block)

(2) Alarm Reset(1EA)

(3) Out(4EA, Terminal Block)

-> Relay Out1(NO/NC/COM)

-> Relay Out2~4(NO/COM)

- USB           : 1EA( Rear 1 x USB 3.0) for DEBUG

- Serial          : Not Supported (internal Debug)

- Reset          : Factory Reset

- 전원코드       : DIN 4P 12V, 4A

12) CPU            : NVIDIA ORIN NX 8GB (SOM)

13) 작동 온도/습도  : +0°C to +40°C(+32℉ to +104℉) / 20% ~ 85% RH

14) 전원            : 100 ~ 240 VAC ±10%; 50/60 Hz, 12V, 4A Adaptor, 28W

15) 사이즈/무게     : 300 x 47.6 x 240.9 mm (11.81 x 1.87 x 9.48 inch) / 1.39kg

http://www.hanwhavision.com/