Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, https://www.hanwhavision.com, https://hanwhavisionamerica.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

Model: CGR-3210B1

Alternates: CGR-1610

GENERAL DESCRIPTION

The Cloud Gateway Recorder (“CGR”) shall record video and audio from network video cameras at up to 250Mbps to a local hard disk storage and connect to Hanwha OnCloud services for remote playback, monitoring, and device configuration.

The CGR uses 12th generation Intel® processor.

The manufacturer shall be responsible for rigorous testing of the CGR’s reliability. The manufacturer shall provide a list of compatible hard disks that have been tested to guarantee reliable recording. The list shall be available on the manufacturer’s website.

The CGR shall provide cloud-based remote monitoring and management environment. All live monitoring, recorded video playback, search for recorded videos, and configure devices through cloud services accessible from web browsers.

Dynamic event: The event function of cameras is available without CGR software (firmware) update.

The CGR shall have the following further general properties

Local Recording Functions

Record video and audio from network cameras to local HDD storage

Support up to 32 channels of recording

Maximum recording bandwidth of 250Mbps

Support continuous, and scheduled recording

Storage

Four (4) SATA HDD bays supporting up to 16TB per drive

Total maximum storage capacity of 64TB

Local storage management and monitoring

Cloud-Based Management and Monitoring:

All live monitoring performed exclusively through Hanwha OnCloud platform

All video playback and search functions performed through cloud services

All camera discovery, registration, and configuration performed through cloud interface

All device settings and configuration performed through cloud interface

Search recorded data by time, event, or other criteria through cloud interface

Remote access from web browsers and mobile devices via cloud services

Secure connection through Hanwha Vision cloud infrastructure

Camera Integration

Support for Hanwha IP cameras and compatible third-party cameras

Camera search and discovery through cloud interface

Basic ONVIF compliance for third-party camera integration.

Network Connectivity

Two Gigabit Ethernet ports (10/100/1000 BASE-T)

WAN port for cloud connection

LAN port for camera connection

CLOUD-BASED OPERATIONS

The CGR does not provide local monitoring, playback, or configuration interfaces.

All user operations must be performed through Hanwha OnCloud services.

Cloud Platform Features

Live monitoring with multi-channel layouts, PTZ control, digital zoom, audio control (on/off/mute), and snapshot capture

Video search by time, date, event type, camera channel, or other filtering criteria

Playback with forward/reverse, variable speed, frame-by-frame navigation, and timeline scrubbing

Video backup and export

Camera discovery, registration, and configuration (resolution, frame rate, codec, bitrate, image settings, day/night mode, motion detection, video analytics)

Recording schedule and profile configuration per channel (continuous, or scheduled recording)

Audio recording enable/disable

Storage management options (overwrite when full / stop recording)

Device network settings and user account management

System information, status monitoring, and firmware upgrade

System and event log access

Storage status, capacity monitoring, and HDD health alerts

Event configuration (video loss, motion detection, video analytics, audio detection, AI metadata with Wisenet AI cameras, dynamic event support)

Event notification setup and event log search

AI search by object attributes with Wisenet AI cameras

DETAILED SPECIFICATIONS

Performance

Operating System: embedded Linux

Recording Bandwidth: Maximum 250 Mbps

Storage

Supported HDD: Up to 16TB per drive

HDD slot: SATA 4ea (Maximum 64TB total capacity)

Interface

Front indicator: Power, Network, HDD1, HDD2, HDD3, HDD4

HDMI: 1ea (Reserved)

DP: 1ea (Reserved)

Audio output: 1ea (Reserved)

Ethernet: RJ-45 2ea (10/100/1000 BASE-T)

WAN: External network (Cloud) connection

LAN: Camera connection

USB: 4ea (Front 2 x USB 2.0, Rear 2 x USB 3.0)

Power inlet: AC 1ea

Environmental

CPU: Quad core

RAM: DDR4 16GB

OS HDD: eMMC 32GB

Operating Temperature: 0°C to +40°C (32℉ to 104℉)

Operating Humidity: 20% ~ 85% RH (Relative Humidity

Electrical

Power input: 100 ~ 240 VAC ±10%; 50/60 Hz

Power Consumption: 80W

Mechanical

Color / Material: Black / Metal

Dimensions (Width x Height x Depth): 440 x 90 x 430 mm (17.3 x 3.5 x 16.9 inch)

Weight: 5.5Kg (12.2lb), excluding HDD

Certification

Emissions & RoHS

FCC EMC – FCC Part 15, Subpart B & ICES-003 (Issue.7)

CE EMC/RoHS – Council Directives 2014/30/EU, Regulations 2016/1091

EN 55032:2015/A11:2020

EN 50130-4:2011/A1:2014

EN 61000-3 2:2014

EN 61000-3 3:2013

EN IEC 63000: 2018

UKCA EMC/RoHS – S.I. 2016:1091, S.I. 2012:3032

BS EN 55032:2015/A11:2020

BS EN 50130-4:2011/A1:2014

BS EN 61000-3 2:2014

BS EN 61000-3 3:2013

BS EN IEC 63000: 2018

VCCI-CISPR 32:2016

RCM- AS/NZS CISPR32:2015, CISPR32 Ed2.0

Safety

UL Listed (E158873-A6225)

IEC 62368-1:2018, EN IEC 62368-1:2020, EN IEC 62368-1:2020/A11:2020

Compliance

NDAA Compliant

FCC Certified

END OF SECTION

8