Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

2MP 3CH MULTI-DIRECTIONAL NETWORK CAMERA

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

MasterFormat 2016:

28 20 00	Video Surveillance

28 2x xx	Surveillance Cameras

28 2x xx	IP Cameras

# Related Requirements:

MasterFormat 2014:

27 20 00	Data Communications

28 23 13	Video Surveillance Control and Management Systems

28 23 16	Video Surveillance Monitoring and Supervisory Interfaces

28 23 19	Digital Video Recorders and Analog Recording Devices

28 23 23	Video Surveillance Systems Infrastructure

MasterFormat 2016

27 15 01.xx	Video Surveillance Communications Conductors and Cables

27 20 00 	Data Communications

28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

28 05 xx	Storage Appliances for Electronic Safety and Security

28 05 xx.xx	Network Video Recorders

28 05 xx	Cyber Requirements for Electronic Safety and Security

28 05 xx	Safety and Security Network Communications Equipment

28 2x 00	Video Management System

2MP 3CH MULTI-DIRECTIONAL NETWORK CAMERA

- GENERAL

  - SUMMARY

    - Section includes a 2MP 3Ch Multi-Directional (omnidirectional) camera with a 360 degree field of view in vandal proof dome housing.

    - B.	Product - A 2MP 3Ch Multi-Directional IP video camera with multi-streaming (H.265, H.264 and MJPEG) capability and variable view modes including panorama and quad view

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC		Auto Gain Control

      - AES		Advanced Encryption Standard

      - API		Application Programming Interface

      - ARP		Address Resolution Protocol

      - AWB		Auto White Balance

      - BLC		Back light compensation

      - CBR		Constant Bit Rate

      - CVBS		Composite Video Baseband Signal

      - DHCP		Dynamic Host Configuration Protocol

      - DNR		Digital Noise Reduction

      - DNS		Domain Name Server

      - DDNS		Dynamic Domain Name Server

      - DSCP		Differentiated Services Code Point

      - fps		frames per second

      - FTP		File Transfer Protocol

      - GOV		Group of Video

      - GUI		Graphical User Interface

      - HD		High Definition

      - HTTP		Hypertext Transfer Protocol

      - HTTPS		Secure HTTP

      - ICMP		Internet Control Message Protocol

      - IGMP		Internet Group Management Protocol

      - IP		Internet Protocol

      - IR		Infrared

      - JPEG		Joint Photographic Experts Group

      - LAN 		Local Area Network

      - LED		Light Emitting Diode

      - LDC		Lens Distortion Correction

      - LLDP		Link Layer Discovery Protocol

      - LPR		License Plate Recognition

      - MJPEG		Motion JPEG

      - MP		Megapixel

      - MPEG		Moving Pictures Experts Group

      - NAS		Network Attached Storage

      - NTP		Network Time Protocol

      - NVR		Network Video Recorder

      - PIM-SM	Protocol Independent Multicast-Sparse Mode

      - PoE		Power over Ethernet

      - PPPoE		Point to Point Protocol over Ethernet

      - QoS		Quality of Service

      - RTP		Real-Time Transport Protocol

      - RTCP		Real-Time Control Protocol

      - RTSP		Real-Time Streaming Protocol

      - SDK		Software Development Kit

      - SFP		Small Form factor Pluggable

      - SMTP		Simple Mail Transfer Protocol

      - SNMP		Simple Network Management Protocol

      - SSDR		Super Smart Dynamic Range

      - SSNR		Super Smart Noise Reduction

      - SSL		Secure Sockets Layer

      - TCP		Transmission Control Protocol

      - UDP		User Datagram Protocol

      - UPnP		Universal Plug and Play

      - VBR		Variable Bit Rate

      - VMS		Video Management System

      - WDR		Wide Dynamic Range

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

        - IPv4 IP addressing version 4

        - IPv6 IP addressing version 6

        - QoS Quality of Service

      - Video

        - ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profiles S, G, T

      - EMC & Safety

        - FCC 47 CFR Part 15 Subpart B

          - ANSI C63.4-2014 Class A

        - IC Regulation ICES-003:2016

          - ANSI C63.4-2014 Class A

        - CE EMC-Directive 2014/30/EU

          - EN 55032:2015 Class A

          - EN 50130-4:2011+A1:2014

        - VCCI-CISPR 32: Class A

        - AS/NZS CISPR32:2015 Class A

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

      - Ingress Protection and Vandal Resistance

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures – IP66

        - IEC EN 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts : IK10

        - IEC 60068-2-75 : IK10

        - NEMA 4x

    - Definitions

      - GOV (Group of Video object planes) - A set of video frames for H.264 and H.265 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of 2 kinds of frames: I-Frame and P-Frame.

      - Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Multi-exposure wide dynamic range - Operation which automatically adjusts shutter speed to provide a wide range between dark and light areas visible at the same time, preventing backlighting issues. Long exposure is used for dark areas and a short exposure is used in light areas.

      - Dynamic fps – Dynamic assignment of fps (frames per seconds) based on the movement of object(s) in the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - WiseStream Ⅱ – Technology that controls quantization parameter, fps, and GOV length in H.265 and H.264 to efficiently manage bitrate of the video stream and reduce the storage required.

      - DORI (Detect, Observe, Recognize, Identify) – A standard system (EN-62676-4) for defining the ability of a camera to distinguish persons or objects within a covered area.

        - Detect : 25PPM / 82PPF

        - Observe : 62.5PPM / 205PPF

        - Recognize : 125PPM / 410PPF

        - Identify : 250PPM / 820PPF

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment specified in section 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide at least a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded camera video analytics free of license charges..

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://hanwha-security.com

    - Model		PNM-8082VT

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - The camera shall provide multi directional view and produce video in various view mode.

    - Hallway view is available on CH1/2/3.

    - Each Channels support SD Card

    - It provides one single power supply for all of multi-channels

    - Video Compression and Transmission – The multi-directional camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

      - H.265 and H.264 – maximum of 30/25fps(60Hz/50Hz) at all resolution

      - MJPEG – maximum of 15/12fps(60Hz/50Hz)

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - 16:9 aspect ratio : 	1920x1080, 1280x720, 800x448, 640x360

        - 4:3 aspect ratio : 	1280x960, 1024x768, 800x600, 640x480, 320x240

        - 5:4 aspect ratio : 	1280x1024, 720x576

        - 3:2 aspect ratio : 	720x480

      - The camera shall support unicast video streaming up to 20 users.

      - The camera shall support multicast video streaming up to 10 profiles.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStream Ⅱ, Dynamic GOV and Dynamic fps to efficiently manage bit rate of the video stream and reduce storage.

    - Camera – The multi-directional camera device shall have the following physical and performance properties:

      - IK10 rated for protection against impacts.

      - True day/night operation with scheduling and options for external devices.

      - Low light level operation to 0.04 lux at F2.2 in color mode and 0.004 lux at F2.2 in black and white mode.

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range.

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 32 privacy masking areas with polygonal zones.

      - The camera shall have the defog feature to remove fogginess of scene which can be triggered automatically from the fog detection event.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, rotation and focus.

    - Intelligence and Analytics – The multi-directional camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

      - Detection of logical events of specified conditions from the camera’s video

        - Defocus detection

        - Directional detection

        - Motion detection

        - Enter/Exit

        - Tampering

        - Virtual line

    - Interoperability – The camera shall be ONVIF Profile S, G and T compliant.

    - The multi-directional camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including Chrome, MS Edge, Firefox and Safari (Mac. OS 10.12,13,14).

      - Micro SD/SDHC/SDXC memory card options, with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - Analytics

          - Network disconnect

        - available notification means upon trigger:

          - File Upload via FTP and E-mail

          - Notification via E-mail

          - Local storage (SD / SDHC / SDXC) or NAS recording at event triggers

      - PoE+ capable

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

  - CAMERA SOFTWARE

    - The multi-directional camera shall have a built in web server which supports browser-based configuration using Google Chrome, Mozilla Firefox, and Apple Safari from a PC or Mac.

    - The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Manual recording to SD or NAS

      - Access recorded data playback and camera configuration menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Recorded data search using date and time range

      - Recorded data search using event type

      - Play a recorded video by event triggering

      - Set resolution

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, crop encoding area

      - User profile to include password, access level, authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - IP Address Filtering

        - SSL/TLS, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

        - Auto IP Configuration

      - Video setup

        - Flip / mirror mode

        - Video output type

        - Privacy zone

      - Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, sharpness, contrast, and color level.

      - Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection, event triggers

      - System function to include reboot, upgrade, check system and event logs, application (SDK) management

      - View profile information

    - Client requirements

      - Recommend Browser : Chrome

      - Acceptable Browser : Chrome, Safari, Firefox, MS Edge(chromium based)

      - Acceptable Operating Systems:	Windows, MAC, Android, iOS, Chrome

      - Verified Environment:

        - Windows10 : Google chrome version 80 above, Firefox version 72 above,

MS Edge version 83 above

        - Mac 10.13/14 : Safari version 11.0.1 above

- Decoding performance in web viewer depends on CPU/GPU performance of user

  - DETAILED SPECIFICATIONS

    - Video

      - Imager(optional 4 different Lens)

        - Sensor:			1/2.8" 2M CMOS: each CH

          - pixels per sensor:	1,945 (H) x 1,097 (V) effective: each CH

          - scanning	:	progressive

        - Minimum illumination

          - Color mode: 		0.04lux (F2.2, 1/30sec)

          - Black & white mode: 	0.004lux (F2.2, 1/30sec)

    - Lens

      - Focal length			3 ~ 6mm (2x) Motorized Varifocal

      - Max. Aperture Ratio		F2.2(Wide) ~ F3.1(Tele)

      - Field of View			[Wide] H: 107˚,     V: 57°,    D: 126°

[Tele]  H:  56.3˚,   V: 31.5°,    D:   64.3°

      - Min. Object Distance		0.5m (1.64ft)

      - Lens Type			DC Auto Iris

      - Focus Control			Simple Focus

      - Mount Type			Board-in Type

    - Pan & Tilt & Rotate

      - PTR Range			0˚~360˚ / 0˚~73˚ / 0˚~90˚

    - Operational Functions

      - Camera Title 			Off / On (Displayed up to 85 characters per line)

        - W/W 			English / Numeric / Special characters

        - China 			English / Chinese / Numeric / Special characters

        - Common			Multi-line (Max. 5), Color (Grey/Green/Red/Blue/Black/White),

Transparency, Auto scale by resolution

      - Day/night setting:		Auto (ICR) / Color / B/W / Schedule

      - Backlight compensation (BLC):	BLC / WDR / HLC / SSDR

      - WDR				extreme WDR (150dB)

      - Digital Noise Reduction (DNR):	Support (SSNR Ⅴ)

      - Defog				Support

      - Motion Detection		Off / On (8ea, 8point polygonal zones)

      - Privacy Masking		Off / On (32ea, 4point polygonal  zones)

        - Color:			Gray / Green / Red / Blue / Black / White

Mosaic

      - Gain Control			Low / Middle / High

      - White Balance 			ATW / AWC / Manual / Indoor / Outdoor

      - Electronic shutter speed: 	Minimum / Maximum / Anti flicker (2 ~ 1/12,000 sec)

      - Image Rotation			Flip, Mirror

Hallway view: 0˚ / 90˚ / 270˚

      - Alarm I/O			None

      - Alarm Triggers			Analytics, Network Disconnection

      - Alarm Events			File Upload via FTP and E-mail, Notification via E-mail,

SD/SDHC/SDXC or NAS recording at event triggers, Handover

      - Analytics			Defocus detection, Directional detection, Motion detection,

Enter/Exit, Tampering, Virtual line

      - Video Out (Installation)		CVBS: 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P)

      - Memory			4GB(LPDDR4), 512MB(NAND)

      - DORI Distance

        - Detect			28.4m ~ 71.8m (11.18ft~28.27ft)

        - Observe			11.4m ~ 28.7m (4.49ft~11.30ft)

        - Recognize			5.7m ~ 14.4m (2.24ft~5.67ft)

        - Identify			2.8m ~ 7.8m (1.10ft~3.07ft)

    - Video Streams

      - Encoding Type 			H.265, H.264, MJPEG

      - Resolution			1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768,

800x600, 800x448, 720x576, 720x480, 640x480,

640x360, 320x240

      - Maximum frame rate:

        - H.265 and H.264:		30 fps (60Hz) / 25 fps (50Hz) at all resolutions

        - MJPEG: 			30 fps (60Hz) / 25 fps (50Hz) at all resolutions

      - Smart Codec			WiseStreamⅡ, Dynamic GOV, Dynamic fps

      - Bit rate control method:

        - H.265 and H.264

          - target bitrate level control

          - constant bit rate (CBR) or variable bit rate (VBR)

        - MJPEG

          - target bitrate level control

          - variable bit rate (VBR)

      - Number of multi-streaming profiles:	10 maximum

      - Simultaneous users (total):		20 maximum (unicast)

      - Storage and Recording

        - The multi-directional camera shall have onboard SD card storage for each sensor.

          - Card type:		Micro SD/SDHC/SDXC

          - Capacity:		2slot 512GB(Each CH)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S / G / T.

      - Single Images - The camera shall support png file image screenshot and export.

    - Network

      - Connectivity: 			Metal Shielded RJ-45(10/100/1000BASE-T)

      - Protocols supported:

        - IP v4 / v6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, PIM-SM, SNMP v1/2c/3 – MIB-2, UPnP, LLDP

        - Media: RTP, SRTP, RTCP, RTSP

        - Multicast: IGMP

        - Notifications: FTP, SMTP

        - Remote Access: PPPoE

      - DDNS – The multi-directional camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS) – Layer 3 DSCP

      - Security features:

        - User password protection

        - The device shall not provide a manufacture default password. Initial password setting shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses.

        - SRTP secured video communication for multicast.

        - Digest login authentication.

        - User access log.

        - 802.1X Authentication (EAP-TLS, EAP-LEAP)

        - Device Certificate (Hanwha Techwin Root CA)

        - HTTPS(SSL/TLS) login authentication and secured video communication with TLS v1.3

        - Disk encryption on SD card recording (AES-256)

        - File encryption on local disk recording via web browser (AES-256)

        - Mutual Authentication (Client Authentication) for secure communication

      - Discovery - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings

    - Electrical

      - Power

        - Input Voltage / Current	PoE+ (IEEE802.3at)

        - Power Consumption:

          - PoE+			Typical: 8.2W, Max.: 16.5W

    - Mechanical And Environmental

      - Material:

        - Housing:			Aluminum

        - Color			White

        - RAL Code			RAL9003

      - Dimensions (W x H):		251 x 116.5 mm (9.88in. x 4.58in.)

      - Weight 				3kg (6.61lb.)

      - Temperature:

        - Operating: 			-40° C to 60° C (40° F to 140° F)

*Start up should be above at -35˚C

        - Storage:  			-50° C to 60° C (-58° F to 140° F)

      - Humidity:			0 - 90%, non-condensing

      - Environmental Rating:

        - Mechanical (Vandal) Protection		IK10

        - Ingress Protection				IP66, NEMA4X

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA) or Network Video Recorder (NVR).

    - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

    - The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

    - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

    - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

8