Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

2MP(PTZ) MULTI-DIRECTIONAL CAMERA with 4ea lenses(2MP / 5MP)

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.
Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

# MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00 	Data Communications

# 28 23 13 	Video Surveillance Control and Management Systems

# 28 23 16 	Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19 	Digital Video Recorders and Analog Recording Devices

# 28 23 23 	Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

# 28 2x 00	Video Management System

2MP(PTZ) MULTI-DIRECTIONAL CAMERA with 4ea lenses(2MP / 5MP)

- GENERAL

  - SUMMARY

    - Section includes an 2MP(PTZ) multi-directional camera with 4ea lenses(2MP / 5MP).

    - Product - An 2MP(PTZ) IP video camera with 4ea lenses of multi-streaming (H.265, H.264 and MJPEG) capability and ability to produce videos in various view

## Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC		Auto Gain Control

      - AES		Advanced Encryption Standard

      - API		Application Programming Interface

      - ARP		Address Resolution Protocol

      - AWB		Auto White Balance

      - BLC		Back light compensation

      - CBR		Constant Bit Rate

      - CVBS		Composite Video Blanking Sync

      - DHCP		Dynamic Host Configuration Protocol

      - DNR		Digital Noise Reduction

      - DNS		Domain Name Server

      - DDNS		Dynamic Domain Name Server

      - DSCP		Differentiated Services Code Point

      - fps		frames per second

      - FTP		File Transfer Protocol

      - GOV		Group of Video

      - GUI		Graphical User Interface

      - HD		High Definition

      - HTTP		Hypertext Transfer Protocol

      - ICMP		Internet Control Message Protocol

      - IGMP		Internet Group Management Protocol

      - IP		Internet Protocol

      - IR		Infrared

      - JPEG		Joint Photographic Experts Group

      - LAN 		Local Area Network

      - LED		Light Emitting Diode

      - LDC		Lens Distortion Correction

      - LPR		License Plate Recognition

      - MJPEG		Motion JPEG

      - MP		Megapixel

      - MPEG		Moving Pictures Experts Group

      - NAS		Network Attached Storage

      - NTP		Network Time Protocol

      - PIM-SM	Protocol Independent Multicast-Sparse Mode

      - PoE		Power over Ethernet

      - PPPoE		Point to Point Protocol over Ethernet

      - QoS		Quality of Service

      - RTP		Real-Time Transport Protocol

      - RTCP		Real-Time Control Protocol

      - RTSP		Real-Time Streaming Protocol

      - SDK		Software Development Kit

      - SMTP		Simple Mail Transfer Protocol

      - SNMP		Simple Network Management Protocol

      - SSDR		Super Smart Dynamic Range

      - SSNR		Super Smart Noise Reduction

      - SSL		Secure Sockets Layer

      - TCP		Transmission Control Protocol

      - UDP		User Datagram Protocol

      - UPnP		Universal Plug and Play

      - VBR		Variable Bit Rate

      - VMS		Video Management System

      - WDR		Wide Dynamic Range

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

      - Video

        - ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - Emissions

        - FCC-47 CFR Part 15 Subpart B Class B

        - CE EN 55022:2010

      - Immunity - CE

        - EN 50130-4:2011

        - EN 61000-3-3:2014

        - EN 61000-4-2:2009

        - EN 61000-4-3:2006+A2:2010

        - EN 61000-4-4:2012

        - EN 61000-4-5:2014

        - EN 61000-4-6:2009

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

      - Ingress Protection and Vandal Resistance

        - ANSI / IEC60529 – Degrees of protection Provided by Enclosures : IP67

        - IEC EN 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts : IK10

        - IEC 60068-2-75 : IK10

    - Definitions

      - GOV (Group of Video object planes) - A set of video frames for H.264 and H.265 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of 2 kinds of frames: I-Frame and P-Frame.

      - WiseStream – Smart Codec that controls quantization parameter in H.265 and H.264 to efficiently manage bitrate of the video stream and reduce the storage required.

      - Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Multi-exposure wide dynamic range - Operation which automatically adjusts shutter speed to provide a wide range between dark and light areas visible at the same time, preventing backlighting issues. Long exposure is used for bright areas and a short exposure is used in dark areas.

      - Dynamic fps – Dynamic assignment of fps (frames per seconds) based on the movement of object(s) in the scene to efficiently manage bitrate of the video stream and reduce the storage required.

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment indicated in 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded camera video analytics free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://www.hanwha-security.com/

    - Model		PNM-9321VQP

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - The camera shall provide multi-directional view and produce video in various viewing modes.

    - The optional lens modules (2M/5M) shall be selectable depending on the customer’s requirements.

    - Smart Handover available – PTZ is interconnected to CH1/2/3/4 when MD/analytics occurs in a specific channel, handover to the PTZ is executed to preset or analytics zone.

    - Smart Zoom feature to allow easy control of PTZ camera by clicking a spot or dragging to select an area on CH1/2/3/4.

    - Hallway view is available on CH1/2/3/4

    - Each Channel supports an SD Card

    - SFP module slot is supported so video transmission is supported at long distances.

    - It provides one single power supply for all multi-channels including PTZ.

    - Video Compression and Transmission – The multi-directional camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 – Maximum of 60 fps at all resolutions

        - MJPEG – Maximum of 30 fps

      - The multi-directional camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, bit rate, and other video settings.

      - The multi-directional camera shall have four lenses, and one PTZ and each lens shall provide the following resolutions.

        - [2MP] : 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

        - [5MP] : 2560 x 1920, 2560 x 1440, 1920 x 1080, 1600 x 1200, 1280 x 1024, 1280 x 960

1280 x 720, 1024 x 768, 800 x 600, 720 x 576, 720 x 480, 640 x 480, 320 x 240

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - The multi-directional camera shall provide smart codec (WiseStream, Dynamic GOV, and Dynamic fps) to efficiently manage bit rate of the video stream and reduce storage while producing video quality that is visually equal to the one without smart codec.

    - Camera – The multi-directional camera device shall have the following physical and performance properties:

      - IK10 rated for protection against impacts.

      - IP66 for protection against dust and water.

      - 2D and 3D digital noise reduction

      - 32 privacy masking regions utilizing polygons

      - The multi-directional camera shall be able to capture high contrast scenes with 150 dB for 2MP and 120dB for 5MP multi-exposure wide dynamic range.

      - Supporting digital image stabilization the camera shall be able to measure movements in three axes and accurately enhance images from distortions caused by instability.

    - Intelligence and Analytics – The multi-directional camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with eight definable detection areas, minimum/maximum object size definition and a learning algorithm that ignores false alarms such as trees and waves on water. The camera shall also be able to send meta-data to NVR or VMS to allow users to search for motion events and generate video summary.

      - Detection of logical events of specified conditions from the camera’s video input

        - Tampering

        - Loitering

        - Directional detection

        - Defocus detection

        - Fog detection

        - Virtual line

        - Enter/exit

        - Appear/disappear

        - Face detection

        - Motion detection

    - Interoperability – The multi-directional camera shall be ONVIF Profile S compliant.

    - The multi-directional camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including Internet Explorer, Firefox, Chrome & Safari

      - Micro SD/SDHC/SDXC memory card options, with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - Alarm notification triggers:

          - Motion detection

          - Video analytics

          - Network disconnect

        - Available notification means upon trigger:

          - File upload via FTP and e-mail

          - Notification via e-mail

          - Record to local storage (SD/SDHC/SDXC card)

          - NAS recording at event triggers

      - Pixel Counter available in the plug-in web viewer.

  - CAMERA SOFTWARE

    - The multi-directional camera shall have a built-in web server which supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari, for which web viewer plug-ins are available, from a PC or Mac.

    - The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Manual recording to SD

      - Audio/microphone control

      - Access Playback and Setup menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Search date and time range

      - Search event type

      - Play an event video

      - Set resolution

      - Play audio if present

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters and crop encoding area

      - User profile to include password, access level and authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - SSL, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

        - Auto configuration

      - Video setup to include flip and mirror mode,  video type and privacy zone

      - Audio setup to include source, audio codec type, gain and bit rate

      - Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, IR illumination, sharpness, contrast, and color level.

      - Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection and event triggers

      - System function to include reboot, upgrade, check system and event logs and application (SDK) management

      - View profile information

    - Client requirements

      - Acceptable Operating Systems:	Windows 7 / 8 / 10, MAC OS X 10.10~10.12

      - Acceptable browsers

        - Non-plugin Webviewer : Google Chrome 54, MS Edge 38, Mozilla Firefox 49

(Windows 64bit only), Apple Safari 9 (Mac OS X only)

  - DETAILED SPECIFICATIONS

    - Video

      - Imager(PTZ Control)

        - Sensor:			1/2.8" 2.4MP CMOS

          - Pixels per sensor:	1,981(H)x1,288(V), 2.55M pixels /

1,944(H)x1,212(V), 2.35M pixels

          - Scanning	:	Progressive

        - Minimum illumination

          - Color mode: 		0.05Lux(1/30sec, F1.6, 30IRE)

          - Black & white mode: 	0.005Lux(1/30sec, F1.6, 30IRE)

          - Pan/Tilt Range		360° Endless / 190°(-5°~185°)

          - Pan/Tilt Speed		Preset:700°/sec, Manual:0.024°/sec~200°/sec

          - Preset/Preset Accuracy	300ea/+-2°

          - Auto tracking		Off/On

          - Focal length(Zoom ratio)	4.44~142.6mm(Optical 32x)

          - Max. Aperture Ratio	F1.6(Wide)~F4.4(Tele)

          - Angular Field of View	H:62.8°(Wide)~2.23°(Tele) / V:36.8°(Wide)~1.26°(Tele)

          - Min. Object Distance	Wide:1.5m, Tele:2m

          - Focus Control		Auto / Manual / One shot AF

          - Lens/Mount type	DC Auto Iris/Board-in type

        - Lens Model

          - SLA-2M2400D(2.4mm Zoom Ratio)

          - SLA-2M2800D(2.8mm Zoom Ratio)

          - SLA-2M3600D(3.6mm Zoom Ratio)

          - SLA-2M6000D(6.0mm Zoom Ratio)

          - SLA-2M1200P(1.2mm Zoom Ratio)

          - SLA-5M3700P(3.7mm Zoom Ratio)

          - SLA-5M4600P(4.6mm Zoom Ratio)

          - SLA-5M7000P(7.0mm Zoom Ratio)

      - Operational

        - The following features with control settings shall be available:

          - Camera Title 				Off / On (Up to 85 characters per line)

- W/W: English / Numeric / Special characters

- China: English / Numeric / Special / Chinese characters

- Common: Multi-line (Max. 5), Color (Grey / Green / Red / Blue / Black / White),

Transparency, Auto scale by resolution

          - Day/night setting:			Multi-directional: Auto (Electrical) / Color / B/W

/Schedule

PTZ: Auto (Removable IR Cut filter) / Color / B/W  /Schedule

          - Backlight compensation (BLC):		Off / BLC / WDR / HLC

          - WDR					150dB(2MP), 120dB(5MP)

          - Contrast Enhancement 			Off / On (SSDR)

          - Digital Noise Reduction			Off / On (SSNR5 : 2D + 3D noise filter)

          - Digital Image Stabilization		On/Off(Built-in gyro sensor)

          - Defog					Off / Auto / Manual

          - Motion Detection			Off / On (8ea polygonal zones)

          - Privacy Masking			Off / On (32ea polygonal  zones)

            - Color:				Gray / Green / Red / Blue / Black / White

Mosaic

          - Gain Control				Off / Low / Middle / High

          - White Balance 				ATW / AWC / Manual / Indoor / Outdoor

(included Mercury and Sodium)

          - Electronic shutter speed:

            - Settings:				Minimum / Maximum / Anti flicker

(2 ~ 1/12,000 sec)

          - Image flip:				Off / On (multi-directional heads)

          - Image mirror:				Off / On (multi-directional heads)

          - Hallway view : 				90˚/270˚ (multi-directional heads)

          - Intelligent video analytics		Tampering, Loitering, Directional Detection,

Defocus Detection, Fog Detection, Virtual Line, Enter/Exit, (Dis) Appear, Motion Detection, Face Detection

          - Alarm Triggers				Motion detection, Video analytics,

Network disconnect

          - Alarm Events				File upload via FTP and E-mail, Notification via

E-mail, Local storage (SD/SDHC/SDXC) recording at event triggers, External output

          - Pixel Counter				Support

      - Video Streams

        - The multi-directional camera shall be able to produce 10 independent video profiles, each of which may have the following properties:

          - Encoding type:

            - H.265

            - H.264

            - MJPEG

          - Resolution:

            - [2MP] : 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

            - [5MP] : 2560 x 1920, 2560 x 1440, 1920 x 1080, 1600 x 1200, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 720 x 576, 720 x 480, 640 x 480, 320 x 240

          - Maximum frame rate:

            - H.265 and H.264:	2MP Max. 60fps, 5MP 30fps at all resolutions

            - MJPEG: 		Max. 30fps

          - Smart Codec		Manual mode (Area-based : 5ea),

          - Bit rate control method:

            - H.265 and H.264

              - Target bitrate level control

              - Constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG

              - Quality level control

              - Variable bit rate (VBR)

      - Number of multi-streaming profiles:	10 maximum

      - Simultaneous users (total):		20 maximum (unicast)

      - Storage and Recording

        - The multi-directional camera shall have onboard SD card storage for each sensor.

          - Card type:	Micro SD/SDHC/SDXC

          - Capacity:	Up to 128 GB per card

          - SD/SDHC/SDXC 5 slots (Each CH) - Motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded.

          - Local PC for Instant Recording

      - Interoperability – Video streams shall be capable of supporting ONVIF profile S.

      - Single Images – The multi-directional camera shall support screenshot and export in jpg format.

    - Network

      - Connectivity: 	10/100 Base-T Ethernet via RJ-45 connector

      - Fiber		SFP slot(100/1000Mbps)

      - Protocols supported:

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

        - Remote Access: Point-to-Point Protocol over Ethernet) (PPPoE)

      - DDNS – The multi-directional camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS) – Layer 3 DSCP

      - Security features:

        - User password protection

        - The device shall not provide a manufacture default password. Default password change shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering - list of allowed or blocked IP addresses

        - HTTPS(SSL) login authentication

        - HTTPS(SSL) secured communications

        - Digest login authentication

        - User access log

        - 802.1x authentication

      - Discovery - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network and enable device configuration remotely and simultaneously for multiple devices to reduce time required for installation and setup.

    - Electrical

      - Power

        - Input Voltage / Current	HPoE (Injector included)

        - Power Consumption:

          - HPoE			Max. 65W

    - Mechanical And Environmental

      - Material:

        - Housing:			Ivory / Metal

      - Dimensions (W x H):		Ø367.8 x 381.5mm(14.49” x 15.02”)

      - Weight 				7.05KG(16.1 lb)

      - Temperature:

        - Operating: 			-35° C to 55° C

        - Storage:  			-50° C to 60° C

      - Humidity:			0 - 90%, non-condensing

      - Environmental Rating:

        - Mechanical (Vandal) Protection		IK10

        - Ingress Protection				IP66

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

    - All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA).

    - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

    - The contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

    - Before permanent installation of the system, the contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

    - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.



END OF SECTION

8