Hanwha Vision is a leading global supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing system solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative products for professional security applications.

For additional information, https://www.hanwhavision.com/en, https://hanwhavisionamerica.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

Model: PNM-A13022RV

Alternates: PNM-C9022RV

GENERAL DESCRIPTION

Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously. (see detailed specifications)

The camera shall be able to configure various resolution selections. (see detailed specifications)

The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings

The camera shall support unicast video streaming up to 20 users.

The camera shall support multiple streaming up to 4 profiles and 3 virtual channels.

The camera shall support multicast video streaming.

The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

The camera shall provide WiseStream based on AI engine to efficiently manage bit rate of the video stream and reduce storage.

The camera shall provide WiseNRⅡ, an AI-based feature to reduce noise and blur on image.

Camera – The camera device shall have the following physical and performance properties.

True day/night operation with removable IR cut filter. The IR function shall provide manual power adjustment, automatic mode based on objects in the center of the screen, and automatic mode based on objects anywhere in the scene.

The camera shall be able to produce clear images in high-contrast scenes with multi-exposure wide dynamic range up to 120dB.

The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

The camera shall be able to configure up to 32 privacy masking areas with 4-point quadrangle zones.

The camera shall support mobile applications (iOS, Android) for remote viewing angle adjustment, rotation and focus control during installation.

The camera shall have a dual NPU capable of running all AI & analytics functions fully on one NPU while the other is available for ISP functions, including AI-based noise reduction, WDR, & low light image enhancements.

Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

Classified object type: Person / Face / Vehicle / License plate

Attributes: Person (Gender, Clothing top/bottom color, Bag), Face (Age, Gender, Mask, Glasses), Vehicle (Type: car / bus / truck / motorcycle / bicycle, Color)

BestShot supported

Analytics events based on AI engine: Motion detection, Object detection, Virtual line(Crossing / Direction), Virtual area(Loitering / Intrusion / Enter / Exit / Appear / Disappear), Slip & Fall detection, Face mask detection, Social distancing detection

Analytics events: Defocus detection, Tampering, Audio detection

Business Intelligence: People / Vehicle / Crowd counting, Queue management, Heatmap based on AI engine

Interoperability

The camera shall be ONVIF Profile S, G, T and M compliant.

The camera shall be compatible with the Hanwha Vision Cloud Portal, including OnCloud VMS, HealthPro, and SightMind, utilizing the provided CloudConnector Open Platform application.

The camera shall be compatible with Hanwha Vision AI packs, utilizing Open Platform, to add additional AI functions.

Additional Features

Micro SD/SDHC/SDXC memory slot with configurable pre-alarm and post-alarm recording intervals

NAS recording option with configurable pre-alarm and post-alarm recording intervals

Alarm triggers and notifications

Impact protection (IK10), Water & Dust protection (IP66)

Support Wisepower (Power monitoring & ECO mode)

The camera uses recycled plastic in construction

Device certificate (Hanwha Vision Root CA), Secure storage (Secure element with FIPS 140-3 level 3/CC EAL6+ certified), S-BOM

The camera shall feature an admin session timeout to prevent a webpage user from leaving access to the camera for longer than a specified period.

DETAILED SPECIFICATIONS

Video

Imaging device: 1/2.8" CMOS

Resolution: 5120x2592, 4112x2080, 3240x1640, 2940x1488, 2560x1296, 2016x1016, 1600x800,1424x720, 1280x648, 1016x512, 832x480, 832x416, 800x600, 640x480

Maximum Framerate

H.265/H.264: Max. 30fps(60Hz) / 25fps(50Hz)

MJPEG: Max. 30fps

Minimum Illumination

Color: 0.1Lux (1/30sec, 30IRE)

B/W: 0.01Lux (1/30sec, 30IRE)

Lens

Focal length: 3.06mm

Max. Aperture Ratio: F1.6

Field of View

H:194°

V:93°

Min. Object Distance: 5m (16.4ft)

Focus Control: Fixed

Pan & Tilt & Rotate Range

Pan: 0°~350°

Tilt : 0°~50°

Operational Functions

Day/Night: Auto (ICR)

Backlight Compensation: BLC / HLC / WDR / SSDR

WDR(Wide Dynamic Range): 120dB

Digital Noise Reduction: SSNRⅤ, WiseNRⅡ(AI-based)

Motion Detection: 8 zones, each with 8-point polygonal shape

Privacy Masking

Quantity: 32 zones

Shape: each with 4-point quadrangle shape

Color: gray/green/red/blue/black/white

Gain Control: Low, Middle, High

White Balance: ATW / AWC / Manual / Indoor / Outdoor

Electronic Shutter Speed

Minimum / Maximum / Anti-flicker (2 ~ 1/12,000sec)

Prefer shutter control (based on AI engine)

Video Rotation: Flip, Mirror

Alarm Interface

Input/Output : 2 configurable I/O ports

Others: support extra Alarm I/O via optional I/O Box

Alarm Triggers: Analytics, Network disconnection, Alarm input, Time schedule, MQTT subscription

Alarm Events (When alarm trigger occurred)

File upload (image): e-mail/FTP/SFTP

File upload (video clip): FTP/SFTP

Recording: SD/SDHC/SDXC or NAS recording at event triggers, Alarm output

Handover: PTZ preset, Send message by HTTP / HTTPS / TCP, Audio clip playback

MQTT: publication

Audio in: Selectable (Mic in / Line in)

Audio out: Line out

Light Type: IR LED

IR Viewable Length: 20m(65.62ft) 4 individual zones with Automatic/Manual level adjustment

IR Wavelength: 850nm

Analytics

Classified object type: Person / Face / Vehicle / License plate

Attributes

Person: Upper/Lower clothes color, Gender, Bag

Vehicle: Type (car / bus / truck / motorcycle / bicycle), Color

Face: Age, Gender, Face mask, Glasses

        - Thumbnail: BestShot supported

Analytics Events

AI-based analytics events: Object detection, Virtual line(Crossing / Direction), Virtual area(Loitering / Intrusion / Enter / Exit), Motion detection, Face mask detection, Social distancing detection, Slip & Fall detection

          - Normal: Defocus detection, Tampering, Audio detection, Virtual area(Appear/Disappear)

        - Business Intelligence: People counting, Queue management, Heatmap, Vehicle counting, Crowd counting

Network

Ethernet: Metal shielded RJ-45 (10/100/1000 BASE-T)

Video compression

H.265/H.264: Main / Baseline / High

MJPEG

Audio Compression

G.711 u-law /G.726 selectable, G.726(ADPCM) 8kHz, G.711 8kHz

G.726: 16kbps,24kbps,32kbps,40kbps

AAC-LC: 48kbps at 16kHz

OPUS: 48kHz(64kbps)

Smart Codec: Manual, WiseStream (Based on AI engine)

Bitrate Control

H.265 / H.264: CBR or VBR

MJPEG: VBR

Streaming

Unicast: up to 20 users

Multicast: Support

Multiple streaming: Up to 4 profiles, 3 virtual channel support

Protocol

Network: IPv4, IPv6, TCP/IP, UDP/IP

Streaming: RTP(UDP), RTP(TCP), RTCP, RTSP, SRTP(TCP, UDP Unicast)

Services: NTP, HTTP, HTTPS, SSL/TLS, DHCP, DNS, DDNS

File Transfer: FTP/SFTP, SMTP

Management: SNMPv1/v2c/v3(MIB-2), IGMP, UPnP, Bonjour, LLDP

Other: ICMP, ARP, QoS, MQTT

Application Programming Interface

ONVIF: Profiles S, G, T, M

Others: SUNAPI (HTTP API), Hanwha Vision Open Platform

Direct Cloud Connection: Optional (needs gateway)

Security

OS / Firmware Protect: Encrypted firmware, Secure boot, Signed firmware

User authentication: Digest authentication, Prevent brute-force attack

Network authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

Secure communication: HTTPS, WSS (WebSocket Secure), SRTP, 802.1AE (MACsec)

Access control: IP-based access control, MAC-based access control, Auto logout

Data protect: Encryption credentials, Encrypt compress for live recording file export

Audit: Access / System / Event log management

Device ID: Device certificate (Hanwha Vision Root CA)

Secure storage: SD card partition encrypt (AES-256), Secure element

Security certificate: Secure element (FIPS 140-3 level 3, CC EAL6+), Consumer IoT Security Standard (ETSI EN 303 645)

General

Memory

RAM: 8GB

Flash: 8GB eMMC

Edge Storage

Micro SD/SDHC/SDXC:  2TB (1TB x 2 slots)

Environmental & Electrical

Operating Condition

Temperature:  -40°C to +55°C (-40°F to +131°F)

Humidity: 0 ~ 100% RH(Condensing)

Storage Condition

Temperature: -50°C to +60°C (-58°F to +140°F)

Humidity: 0 ~ 90% RH (non-condensing)

EPA(Effective Projected Area): 0.0286㎡

Input Voltage: PoE+ (IEEE802.3at class 4), 12V DC

Power Consumption

PoE: Max. 22.3W, typical 11.5W

12V DC: Max. 20.0W, typical 10.0W

Power Redundance Failover: Support

Mechanical

Color: White

Material: Aluminum

Bubble: Hard-coated dome

RAL Code: RAL9003

Product Dimensions (Width x Height): ø253 x 170mm (ø9.96 x 6.69")

Weight: 2,800g(6.17lb)

Compatible Conduit Hole: Conduit: 19.1mm(3/4") (M25)

Compatible Conduit Gang box: Single, double, 4" octagon, square

Certifications & Standards

EMC

FCC 47 CFR Part 15 Subpart B Class A

ICES-3(A)/NMB-3(A)

CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

VCCI CISPR 32 Class A

RCM AS/NZS CISPR 32 Class A

KS C 9832 Class A, KS C 9835

Safety

UL 62368-1, CAN/CSA C22.2 NO. 62368-1

IEC/EN 62471

Environment

IEC/EN 63000

IEC/EN 60529 IP66

IEC/EN 62262 IK10

NEMA 250 Type 4X

IEC 60068-2-1, IEC 60068-2-2, IEC 60068-2-6, IEC 60068-2-14, IEC 60068-2-27, IEC 60068-2-78

DORI Distance

Detect (25PPM / 8PPF): Wide 43.8m(143.85ft)

Observe (63PPM / 19PPF): Wide 17.5m(57.54ft)

Recognize (125PPM / 38PPF): Wide 8.8m(28.77ft)

Identify (250PPM / 76PPF): Wide 4.4m(14.39ft)

END OF SECTION

10