Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com, https://hanwhavisionamerica.com)

    - Model: PNM-C20000QB

    - Alternates: none

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265/H.264: Max. 30fps/25fps(60Hz/50Hz)

        - MJPEG: Max. 30fps/25fps(60HZ/50Hz)

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections

        - 2592x1944, 2560x1440, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240, 320x180

      - Simultaneous unicast access by up to 20 users

      - The camera shall support multicast video streaming up to4 profiles per channel.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStream Ⅱ to efficiently manage bit rate of the video stream and reduce storage.

      - The camera shall provide WiseNRⅡ that working based on AI engine for reduce noise and blur on image.

    - Camera – The camera device shall have the following physical and performance properties:

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range.

      - Auto day/night operation with electrical

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 16 privacy masking areas with 4 points quadrangle zones.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Classified object type: Person/Face/Vehicle(Type: car/bus/truck/motorcycle/bicycle)/License plate

      - Attributes: Person(Clothing top/bottom color), Vehicle(Type: car/bus/truck/motorcycle/bicycle, Color)

      - Support BestShot

      - Analytics events based on AI engine: Object detection, Virtual line(Crossing/Direction), Virtual area(Loitering/Intrusion/Enter/Exit/Appear/Disappear)

      - Analytics events: Motion detection, Defocus detection, Tampering, Shock detection, Audio detection

      - Business Intelligence: People/Vehicle counting(based on AI engine)

    - Interoperability – The camera shall be ONVIF Profile G/S/T/M compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarm triggers and notifications

      - PoE+ or 12VDC capable

  - DETAILED SPECIFICATIONS

    - Video

      - Resolution: 2592x1944, 2560x1440, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240, 320x180

      - Max. Framerate

        - H.265/H.264: Max. 30fps/25fps(60Hz/50Hz)

        - MJPEG: Max. 30fps/25fps(60HZ/50Hz)

      - Video out

        - Micro USB type C for installation

        - HDMI (1920x1080, Single/Quad view)

    - Lens (Optional)

      - SLA-T1080FA, SLA-T2480A, SLA-T2480VA, SLA-T4680A, SLA-T4680VA, SLA-T4680DA, SLA-T4680DSA, SLA-T2880BA

    - Operational Functions

      - Camera Title: Displayed up to 85 characters

      - Day and Night: Auto(Electrical)

      - Backlight Compensation: BLC / WDR / SSDR

      - WDR: 120dB

      - Digital Noise Reduction: SSNRⅤ, WiseNRⅡ(Based on AI engine)

      - Digital Image Stabilization: support

      - Defog: support

      - Motion Detection: 8ea, 8 points polygonal zones

      - Privacy Masking: 16ea, 4 points quadrangle zones(Color: Gray/Green/Red/Blue/Black/White)

      - Gain Control: Low / Middle / High

      - White Balance: ATW / AWC / Manual / Indoor / Outdoor

      - Lens Distortion Correction: support

      - Electronic Shutter Speed: Minimum / Maximum / Anti-flicker (2 ~ 1/12,000sec), Prefer shutter control(Based on AI engine)

      - Rotation: Flip, Mirror, Hallway view (90°/270°) - each channel separately

      - Analytics

        - Classified object type: Person/Face/Vehicle(Type: car/bus/truck/motorcycle/bicycle)/License plate

        - Attributes: Person(Clothing top/bottom color), Vehicle(Type: car/bus/truck/motorcycle/bicycle, Color)

        - Support BestShot

        - Analytics events based on AI engine: Object detection, Virtual line(Crossing/Direction), Virtual area(Loitering/Intrusion/Enter/Exit/Appear/Disappear)

        - Analytics events: Motion detection, Defocus detection, Tampering, Shock detection, Audio detection

      - Business Intelligence: People/Vehicle counting(based on AI engine)

      - Alarm I/O: 4 configurable I/O ports

      - Alarm Triggers: Analytics, Network disconnection, Alarm input, App event, Time schedule, MQTT subscription

      - Alarm Events (When alarm trigger occurred)

        - File upload(image): e-mail/FTP

        - Notification: e-mail

        - Recording: SD/SDHC/SDXC or NAS recording at event triggers

        - Alarm output

        - Handover: PTZ preset, send message by HTTP/HTTPS/TCP

        - Audio clip playback

        - MQTT: publication

      - Audio In: 4 x selectable (Mic in / Line in)

      - Audio Out: Line out

      - Webpage Language: English, Korean, Simplified Chinese, Traditional Chinese, French, Italian, Spanish, German, Japanese, Russian, Portuguese, Czech, Polish, Turkish, Dutch, Hungarian, Greek

      - Edge storage: Micro SD / SDHC / SDXC 2 slot Max. 2TB (1TB x 2)

      - Memory: 8GB RAM, 8GB eMMC

    - Network

      - Connectivity: Metal shielded RJ-45 10/100/1000 Base-T Ethernet

      - Video compression

        - H.265/H.264: Main / Baseline / High

        - MJPEG

      - Audio Compression

        - G.711(PCM): 8kHz(64Kbps)

        - G.726(ADPCM): 8kHz(16/24/32/40Kbps)

        - AAC-LC: 16kHz(48Kbps)

      - Smart Codec: WiseStream (Based on AI engine)

      - Bitrate Control

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming

        - Unicast(20 users) / Multicast

        - Multiple streaming (Up to 4 profiles per channel)

      - Application Programming Interface

        - ONVIF Profile S / G / T/ M

        - SUNAPI (HTTP API)

        - Hanwha Vision Open Platform

      - Protocols: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP/SFTP, SMTP(StartTLS), ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, SRTP(TCP, UDP Unicast), CDP, MQTT

      - DDNS – The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS) – Layer 3 DSCP

      - Security features:

        - OS / Firmware Protect: Encrypted Firmware, Secure boot, Signed Firmware

        - User authentication: Digest Authentication

        - Network authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

        - Secure Communication: HTTPS, SRTP, WSS (WebSocket Secure)

        - Access Control: IP-based access control

        - Data Protect: Encryption Credentials, Encrypt compress for live recording file EXPORT

        - Audit: Access / System / Event Log management

        - Device ID: Device certificate (Hanwha Vision Root CA)

        - Secure Storage: Secure element, SD card partition encrypt

        - Security Certificate: Secure element (FIPS 140-3 level3, CC EAL6+)

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Environment and Electrical

      - Operating Temperature & Humidity: -10°C ~ +45°C(14°F ~ +113°F) / 0~100% RH(condensing)

      - Storage Temperature & Humidity: -50°C ~ +60°C(-58°F ~ +140°F) / 0~90% RH

      - Input Voltage: PoE+(IEEE802.3at, Class4), 12VDC

      - Power Consumption

        - PoE++: Max 25.5W, typical 23.8W

        - PoE+: Max 23.1W, typical 21.5W

    - Mechanical

      - Color/Material: Black / Plastic

      - RAL Code: RAL9005

      - Dimensions (W x H x D): 200 x 37 x 146mm (7.87 x 1.46 x 5.75")

      - Weight: 1.015Kg(2.24lb)

    - Certifications

      - EMC

        - FCC 47 CFR 15 Subpart B Class A

        - ICES-3(A)/NMB-3(A)

        - CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

        - VCCI CISPR 32 Class A

        - RCM AS/NZS CISPR 32 Class A

        - KS C 9832 Class A, KS C 9835

      - Safety

        - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

        - IEC/EN 62471

      - Environmental

        - IEC/EN 63000

    - Compatible Lenses

      - SLA-T1080FA

        - Imaging Device: 1/2.8” 2MP CMOS

        - Min illumination: Color 0.03Lux(F1.6, 1/30sec)

        - Focal Length: 1.6mm fixed focal

        - Max. Aperture Ratio: F1.6

        - Angular Field of View: 187°(H), 113.9°(V)

        - Mount Type: RJ12 connected with main unit

        - Focus Control: fixed

        - Color / Material: Black / Plastic, Metal

        - Product dimension: Ø35 x L68.9mm

        - Weight: 430g

      - SLA-T2480A

        - Imaging Device: 1/2.8” 2MP CMOS

        - Min illumination: Color 0.045Lux(F2.0, 1/30sec)

        - Focal Length: 2.4mm fixed focal

        - Max. Aperture Ratio: F2.0

        - Angular Field of View: 138°(H), 165°(V)

        - Mount Type: RJ12 connected with main unit

        - Focus Control: fixed

        - Color / Material: Black / Plastic, Metal

        - Product dimension: Ø24xL49.8mm

        - Weight: 318g

      - SLA-T2480VA

        - Imaging Device: 1/2.8” 2MP CMOS

        - Min illumination: Color 0.045Lux(F2.0, 1/30sec)

        - Focal Length: 2.4mm fixed focal

        - Max. Aperture Ratio: F2.0

        - Angular Field of View: 138°(H), 73°(V), 165°(D)

        - Mount Type: RJ12 connected with main unit

        - Focus Control: fixed

        - Color / Material: Black / Plastic, Metal

        - Product dimension: 30(W) x 27.9(H) x 45(D) mm

        - Weight: 328g

      - SLA-T4680A

        - Imaging Device: 1/2.8” 2MP CMOS

        - Min illumination: Color 0.06Lux(F2.5, 1/30sec)

        - Focal Length: 4.6mm fixed focal

        - Max. Aperture Ratio: F2.5

        - Angular Field of View: 73°(H), 39°(V), 85°(D)

        - Mount Type: RJ12 connected with main unit

        - Focus Control: fixed

        - Color / Material: Black / Plastic, Metal

        - Product dimension: Ø24xL49.8mm

        - Weight: 310g

      - SLA-T4680VA

        - Imaging Device: 1/2.8” 2MP CMOS

        - Min illumination: Color 0.06Lux(F2.5, 1/30sec)

        - Focal Length: 4.6mm fixed focal

        - Max. Aperture Ratio: F2.5

        - Angular Field of View: 73°(H), 39°(V), 85°(D)

        - Mount Type: RJ12 connected with main unit

        - Focus Control: fixed

        - Color / Material: Black / Plastic, Metal

        - Product dimension: 30(W) x 27.9(H) x 45(D) mm

        - Weight: 321g

END OF SECTION

8