Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwhavision.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Vision (https://www.hanwhavision.com)

    - Model : PNM-C32083RVQ

    - Alternates : none

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 – frame rates to 15/15 fps (60Hz/50Hz)

        - MJPEG – frame rates to 15/15 fps (60Hz/50Hz) @4MP Max. 5fps

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections

        - 3840x2160, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Simultaneous unicast access by up to 10 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range.

      - IP66, IK10, NEMA4X, NEMA-TS 2(2.2.8, 2.2.9) rated for protection against impacts.

      - Auto day/night operation with removable IR cut filter

        - Low light level operation to 0.1 lux (color) and 0 lux (black and white)

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of up to 20m(with HPoE) at 0 Lux when activated in Black & White mode.

      - 6 privacy masking regions with rectangular.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

      - Detection of logical events of specified conditions from the camera’s video

        - Classified object type:

          - Person

          - Vehicle

        - Attributes

          - Vehicle Type: car/bus/truck/motorcycle/bicycle

        - Support BestShot

        - Analytics events based on AI engine

          - Object detection

          - Virtual line(Crossing/Direction)

          - Virtual area(Loitering/Intrusion/Enter/Exit)

        - Analytics events

          - Defocus detection

          - Motion detection

          - Tampering

          - Audio detection

          - Sound classification

          - Virtual area(Appear/Disappear)

    - Interoperability – The camera shall be ONVIF Profile S and T compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including Internet Explorer, Firefox, Chrome & Safari

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Bi-directional audio

      - Alarms and notifications

        - alarm notification triggers:

          - Analytics

          - Network disconnect

          - Alarm input with NW I/O Box

          - App event

          - Time schedule

          - MQTT subscription

        - available notification means upon trigger:

          - File upload(image): e-mail/FTP

          - Notification: e-mail

          - Recording: SD/SDHC/SDXC or NAS recording at event triggers

          - Alarm output

          - Handover: PTZ preset, send message by HTTP/HTTPS/TCP

          - Audio clip playback

          - MQTT: publication

      - Pixel Counter available in the plug-in web viewer.

      - HPoE capable

      - IP66, NEMA4X, IK10

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

  - CAMERA SOFTWARE

    - The camera shall have a built in web server which supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari, for which web viewer plug-ins are available, from a PC or Mac.

    - The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Manual recording to SD or NAS

      - Audio/microphone control

      - Access Playback and Setup menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Search date and time range

      - Search event type

      - Play an event video

      - Set resolution

      - Play audio if present

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, crop encoding area

      - User profile to include password, access level, authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - SSL, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

        - Auto configuration

      - Video setup to include flip and mirror mode, hallway view mode, video type, privacy zone

      - Audio setup to include source, audio codec type, gain, and bit rate

      - Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, IR illumination, sharpness, contrast, and color level.

      - Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection, event triggers

      - System function to include reboot, upgrade, check system and event logs, application (SDK) management

      - View profile information

    - Client requirements

      - Acceptable Operating Systems:	Windows 7 / 8.1 / 10, MAC OS X 10.9~10.11

      - Acceptable browsers

        - Plug-in free web viewer: Google Chrome 47, MS Edge 20

        - Plug-in web viewer: 	MS Explore 11, Mozilla Firefox 43, Apple Safari 9 * Mac OS X only

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : 1/2.8" CMOS x 4CH

        - Minimum illumination

          - Color mode : 0.1Lux (1/30sec, F1.6, 30IRE)

          - Black & white mode :

            - 0.01Lux (IR LED off)

            - 0Lux (IR LED on)

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 85 characters per line)

            - W/W : English / Numeric / Special characters

            - China : English / Numeric / Special / Chinese characters

            - Common : Multi-line (Max. 5), Color (Grey / Green / Red / Blue / Black / White), Transparency, Auto scale by resolution

          - Day/night setting

            - Auto (ICR) / Color / B/W / External / Schedule

          - Backlight compensation (BLC)

            - BLC, WDR, SSDR

          - WDR

            - 120dB

          - Contrast Enhancement

            - Off / On

          - Digital Noise Reduction (DNR)

            - SSNRⅤ,

            - WiseNRⅡ(Based on AI engine)

          - Motion Detection

            - Off / On (8ea polygonal zones)

          - Privacy Masking

            - Off / On (6ea rectangular zones)

            - Color: Grey/Black/White

          - Gain Control

            - Low / Middle / Max

          - White Balance

            - ATW / AWC / Manual / Indoor / Outdoor

          - Electronic shutter speed

            - Minimum / Maximum / Anti flicker(1/5~1/12,000sec)

            - Prefer shutter control(Based on AI engine)

          - Image flip

            - Off / On

          - Image mirror

            - Off / On

          - Hallway view

            - 90°/270° - Each channel separately

          - Alarm I/O

            - External device support

          - Alarm Triggers

            - Analytics, Network disconnect, Alarm input, App event, Time schedule, MQTT subscription

          - Alarm Events

            - File upload(image): e-mail/FTP

            - Notification: e-mail

            - Recording: SD/SDHC/SDXC or NAS recording at event triggers

            - Alarm output

            - Handover: PTZ preset, send message by HTTP/HTTPS/TCP

            - Audio clip playback

            - MQTT: publication

          - Pixel Counter

            - Available in the plug-in web viewer.

        - Lens : 3.3~5.7mm(1.7x) motorized varifocal

          - Aperture Ratio

            - F1.5(Wide)~F1.9(Tele)

          - Angle of view:

            - H: 109°(Wide)~56°(Tele)

            - V: 55°(Wide)~31°(Tele)

            - D: 136°(Wide)~65°(Tele)

          - Min. Object Distance

            - 1.3m(4.27ft)

          - Focus control

            - Simple focus

          - Lens Type

            - Fixed iris

      - IR Viewable Length :

        - PoE+: WiseIR 15m(49.21ft)

        - HPoE: WiseIR 20m(65.62ft)

      - PTR

        - PAN

          - 360° Endless

        - TILT

          - 0°~65°

        - Rotate

          - 0°~90°

      - Video Streams

        - The camera shall be able to produce 10 video profiles, each of which may have the following properties:

          - Encoding type:

            - H.265

            - H.264

            - MJPEG

          - Resolution:

            - 3840x2160, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

          - Maximum frame rate:

            - H.265/H.264: Max. 15fps/15fps(60Hz/50Hz)

            - MJPEG: Max. 15fps/15fps(60Hz/50Hz) (@4MP Max. 5fps)

          - Bit rate control method :

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

      - Audio

        - Audio In

          - Selectable(mic in/line in)

        - Audio Out

          - Line out

        - Compression

          - G.711 u-law / G.726 Selectable

          - G.726 (ADPCM) 8KHz, G.711 8KHz

          - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

          - AAC-LC: 48Kbps at 16KHz

      - Number of multi-streaming profiles

        - 10 maximum

      - Simultaneous users (total)

        - 10 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : 2slot 1TB(512GB x 2)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

      - Interoperability

        - Video streams shall be capable of supporting ONVIF protocol, profiles S and T.

      - Single Images

        - The camera shall support jpg file image screenshot and export.

      - Memory

        - 8GB RAM, 1GB Flash

    - Network

      - Connectivity

        - Metal shielded RJ-45(10/100/1000BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

        - Remote Access: Point-to-Point Protocol over Ethernet) (PPPoE)

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - Secure boot, Signed firmware, Firmware encryption

        - Digest Authentication, Privent brute-force attack

        - 802.1X Authentication(EAP-TLS, EAP-LEAP, EAP-PEAP MSCHAPv2)

        - HTTPS, SRTP, WSS(Websocket secure)

        - IP address filtering

        - Authentication information encryption, ZIP compression encryption

        - User Access/System/Event log

        - Device Certificate(Hanwha Private Root CA)

        - TPM 2.0 with FIPS 140-2 level2

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Application Programming Interface

      - ONVIF Profile S/T

      - SUNAPI(HTTP API)

      - Wisenet open platform

    - Electrical

      - Power

        - Input Voltage

          - PoE+(IEEE802.3at Type2, Class4)

          - HPoE(IEEE802.3bt Type3, Class6)

        - Power Consumption

          - PoE+: Max 25.5W, typical 22.0W

          - HPoE: Max 33.0W, typical 22.0W

    - Mechanical And Environmental

      - Color / Material

        - White / Aluminum

        - Bubble: Hard-coated dome

      - Dimensions (W x H)

        - ø250x114.2mm(ø9.84x4.50")

      - Weight

        - 2820g(6.22 lb)

      - RAL Code

        - RAL9003

      - Compatible Conduit hole / Gangbox

        - Conduit hole: 19.1mm(3/4"")(M25)

        - Gangbox: Single, Double, 4"" Octagon, Square

      - Hanging Mount (Dome)

        - SBP-250HMW, SBP-276HMW

      - Temperature:

        - Operating

          - -40°C~+55°C(-40°F~+131°F)

* Start up should be done at above -35°C

        - Storage

          - -50°C~+60°C(-58°F~+140°F) / 0~90% RH

        - Humidity

          - 0~95% RH(non-condensing)

          - Humidity control /w Air vent

      - Certifications

        - EMC

          - FCC 47 CFR 15 Subpart B Class A

          - ICES-3(A)/NMB-3(A)

          - CE/UKCA

            - EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

          - VCCI CISPR 32 Class A

          - RCM AS/NZS CISPR 32 Class A

          - KS C 9832 Class A , KS C 9835

        - Safety

          - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

          - IEC/EN 62471

        - Environmental

          - IEC/EN 63000

          - IEC/EN 60529 IP66, IEC/EN 62262 IK10

          - NEMA 250 type 4X

    - DORI

      - Detect (25PPM/ 8PPF) : Wide: 54.8m(179.73ft) / Tele: 144.4m(473.88ft)

      - Observe (63PPM/ 19PPF) : Wide: 21.9m(71.89ft) / Tele: 57.8m(189.55ft)

      - Recognize (125PPM/ 38PPF) : Wide: 11.0m(35.95ft) / Tele: 28.9m(94.78ft)

      - Identify (250PPM/ 76PPF) : Wide: 5.5m(17.97ft) / Tele: 14.4m(47.39ft)

END OF SECTION

8