Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

    - Model: PNO-A9311RLP

    - Alternates: None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 / H.264: Max. 30fps / 25fps (60Hz / 50Hz)

        - MJPEG: Max. 30fps

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - 3840x2160, 3072x1728, 2592x1944, 2688x1520, 2560x1440, 2048x1536, 1920x1080, 1600x1200, 1280x1024,1280x960, 1280x720, 1024x768, 800 x 600, 800 x 448, 720 x 576, 720x480, 640x480, 640x360

      - The camera shall support unicast video streaming up to 6 users.

      - The camera shall support multicast video streaming.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStream Ⅱ, WiseStreamⅢ on AI engine to efficiently manage bit rate of the video stream and reduce storage.

      - The camera shall provide WiseNRⅡ that working based on AI engine for reduce noise and blur on image.

    - Camera – The camera device shall have the following physical and performance properties

      - Rated for protection against impacts.

      - True day/night operation with removable IR cut filter

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 120dB.

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 3 privacy masking areas with rectangle zones.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - CNN (Convolutional Neural Network) AI with improved accuracy in challenging conditions.

      - Analytics events based on AI engine

        - Vehicle

        - License plate

        - Support License plate number OCR

        - Support Vehicle MMCR

    - Interoperability – The camera shall be ONVIF Profile S, G, T and M compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarm triggers and notifications

      - Pixel Counter available in the web viewer.

      - PoE+ capable including heater

      - Impact protection, Water & Dust protection

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

      - NDAA Compliant

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device: 1/1.8" CMOS

      - Scanning: Progressive

      - Minimum Illumination

        - Color: 0.075Lux (F1.36, 1/30sec)

        - BW: 0.0075Lux (F1.36, 1/30sec), 0Lux (IR LED on)

    - Lens:

      - Focal length: 6.91 ~ 214.7mm (31x) motorized varifocal

      - Max. Aperture Ratio: F1.36(Wide) ~ F4.6 (Tele)

      - Field of View

        - H:60.92°(Wide) ~ 2.2°(Tele)

        - V:35.26°(Wide)~1.24°(Tele)

        - D:69.18°(Wide)~2.52°(Tele)

      - Min. Object Distance: 3m (9.84ft)

      - Focus Control: Simple focus

      - Lens Type: P Iris

    - Pan & Tilt & Rotate

      - Range: None

    - Operational Functions

      - Camera Title: Displayed up to 85 characters

      - Day/Night Setting: Auto (ICR) / Color / B/W / Schedule

      - Backlight Compensation: Off / BLC / WDR / SSDR

      - WDR: 120dB

      - Digital Noise Reduction: SSNRⅤ, WiseNRⅡ(Based on AI engine)

      - Digital Image Stabilization: Support (built-in gyro sensor)

      - Motion Detection: 8ea, Polygonal zones

      - Privacy Masking: 3ea, Rectangle zones (Color: Gray / Black / White)

      - Gain Control: Off / Low / Middle / High

      - White Balance: ATW / AWC / Manual / Indoor / Outdoor

      - Electronic Shutter Speed

        - Min / Max / Anti-flicker (1/5 ~ 1/12,000sec)

      - Digital PTZ: None

      - Video Rotation: Flip, Mirror, Hallway view (90° / 270°)

      - Alarm I/O: 2 configurable I/O ports, DC 12V output (Max. 50mA)

      - Alarm Triggers: Analytics, Network disconnect, Alarm input, App event, Time schedule, MQTT subscription

      - Alarm Events (When alarm trigger occurred)

        - File upload(image): e-mail/FTP

        - Notification: e-mail

        - Recording: SD/SDHC/SDXC or NAS recording at event triggers

        - Alarm output

        - Handover: PTZ preset, Send message by HTTP / HTTPS / TCP

        - Audio clip playback

        - MQTT: publication

      - Audio in: Selectable (mic in / line in)

      - Audio out: Line out

      - Audio Compression

        - G.711 u-law / G.726 Selectable,

        - G.726(ADPCM) 8KHz, G.711 8KHz

        - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps,

        - AAC-LC: 48Kbps at 16KHz

      - Light Type: IR LED (850nm)

      - IR Viewable Length: WiseIR 70m(229.66ft)

      - Video Out (for Installation)

        - CVBS: 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P)

        - USB: Micro USB Type B, 1280x720

      - Edge Storage: Micro SD/SDHC/SDXC 1slot (32GB Card preinstalled)

      - Memory: 4096MB RAM, 1024MB Flash

    - Video Streams

      - Video compression: H.265, H.264, MJPEG

      - Resolution: 3840x2160, 3072x1728, 2592x1944, 2688x1520, 2560x1440, 2048x1536, 1920x1080, 1600x1200, 1280x1024,1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360

      - Maximum Framerate

        - H.265 / H.264: Max. 30fps / 25fps (60Hz / 50Hz)

        - MJPEG: Max. 30fps

      - Smart Codec: Manual (5ea area), WiseStreamⅡ, WiseStreamⅢ (using AI engine)

      - Bitrate Control Method

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming method: Unicast / Multicast

      - Streaming Capability: Multiple streaming (Up to 6 profiles)

      - Simultaneous Users: Max. 6 users (Unicast)

      - Profile set: Max. 10ea

      - Application Programming Interface

        - ONVIF Profile S / G / T/ M

        - SUNAPI (HTTP API)

    - Network

      - Ethernet: Metal shielded RJ-45 (10/100/1000 BASE-T)

      - Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, SRTP (TCP, UDP Unicast), MQTT

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication (EAP-TLS, EAP-LEAP)

        - Device Certificate (Hanwha Private Root CA)

        - TPM 2.0 with FIPS 140-2 level2

        - Secure boot, Verify firmware forgery

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Security

      - OS / Firmware Protect: Encrypted Firmware, Secure boot, Signed Firmware

      - User authentication: Digest Authentication, Prevent brute-force attack

      - Network authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

      - Secure Communication: HTTPS, WSS (WebSocket Secure)

      - Access Control: IP-based access control

      - Data Protect: Encryption Credentials, Encrypt compress for live recording file

      - Audit: Access / System / Event Log management

      - Device ID: Device certificate (Hanwha Private Root CA)

      - Secure Storage: TPM, SD card partition encrypt

      - Security Certificate: TPM with FIPS 140-2 level 2

    - Electrical

      - Input Voltage: PoE+(IEEE802.3at, Class4), 12VDC

      - Power Consumption

        - PoE+: Max 25.5W, typical 14.0W

        - 12VDC: Max 24.0W, typical 12.7W

    - Mechanical And Environmental

      - Color/Material: White / Aluminum, Bubble: Hard-coated window

      - RAL Code: RAL9003

      - Dimensions (W x H): ø136.2 x 358.7mm (ø5.36 x 14.12") *without sunshield

      - Weight: 3200g(7.05lb)

      - Compatible Conduit Hole: 19.1mm (3/4”) (M25)

      - Gangbox: single, double,4” octagon, 4” square

      - Temperature & Humidity

        - Operating: -40°C ~ +55°C (-40°F ~ +131°F) / Less than 90% RH

* Start up should be done at above -30°C

        - Storage: -50°C ~ +60°C (-58°F ~ +140°F) / Less than 90% RH

    - DORI Distance

      - Detect (25PPM / 8PPF): Wide: 130.6m(428.4ft) / Tele: 4,000m(13,122ft)

      - Observe (63PPM / 19PPF): Wide: 52.2m(171.4ft) / Tele: 1,600m(5,249ft)

      - Recognize (125PPM / 38PPF): Wide: 26.1m(85.7ft) / Tele: 800m(2,625ft)

      - Identify (250PPM / 76PPF): Wide: 13.1m(42.8ft) / Tele: 400m(1,312ft)

    - Certification

      - EMC

        - FCC 47 CFR 15 Subpart B Class A

        - ICES-3(A)/NMB-3(A)

        - CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

        - VCCI CISPR 32 Class A

        - RCM AS/NZS CISPR 32 Class A

      - Safety

        - UL 62368-1, CAN/CSA C22.2 NO. 62368-1, IEC/EN 62471

      - Environment

        - IEC/EN 63000, IEC/EN 60529 IP66/67, IEC/EN 62262 IK10

    - Wisenet Road AI LPR/ANPR/MMCR

      - APP Type			CNN (Convolutional Neural Network) AI

      - LPR Usage conditions		City Traffic

      - Speed Description		High Speed

      - Lane Coverage

        - NA

          - 1 lane, 3.6m/12ft wide (with built in IR)

          - 2 lanes, 8.1m/26ft wide (with built in IR)

        - EU

          - 1 lane, 3.6m/12ft wide (with built in IR)

          - 2 lanes, 8.1m/26ft wide (with built in IR)

          - 3 lanes, 16.2m/53ft wide (with built in IR)

      - Plate Capture Resolution	4K

      - Speed Limit

        - NA

          - 1 lane, up to 250km/h (155mph)

          - 2 lanes, up to 160km/h (99mph)

        - EU

          - 1 lane, up to 250km/h (155mph)

          - 2 lanes, up to 200km/h (124mph)

          - 3 lanes, up to 150km/h (93mph)

      - Minimum Forward Distance

        - NA

          - 1 lane, 13m (42.6ft)

          - 2 lanes, 33m (108.2ft)

        - EU

          - 1 lane, 7m (22.9ft)

          - 2 lanes, 20m (65.6ft)

          - 3 lanes, 33m (108.2ft)

      - Maximum Forward Distance	70m (229.6ft)

      - Maximum Horizontal Angle	45º

      - Maximum Vertical Angle		30º

      - Vehicle Recognition		100+ Makes, 4700+ Models, 11 Colors

      - Available Countries		United States of America, Canada, Peru(LPR only), Colombia(LPR only), Albania, Armenia, Austria, Azerbaijan, Belarus, Belgium, Bosnia and Herzegovina, Bulgaria, Croatia, Cyprus, Czech Republic, Denmark, Estonia, Finland, France, Gibraltar, Georgia, Germany, Greece, Hungary, Iceland, Ireland, Israel, Italy, Kosovo, Kazakhstan, Kyrgyzstan, Latvia, Liechtenstein, Lithuania, Luxembourg, Macedonia, Malta, Moldova, Monaco, Mongolia, Montenegro, Netherlands, Norway, Poland, Portugal, Romania, Russian Federation, San Marino, Serbia, Slovakia, Slovenia, Spain, Sweden, Switzerland, Tajikistan, Turkey, Turkmenistan, Ukraine, United Kingdom, Uzbekistan, Australia(LPR only), New Zealand (LPR only), Bahrain, Kuwait, Oman, Qatar, KSA, UAE, South Africa, Hong Kong(LPR only), Indonesia, Vietnam, Taiwan(LPR only)

END OF SECTION

10