Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com)

    - Model: QNE-C8013RL

    - Alternates: None

    - Note: This specification is based on the datasheet dated 21 September 2026 (Ver 202409). Design and specifications are subject to change without notice.

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264: Max. 30fps/25fps (60Hz/50Hz) at all resolutions

        - MJPEG: Max. 30fps (@5MP Max. 5fps)

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - 2592x1944, 2560x1440, 1920x1080, 1280x960, 1280x720, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - The camera shall support unicast video streaming up to 20 users.

      - The camera shall support multicast video streaming.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStreamⅢ, Dynamic GOV and Dynamic fps to efficiently manage bit rate of the video stream and reduce storage.

      - The camera shall provide WiseNRⅡ that works based on AI engine to reduce noise on image.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IP67, IK10 rated for protection against impacts.

      - True day/night operation with removable IR cut filter.

      - The camera shall be able to produce clear images in highly contrast scenes with wide dynamic range up to 120dB.

      - The camera shall be able to configure 32 privacy masking areas with 4-point quadrangle zones.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Analytics events based on AI engine

        - Object detection: Person / Vehicle (Car / Bus / Truck / Motorcycle / Bicycle)

        - Virtual line (Crossing / Direction)

        - Virtual area (Loitering / Intrusion / Enter / Exit)

        - Motion detection

      - Analytics events: Defocus detection, Tampering, Virtual area (Appear / Disappear)

      - Motion detection with 8 definable detection areas with 8-point polygonal zones.

      - Defocus detection – an event can be created when the camera lens is detected to be out of focus.

      - Tampering detection – an event can be created when the screen is blocked or the camera position is changed.

      - Classified object types are Person and Vehicle.

        - Person: Upper/Lower clothes color

        - Vehicle: Type (car / bus / truck / motorcycle / bicycle), Color

      - The camera shall support BestShot thumbnail capture per detected object.

      - Business Intelligence based on AI engine.

        - People counting, Queue management, Heatmap, Vehicle counting

    - Interoperability – The camera shall be ONVIF Profile S, G, T and M compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via non-plugin browsers including Google Chrome, Microsoft Edge, Mozilla Firefox and Apple Safari.

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals.

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals.

      - Alarm triggers and notifications, including MQTT publication.

      - Pixel Counter available in the web viewer.

      - PoE capable, including heater operation powered by PoE.

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device: 1/2.8" CMOS, progressive scan

      - Minimum illumination

        - Color (1/30sec, 30IRE): 0.022 Lux

        - BW (1/30sec, 30IRE): 0.0022 Lux

        - 0 Lux (IR LED on)

    - Lens

      - Focal length: 3.0mm

      - Max. Aperture Ratio: F1.2

      - Angular Field of View

        - H: 97°

        - V: 71°

        - D: 130°

      - Min. Object Distance: 2.0m (6.56ft)

      - Focus Control: Fixed

      - Lens Type: Fixed iris

      - Mount Type: M12

    - Pan & Tilt & Rotate

      - Pan Range: 0° ~ 350°

      - Tilt Range: 0° ~ 75°

      - Rotate Range: 0° ~ 360°

    - Operational Functions

      - Camera Title: Off / On (displayed up to 85 characters)

      - Day & Night: Auto (ICR) / Color / B/W / Schedule

      - Backlight Compensation: BLC, WDR, SSDR

      - Wide Dynamic Range: 120dB

      - Digital Noise Reduction: WiseNRⅡ (Based on AI engine), SSNRⅤ

      - Motion Detection: 8ea, 8-point polygonal zones

      - Privacy Masking: 32ea, 4-point quadrangle zones (Color/Option: Gray, Green, Red, Blue, Black, White, Mosaic; Dynamic Privacy Masking option: Mosaic)

      - Gain Control: Low / Middle / High

      - White Balance: ATW / AWC / Manual / Indoor / Outdoor

      - Lens Distortion Correction: Support

      - Electronic Shutter Speed: Minimum / Maximum / Anti-flicker, Prefer shutter control (Based on AI engine); Speed range 1/5 ~ 1/25,000sec

      - Digital PTZ: Support (Preset, Group)

      - Video Rotation: Flip, Mirror, Hallway view (90° / 270°)

      - Alarm Triggers: Analytics, Network disconnect, MQTT subscription

      - Alarm Events (when alarm trigger occurred)

        - File upload (image): e-mail / FTP

        - Notification: e-mail

        - Recording: SD/SDHC/SDXC or NAS recording at event triggers

        - Handover: PTZ preset, send message by HTTP / HTTPS / TCP

        - MQTT: publication

        - Light alarm

      - Light Type: Warm color light, IR LED (integrated dual light)

      - Light Viewable Length

        - Warm light: 30m (98.42ft)

        - IR: 30m (98.42ft)

      - IR Wavelength: 850nm

      - Pixel Counter: Support

      - Storage

        - Edge Storage: Micro SD/SDHC/SDXC, Max. 256GB x 1 slot

        - Edge Storage (OnCloud): Micro SD/SDHC/SDXC, Max. 1TB x 1 slot. Hanwha Vision branded microSD cards are certified and recommended for OnCloud edge recording.

      - Memory: 2GB RAM, 1GB Flash

    - Network

      - Ethernet: RJ-45 (10/100 BASE-T)

      - Video compression: H.265 / H.264 (Main, High), MJPEG

      - Smart Codec: Manual, WiseStreamⅢ (Based on AI engine)

      - Bitrate Control Method

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming

        - Unicast: up to 20 users

        - Multicast: Support

        - Multiple streaming: up to 5 profiles

      - Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP, SFTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, CDP, SRTP(TCP, UDP Unicast), MQTT

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings.

      - QoS – Layer 3 DSCP

      - Application Programming Interface

        - ONVIF Profile S / G / T / M

        - SUNAPI (HTTP API)

        - Hanwha Vision Open Platform

      - Direct Cloud Connection: Support

    - Security Feature

      - OS / Firmware Protect: Encrypted firmware, Secure boot, Signed firmware

      - User Authentication: Digest authentication, Prevent brute-force attack

      - Network Authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

      - Secure Communication: HTTPS, WSS (WebSocket Secure), SRTP

      - Access Control: IP-based access control

      - Data Protect: Encryption credentials, encrypt compress for live recording file export

      - Audit: Access / System / Event log management

      - Device ID: Device certificate (Hanwha Vision Root CA)

      - Secure Storage: SD card partition encryption (AES-256)

    - General

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Input Voltage: PoE (IEEE802.3af, Class3)

      - Power Consumption: Max. 10.0W, typical 4.5W

    - Mechanical And Environmental

      - Color: White

      - Material: Aluminum

      - RAL Code: RAL9003

      - Dimensions (Diameter x Height): ø140.0 x 99.6mm (ø5.51 x 3.92")

      - Weight: 790g (1.74lb)

      - Compatible Gang Box: Single, Double, 4" Octagon

      - Operating Temperature & Humidity: -40°C ~ +60°C (-40°F ~ +140°F) / 0~100% RH (condensing). Start-up shall be performed at or above -30°C. Humidity control with AIR vent.

      - Storage Temperature & Humidity: -50°C ~ +60°C (-58°F ~ +140°F) / 0~95% RH (non-condensing)

    - DORI Distance (EN 62676-4 standard)

      - Detect (25PPM / 8PPF): 45.9m (150.49ft)

      - Observe (63PPM / 19PPF): 18.3m (60.19ft)

      - Recognize (125PPM / 38PPF): 9.2m (30.09ft)

      - Identify (250PPM / 76PPF): 4.6m (15.05ft)

    - Certification

      - EMC

        - FCC 47 CFR 15 Subpart B Class A

        - ICES-3(A)/NMB-3(A)

        - CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

        - VCCI CISPR 32 Class A

        - RCM AS/NZS CISPR 32 Class A

      - Safety

        - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

        - IEC/EN 62471

      - Environment

        - IEC/EN 63000

        - IEC 60529 IP66, IEC 60529 IP67

        - IEC 62262 IK10

END OF SECTION

8