Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com, https://hanwhavisionamerica.com)

    - Model: QNF-C9010

    - Alternates: none

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264

          - Original view: Max.20fps@3008x3008,

          - Double panorama: Max.20fps@3584x1792

          - Single panorama: Max.20fps@3584x896

          - Quad view: Max.20fps@3584x2688

          - Q1/Q2/Q3/Q4(4:3): Max.20fps@1792x1344

        - MJPEG – Max. 5fps

      - The camera shall be able to configure up to 5 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - Overview: 3008x3008, 2560x2560, 1920x1920, 1504x1504, 1280x1280, 800x800, 640x640, 480x480

        - Double panorama: 3584x1792, 1920x960, 1280x640, 1024x512, 640x320

        - Single panorama: 3584x896, 1920x480, 1280x320, 1024x256, 640x160

        - Quad view: 3584x2688, 2048x1536, 1792x1344, 1024x768, 768x576, 512x384

        - Q1/Q2/Q3/Q4(4:3): 1792x1344, 1024x768, 768x576, 512x384

      - The camera shall support multicast and unicast video streaming up to 20 users.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide smart codec(WiseStream Ⅲ: Based on AI engine) to efficiently manage bit rate of the video stream and reduce storage while producing video quality that is visually equal to the one without smart codec.

    - Camera – The camera device shall have the following physical and performance properties:

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 120dB.

      - Automatic, manual, scheduled day and night operation. Images are available in color or black and white.

        - Low light level operation to 0.02Lux (F1.8, 1/30sec) in color mode

        - Low light level operation to 0.001Lux (F1.8, 1/30sec) in b/w mode

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - Configurable 32 privacy masking regions utilizing 4 points quadrangle zones.

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Classified object type: Person/Vehicle

      - Attributes: Person(Upper/lower clothes color), Vehicle(color)

      - Support BestShot

      - Analytics events based on AI engine: Motion detection*, Object detection, Virtual line *(Crossing/Direction), Virtual area *(Loitering/Intrusion/Enter/Exit/Appear/Disappear) * Some of the video analytics only works with people and vehicle detection

      - Analytics events: Defocus detection, Tampering

      - Business Intelligence: (Based on AI engine) People counting, Queue management, Heatmap

    - Interoperability – The camera shall be ONVIF Profile S, G and M compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers: Video & Audio analytics, Network disconnect

        - available notification means upon trigger: File Upload via FTP and E-mail, Notification via E-mail, Local storage (SD / SDHC / SDXC) or NAS recording at event triggers

      - NDAA Compliant

      - PoE capable

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging Device: 1/1.6" 12MP CMOS

      - Resolution

        - Overview: 3008x3008, 2560x2560, 1920x1920, 1504x1504, 1280x1280, 800x800, 640x640, 480x480

        - Double panorama: 3584x1792, 1920x960, 1280x640, 1024x512, 640x320

        - Single panorama: 3584x896, 1920x480, 1280x320, 1024x256, 640x160

        - Quad view: 3584x2688, 2048x1536, 1792x1344, 1024x768, 768x576, 512x384

        - Q1/Q2/Q3/Q4(4:3): 1792x1344, 1024x768, 768x576, 512x384

      - Maximum Framerate

        - Original view: Max. 20fps@3008x3008,

        - Double panorama: Max. 20fps@3584x1792

        - Single panorama: Max. 20fps@3584x896

        - Quad view: Max.20fps @3584x2688

        - Q1/Q2/Q3/Q4(4:3): Max. 20fps@1792x1344

        - MJPEG: Max.5fps

      - Minimum Illumination

        - Color: 0.02Lux (F1.8, 1/30sec)

        - B/W: 0.001Lux (F1.8, 1/30sec)

      - Video Out (Installation): Micro USB Type B, 1280x720 for installation

    - Lens

      - Focal Length: 1.76mm

      - Max. Aperture Ratio: F1.8

      - Angle of view: H 185˚, V 185°, D 185°

      - Min. Object Distance: 0.5m (1.64ft)

      - Lens Type: Fixed

    - Pan / Tilt / Rotate

      - Pan / Tilt / Rotate Range: Digital Rotation

    - Operational

      - Camera Title: Off / On (Displayed up to 85 characters)

      - Day/Night: Auto(ICR)

      - Backlight Compensation: BLC / WDR / SSDR

      - WDR: 120dB

      - Digital Noise Reduction: SSNR V, WiseNRⅡ(Based on AI engine)

      - Digital Image Stabilization: None

      - Defog: none

      - Motion Detection: 8ea, 8 points polygonal zones

      - Privacy Masking: 32ea, 4 points quadrangle zones

      - Gain control: Low / Middle / High

      - White Balance: ATW / AWC / Manual / Indoor / Outdoor (Including Mercury and Sodium)

      - Lens Distortion Correction: support

      - Electronic Shutter Speed: Min / Max / Anti-flicker (1/5 ~ 1/12,000sec)

      - Digital PTZ: support

      - Video Rotation: Flip, Mirror

      - Analytics

        - Classified object type: Person/Vehicle

        - Attributes: Person(Upper/lower clothes color), Vehicle(color)

        - Support BestShot

        - Analytics events based on AI engine: Motion detection*, Object detection, Virtual line *(Crossing/Direction), Virtual area *(Loitering/Intrusion/Enter/Exit/Appear/Disappear) * Some of the video analytics only works with people and vehicle detection

        - Analytics events: Defocus detection, Tampering

      - Business Intelligence: (Based on AI engine) People counting, Queue management, Heatmap

      - Alarm Triggers: Analytics, Network disconnect

      - Alarm Events (When alarm trigger occurred)

        - When alarm trigger occurred

        - File upload(image): e-mail/FTP/SFTP

        - Notification: e-mail

        - Recording: SD/SDHC/SDXC or NAS recording at event triggers

        - Handover (PTZ preset, Send message by HTTP/HTTPS/TCP)

        - MQTT: publication

      - Webpage Language: English, Korean, Simplified Chinese, Traditional Chinese, French, Italian, Spanish, German, Japanese, Russian, Portuguese, Czech, Polish, Turkish, Dutch, Hungarian, Greek

      - Edge Storage: Micro SD/SDHC/SDXC 1slot 256GB

      - Memory: 2GB RAM, 1GB Flash

    - Network

      - Ethernet: RJ-45 connector(10/100 Base-T)

      - Video Compression

        - H.265 / H.264: Main / High

        - MJPEG

      - Smart Codec: Manual(5ea area), WiseStreamⅢ(Based on AI engine)

      - Video Quality Adjustment

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming: Unicast (20 users) / Multicast, Multiple streaming (up to 5 profiles)

      - Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP/SFTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, CDP, SRTP (TCP, UDP Unicast), MQTT

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Application Programming Interface

        - ONVIF Profile S / G / T / M

        - SUNAPI (HTTP API)

        - Hanwha Vision Open Platform

      - Security Feature

        - OS / Firmware Protect: Secure boot, Signed Firmware, Firmware encryption

        - User authentication: Digest Authentication, Prevent brute-foce attack

        - Network authentication: IEEE 802.1X Authentication (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

        - Secure Communication: HTTPS, SRTP, WSS (WebSocket Secure)

        - Access Control: Access control based on IP address

        - Data Protect: Authentication information encryption, ZIP compression encryption

        - Audit: User Access / System / Event Log management

        - Device ID: Device certificate (Hanwha Private Root CA)

        - Secure Storage: SD card partition encrypt

        - Security Certificate: Secure by default

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings

    - Compatible Models

      - Hanging Adaptor: SBP-099HMW

      - Ceiling Mount(Assy): SBP-150CMI/300CMI, SBP-300CMW1/900CMW, SBP-300CMTW, SBP-300CMTS

      - Ceiling Mount(Single Unit): SBP-140CMB/180CMB/180CMS, SBP-150CMP/300CMP/900CMP, SBP-C15P, SBP-C15H

      - Wall Mount: SBP-125WMW1, SBP-250WMW, SBP-400WMW, SBP-300WMW/300WMW1

      - Wall Mount Adaptor: SBP-115PFA, SBP-300BW1

      - Pole Mount: SBD-140PMB, SBP-300PMW2

      - In-Ceiling Mount: SHD-1128FPW

      - Corner Mount: SBD-140KMB, SBP-156KMW, SBP-300KMW1

      - Parapet Mount: SBP-300LMW, SBP-156LMW1

      - Tilt Mount: SBV-099TMW

      - Cabinet: SBP-300NBW

      - Gang Plate: SBD-110GP1

    - Environmental & Electrical

      - Temperature

        - Operating: -10°C to 40°C (+14°F to 104°F)

        - Storage: -50°C to 60°C (-58°F to 140°F)

      - Humidity: Less than 90% RH

      - Power

        - Input Voltage: PoE (IEEE 802.3af, class3)

        - Power Consumption: Max 7.3W, typical 4.3W

    - Mechanical

      - Material: White / Aluminum

      - RAL Code: RAL9003

      - Dimensions (D x H): Ø99 x 52mm (ø3.9x2.05")

      - Weight: 360g(0.79lb)

    - Certification

      - EMC & Safety

        - FCC 47 CFR 15 Subpart B Class A

        - ICES-3(A)/NMB-3(A)

        - CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

        - VCCI CISPR 32 Class A

        - RCM AS/NZS CISPR 32 Class A

        - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

        - IEC/EN 62471

      - Environment

        - IEC/EN 63000

        - IEC/EN 60529 IP42

    - DORI Distance (en62676-4 standard)

      - Detect: 36.9m (121.03ft)

      - Observe: 14.8m (48.42ft)

      - Recognize: 7.4m (24.21ft)

      - Identify: 3.7m (12.10ft)

END OF SECTION

8