SBP 301HF PTZ MOUNTS

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SBP 301HF PTZ MOUNTS

- This PTZ mount has built-in SFP (Small Form-Factor Pluggable) for PTZ cameras.

- This PTZ mount is Ivory color in appearance with aluminum body.

2.03	Compatible Model

| A. Camera List | SNP-6321H/5321H |
| --- | --- |

2.04	External Connection

| A. Alarm I/O | Input 4ea / Output 2ea |
| --- | --- |
| B. Remote Control Interface | RS-485/422 |
| C. Audio | Input / Output, 3.5mm type |
| D. CVBS | BNC type, For installation |
| E. Power Line | 24V AC, 2-wire |

2.05	Optical Characteristics

| A. Support SFP Voltage | 3.3V typical |
| --- | --- |
| B. Support SFP Current | Max. 300mA |
| C. Support SFP Cage | Standard mini-SFP cage (For 6.5mm pitch SFP) |
| D. Support SFP Port Type | LC type |
| E. Required SFP Speed | 1250Mbps (For 1000Mbps Ethernet), 155Mbps (For 100Mbps Ethernet) |
| F. Required SFP's BER (Bit Error Rate) | Max. 10^-12 |
| G. Transmission Method (Optical) | Mode depends on SFP module |
| H. Optical Mode (Long / Short Wave) | Mode depends on SFP module |
| I. Cable Insertion Loss (dB) | Mode depends on SFP module |

2.06	Network Characteristics

| A. RJ-45 : Ethernet Speed | 100Mbps |
| --- | --- |
| B. RJ-45 : Ethernet Physical Layer | 100 Base-T |

| C. RJ-45 : Ethernet Standard | IEEE 802.3u support |
| --- | --- |
| D. Speed Transition (100 1000Mbps) | External Dip switch |
| E. Fiber : Default Speed | 1000Mbps |

| F. Fiber : Ethernet Speed | 100Mbps, 1000Mbps |
| --- | --- |
| G. Fiber : Ethernet Physical Layer | 100 Base-FX, 1000 Base-X |
| H. Fiber : Ethernet Standard | IEEE 802.3u, IEEE 802.3z support |

2.07	Environmental

| A. Operating Temperature / Humidity | -50°C ~ +55°C (+14°F ~ 131°F) / Less than 90% RH |
| --- | --- |
| B. Ingress Protection | IP66 |
| C. Vandal Resistance | IK10 |

2.08	Electrical

| A. Power Consumption | Max. 65W, 3.1A (Heater on) |
| --- | --- |
| B. Input Voltage / Current | 24V AC |

2.09	Mechanical

| A. Dimension (WxHxD) | Ø163.9 x 157.9mm (Ø6.45" x 6.22"), Ø163.9 x 187.9mm (Ø6.45" x 6.7") (With NPT-Coupling) |
| --- | --- |
| B. Weight | 1.1Kg (2.43 lb.), 1.4Kg (3.09 lb.) (With NPT-Coupling) |
| C. Color/Material | Ivory / Aluminum |

2.10	WARRANTY

A. 3 years, parts and labor.