Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com)

    - Model: SCB-6005

    - Alternates: None

    - Note: This specification is based on the datasheet dated 5 September 2025. Design and specifications are subject to change without notice.

  - GENERAL DESCRIPTION

    - Video and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - 2 MP resolution, 30fps (NTSC) / 25fps (PAL).

      - Video signals shall be output in AHD or CVBS (selectable) format via BNC terminal.

      - Video shall be capable of transmission up to 500 meters using 5C2V coaxial cable.

    - Camera – The camera device shall have the following physical and performance properties:

      - True day/night operation with removable IR cut filter.

        - Low light level operation to 0.04 lux in color mode and 0.004 lux in black and white mode at F1.2, 1/30sec.

      - The camera shall be able to produce clear images in highly contrast scenes with digital wide dynamic range.

      - The camera shall provide digital noise reduction using SSNRⅣ technology.

      - The camera shall provide a defog function.

      - The camera shall be able to configure 8 privacy masking areas with rectangular zones.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Motion detection with 4 definable detection areas.

      - The camera shall be controllable by ACP or Coaxitron over the coaxial video line, and by RS-485 serial interface.

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device: 1/2.8" CMOS, progressive scan

      - Pixels per sensor: 1,945(H) x 1,109(V) total; 1,945(H) x 1,097(V) effective

      - Minimum illumination

        - Color (1/30sec, 30IRE): 0.04 Lux

        - BW (1/30sec, 30IRE): 0.004 Lux

      - Signal to noise ratio: 52dB

      - Video Out: BNC (AHD / CVBS selectable)

      - Resolution: 1920x1080

      - Maximum frame rate: 30fps (NTSC) / 25fps (PAL) at 1920x1080

      - Video Transmission Distance: 500m (1,640.4ft) with 5C2V coaxial cable

    - Lens

      - Lens Type: DC auto iris (IR corrected)

      - Focus Control: Manual

      - Mount Type: C mount, CS mount

    - Operational Functions

      - On Screen Display: displayed up to 15 characters

      - Day & Night: Auto (ICR) / External / Color / B/W

      - Backlight Compensation: Off / User BLC / HLC

      - Wide Dynamic Range: D-WDR (Off / On)

      - Digital Noise Reduction: SSNRⅣ

      - Defog: Support

      - Motion Detection: Off / On (4 zones)

      - Privacy Masking: Off / On (8 zones, rectangular)

      - Gain Control: Off / Low / Middle / High / Very High

      - White Balance: ATW / Outdoor / Indoor (4,500K° ~ 8,500K°) / Manual / AWC (1,800K° ~ 10,500K°)

      - Electronic Shutter Speed: 1 ~ 1/12,000sec

      - Video Rotation: Flip, Mirror, Hallway view (90° / 270°)

      - Alarm I/O: Output 1ea; External Day/Night input

      - Alarm Triggers: Analytics

      - Alarm Events (when alarm trigger occurred): Alarm output

      - Serial Interface: RS-485

      - Coaxial Protocol

        - AHD: ACP (AHD Coax Protocol)

        - CVBS: Pelco-C (Coaxitron)

        - RS-485: Samsung-T/E, Pelco-D/P, Panasonic, Bosch, Honeywell, Vicon, AD, GE

    - Electrical

      - Input Voltage: 24VAC (50Hz/60Hz), 12VDC

      - Power Consumption

        - 12VDC: 0.32A (Max 3.5W)

        - 24VAC: 0.37A (Max 3.6W)

    - Mechanical And Environmental

      - Color: Ivory, Black

      - Material: Aluminum, PC (Polycarbonate)

      - Dimensions (W x H x D): 68.4 x 59.6 x 92.7mm (2.69" x 2.35" x 3.65")

      - Weight: 186g (0.41lb)

      - Operating Temperature & Humidity: -10°C ~ +55°C (+14°F ~ +131°F) / 0~90% RH (non-condensing)

    - Certification

      - EMC

        - FCC 47 CFR 15 Subpart B, ANSI C63.4-2014, Class A

        - CE EN 55022:2010

        - IC Regulation ICES-003:2016, CAN/CSA CISPR 22-10 Class A

        - AS/NZS CISPR 32:2013 Class A

        - VCCI V-3/2015.04 Class A

        - EN 55032:2012/AC:2013 Class A

        - EN 50130-4:2011 +A1:2014

      - Safety: UL Listed

      - Environment: EN 50581:2012 (hazardous substances)

END OF SECTION

8