# Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

# For additional information, visit http://security.hanwhatechwin.com/

1080p HD-SDI IR DOME CAMERA

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

# MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00 Data Communications

# 28 23 13 Video Surveillance Control and Management Systems

# 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19 Digital Video Recorders and Analog Recording Devices

# 28 23 23 Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

28 2x 00	Video Management System

1080p HD-SDI IR DOME CAMERA

- GENERAL

  - SUMMARY

    - Section includes 1080p analog camera in dome housing.

    - Product - A 1080p analog camera with integral IR illuminator.

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Back light compensation.

      - CBR – Constant Bit Rate

      - CVBS – Color, Video, Blanking, Sync

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - DSCP – Differentiated Services Code Point

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GOV – Group of Video

      - GUI – Graphical User Interface

      - HD – High Definition

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - MP - Megapixel

      - MPEG - Moving Pictures Experts Group

      - NAS – Network Attached Storage

      - NTP - Network Time Protocol

      - PIM-SM - Protocol Independent Multicast-Sparse Mode

      - PoE - Power over Ethernet

      - PPPoE – Point to Point Protocol over Ethernet

      - RTP - Real-Time Transport Protocol

      - RTCP – Real-Time Control Protocol

      - RTSP - Real-Time Streaming Protocol

      - SDK – Software Development Kit

      - SMTP - Simple Mail Transfer Protocol

      - SNMP – Simple Network Management Protocol

      - SSL – Secure Sockets Layer

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - UPnP – Universal Plug and Play

      - VBR – Variable Bit Rate

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

      - DWDR – Digital Wide Dynamic Range

    - Reference Standards

      - Emissions

        - FCC-47 CFR Part 15 Subpart B Class B

        - CE EN 55022:2010

      - Immunity - CE

        - EN 50130-4:2011

        - EN 61000-3-3:2014

        - EN 61000-4-2:2009

        - EN 61000-4-3:2006+A2:2010

        - EN 61000-4-4:2012

        - EN 61000-4-5:2014

        - EN 61000-4-6:2009

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide video analytics embedded in the camera free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://security.hanwhatechwin.com/

    - Model		SCD-6081R

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - Video – Full HD (1080p) resolution at 30 fps and HD (720p) at 60 fps.

    - Camera – The 1080p analog camera device shall have the following physical and performance properties:

      - True day/night operation with removable IR cut filter

        - Low light level operation to 0.015 lux (color mode) and 0 lux (black and white)

      - 100dB Wide Dynamic Range, providing a wide range between dark and light areas visible at the same time

      - Digital noise reduction

      - Digital image stabilization

      - Digital zoom

      - Integral IR illumination, providing effective visibility of 15m at 0 Lux when activated in Black & White mode.

      - Sens-up providing high sensitivity in low light conditions without the need for artificial light.

      - 20 privacy masking regions utilizing a 4 or more point polygon

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with a definable detection area, minimum / maximum object size definition, alarm output, and a learning algorithm that ignores false alarms such as trees and waves on water.

      - Detection of logical events of specified conditions from the camera’s video input

        - line crossing (passing)

        - appear/disappear

        - camera tamper (scene change)

        - enter/exit a predefined zone

  - DETAILED SPECIFICATIONS

    - Video

      - Imager

        - Sensor:			1/3" CMOS

          - pixels per sensor:	2010 (H) x 1108 (V) total; 1944 (H) x 1092 (V) effective

          - scanning	:	progressive

        - Minimum illumination

          - Color: 			1Lux (F1.2), 0.015Lux (Sen-up 60x)

          - Black and White	0Lux (IR LED on)

        - Signal to Noise Ratio	50dB (AGC off, Weight on)

        - Video Output:		SMPTE292M (HD-SDI)

        - Resolution			1920 x 1080, 1280 x 720

        - Maximum Frame rate	30fps at 1080p, 60fps at 720p

        - The following features with control settings shall be available:

          - On Screen Display			English, Korean, French, German, Spanish,

Italian, Chinese, Russian, Polish, Swedish, Danish, Portuguese, Japanese, Serbian, Czech, Romanian, Turkish

          - Camera Title: 				On / Off (up to 15 characters)

          - Day / Night				Auto (ICR) / External / Color / BW

          - Backlight compensation (BLC):		Off / User BLC / HLC / WDR

          - Wide Dynamic Range			100dB

          - Contrast Enhancement			On / Off (Samsung Super Dynamic Range)

          - Digital Noise Reduction (DNR):		On / Off (Samsung Super Noise Reduction III)

          - Digital Image Stabilization		On / Off

          - Motion detection

            - selectable for On/Off

          - Privacy masking

            - 16 zones with 4 sided polygon

            - 4 polygonal zones

            - selectable for On/Off

          - Sens-up (Frame Integration)		2x ~ 60x

          - Gain control				3 levels or Off

          - White Balance				ATW / Outdoor / Indoor / Manual / AWC

(1,700K° ~ 11,000K°)

          - Electronic shutter speed: 		ESC / A.FLK / Manual.

          - Digital Zoom				On / Off (1x ~ 16x)

          - Reverse:				Off / H-Rev / V-Rev / HV-Rev

          - Profile					Built-in

          - PIP					On / Off

        - Lens:					3 ~ 8.5 mm (2.8x) motorized varifocal

          - Maximum aperture ratio:   		F1.2

          - Angle of view:				H : 105.5°(Wide) ~ 37.1°(Tele)

V : 57.5°(Wide) ~ 21.0°(Tele)

          - Minimum Object Distance: 		0.5 m (1.64ft.)

          - Lens type:				DC auto iris

          - Mount type:				Board-in

        - Manual positional settings:

            - pan:		0 – 354o

            - tilt:			0 – 67o

            - rotation		0 – 355o

      - IR Viewable Length		15m (49.21ft, IR LED : 12ea)

    - Protocol

      - Remote Control Interface: 	RS-485

      - Protocols:			RS-485 : Samsung-T/E, Pelco-D/P, Panasonic, Bosch,

Honeywell, Vicon, AD, GE

    - Alarm

      - Motion Detection 		1 output

      - External Day and Night		1 input

    - Electrical

        - Input Voltage and Current	Dual, 24V AC and 12V DC +-10%

        - Power Consumption	Maximum 6W

    - Mechanical And Environmental

      - Material: 			plastic

      - Dimensions (W x H):  		132.1 x 107.6 mm (5.2 x 4.24 in.)

      - Weight:				525g (1.16 lb.)

      - Operating Temperature: 	-10° C to 55° C (+14° F to 131° F)

      - Humidity:			0 - 90%, non-condensing

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The design and configuration shall be verified for compatibility and performance with the camera(s).

    - Configuration shall be tested and qualified by the Contractor prior to camera installation.

    - All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA), Digital Video Recorder (DVR), or Network Video Recorder (NVR).

  - INSTALLATION

    - The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

    - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

    - The camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

8