SCO-5083RP 1000TVL (1280H) WEATHERPROOF IR CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETIES AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SCO-5083RP 1000TVL (1280H) WEATHERPROOF IR CAMERA

- This analog camera features high resolution of 1000TV lines in black and white as well as in color.

- Minimum illumination of this camera is 0Lux with IR led on. IR distance is 50m.

- This camera has 3.3x varifocal lens which varies from 3 to 10 mm.

- This camera features Wide Dynamic Range (WDR) which can record wider range of light than normal hence provide clear image even in backlighting.

- This camera features Day & Night mode that incorporates an infrared cutfilter removal (ICR) mechanism for greater light sensitivity and true color reproduction in the low light environment.

- This camera features Samsung Super Dynamic Range (SSDR) which enhances brightness of picture in dark areas.

- This camera provides Samsung Super Noise Reduction IV (SSNR IV).

- This camera provides Defog mode which intensifies color and contrast to produce higher quality video in foggy weather.

- This camera provides intelligent video analytics such as Fence, Appear/Disappear, and Counting.

- This camera has IP66 rating which shows high degree of protection against dust and liquids.

- This camera has IK10 rating which shows high degree of protection against vandalism.

- The protocols used by this camera are Pelco-C (Coaxitron), RS-485, and Dual Power.

2.03	VIDEO

| A. Imaging Device |  | 1/3" 1.3M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,312(H) x 1,069(V) |
| C. Effective Pixels |  | 1,305(H) x 1,049(V) |
| D. Scanning System |  | Progressive scan |
| E. Synchronization |  | Internal |
| F. Frequency |  | H : 15.625KHz / V : 50Hz |
| G. Horizontal Resolution |  |  |
|  | Color | 1000TV lines |
|  | B/W | 1000TV lines |
| H. Min. Illumination |  |  |
|  | Color | 0.05Lux (F1.4, 50IRE), 0.001Lux (1sec, F1.4, 50IRE), 0.02Lux (F1.4, 30IRE), 0.0006Lux (1sec, F1.4, 30IRE) |
|  | B/W | 0Lux (IR LED on) |
| I. S / N Ratio |  | 52dB (AGC off, Weight on) |
| J. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite |

2.04	LENS

| A. Focal Length (Zoom Ratio) | 3 ~ 10mm (3.3x) varifocal |
| --- | --- |
| B. Max. Aperture Ratio | F1.4 |
| C. Angular Field of View | H : 82.0°(Wide) ~ 26.5°(Tele) V : 59.7°(Wide) ~ 19.9°(Tele) |
| D. Min. Object Distance | 0.5m (1.64ft) |
| E. Focus Control | Manual |
| F. Lens Type | DC auto iris |
| G. Mount Type | Board type |

2.05	OPERATION

| A. On Screen Display |  | English, Chinese, Korean, Japanese, German, Italian, French, Spanish, Russian, Czech, Polish, Romanian, Serbian, Swedish, Danish, Turkish, Portuguese |
| --- | --- | --- |
| B. Camera Title |  | Off / On (Displayed 15 characters) |
| C. Day & Night |  | Auto (ICR) / External / Color / B/W |
| D. Anti IR Saturation |  | Top / Bottom / Left / Right |
| E. Backlight Compensation |  | Off / User BLC / HLC / WDR |
| F. Wide Dynamic Range |  | 120dB |
| G. Contrast Enhancement |  | SSDR (Off / On) |
| H. Digital Noise Reduction |  | SSNR IV (Off / On) |
| I. Defog |  | Off / Auto / Manual |
| J. Purple Fringe Reduction |  | Off / Low / Middle / High |
| K. Digital Image Stabilization |  | Off / On |
| L. Tampering |  | Off / On |
| M. Motion Detection |  | Off / On |
| N. Intelligent Video Analytics |  | Fence, (Dis)Appear, Counting (Off / On) |
| O. Privacy Masking |  | Off / On (24 programmable zones with 4 points polygonal masking) |
| P. Gain Control |  | Off / Low / Middle / High |
| Q. White Balance |  | ATW / Outdoor / Indoor / Manual / AWC / Mercury (1,800K° ~ 10,500K°) |
| R. Electronic Shutter Speed |  | 1sec ~ 1/12,000sec |
| S. Digital Zoom |  | Off / On (1x ~ 16x) |
| T. Reverse |  | Off / H-Rev / V-Rev / HV-Rev |
| U. Profile |  | Basic, Day & Night, Backlight, ITS, Indoor, User |
| V. Alarm |  | 1 Out |
| W. Communication |  | Coaxial control (SPC-300 compatible) |
| X. Protocol |  |  |
|  | Coax | Pelco-C (Coaxitron) |
|  | RS-485 | Samsung-T/E, Pelco-D/P, Panasonic, Bosch, Honeywell, Vicon, AD, GE |
| Y. IR Distance |  | 50m (164.04ft) (IR LED 36ea) |

2.06	ENVIRONMENTAL

| A. Operating Temperature / Humidity | -30°C ~ +55°C (-22°F ~ +131°F) / Less than 90% RH *Start up should be done at above -10°C (+14°F) |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |
| C. Ingress Protection | IP66 |
| D. Vandal Resistance | IK10 |

2.07	Electrical

| A. Input Voltage / Current | Dual (24V AC & 12V DC ±10%) |
| --- | --- |
| B. Power Consumption | Max. 9.5W |

2.08	Mechanical

| A. Color / Material | Dark gray / ALDC |
| --- | --- |
| B. Dimension (WxHxD) | Ø78.0 x 272.8mm (3.07" x 10.74") (Without sun shield) |
| C. Weight | 1.5Kg (3.31 lb) |

2.09	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.10	WARRANTY

A. 3 years, parts and labor.