# Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

# For additional information, visit http://security.hanwhatechwin.com/

1000TVL (1280H) VANDAL-RESISTANT DOME CAMERA

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

# MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00 Data Communications

# 28 23 13 Video Surveillance Control and Management Systems

# 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19 Digital Video Recorders and Analog Recording Devices

# 28 23 23 Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

28 2x 00	Video Management System

1000TVL (1280H) VANDAL-RESISTANT DOME CAMERA

- GENERAL

  - SUMMARY

    - Section includes 1000TVL (1280H) analog camera.

    - Product – An analog camera in a vandal resistant IP66/IK10 rated dome housing.

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Back light compression

      - CBR – Constant Bit Rate

      - CVBS – Color, Video, Blanking, Sync

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - DSCP – Differentiated Services Code Point

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GOV – Group of Video

      - GUI – Graphical User Interface

      - HD – High Definition

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - MP - Megapixel

      - MPEG - Moving Pictures Experts Group

      - NAS – Network Attached Storage

      - NTP - Network Time Protocol

      - PIM-SM - Protocol Independent Multicast-Sparse Mode

      - PoE - Power over Ethernet

      - PPPoE – Point to Point Protocol over Ethernet

      - RTP - Real-Time Transport Protocol

      - RTCP – Real-Time Control Protocol

      - RTSP - Real-Time Streaming Protocol

      - SDK – Software Development Kit

      - SMTP - Simple Mail Transfer Protocol

      - SNMP – Simple Network Management Protocol

      - SSL – Secure Sockets Layer

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - UPnP – Universal Plug and Play

      - VBR – Variable Bit Rate

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

      - DWDR – Digital Wide Dynamic Range

    - Reference Standards

      - Emissions

        - FCC-47 CFR Part 15 Subpart B Class B

        - CE EN 55022:2010

      - Immunity - CE

        - EN 50130-4:2011

        - EN 61000-3-3:2014

        - EN 61000-4-2:2009

        - EN 61000-4-3:2006+A2:2010

        - EN 61000-4-4:2012

        - EN 61000-4-5:2014

        - EN 61000-4-6:2009

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide video analytics embedded in the camera free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://security.hanwhatechwin.com/

    - Model		SCV-5082

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - Video – 1000TVL (1280h) resolution

    - Camera – The 1000TVL (1280h)  analog camera device shall have the following physical and performance properties:

      - Auto day/night operation with removable IR cut filter

        - Low light level operation to 0.0006 lux (color) and 0.00006 lux (black and white)

      - Digital noise reduction

      - Digital zoom

      - Defog

      - 24 privacy masking regions utilizing a 4 point polygon

      - Digital image stabilization

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection

      - Tampering

  - DETAILED SPECIFICATIONS

    - Video

      - Imager

        - Sensor			1/3" 1.3 MP CMOS

          - pixels per sensor	1,312(H) x 1,069(V) total; 1,305(H) x 1,049(V) effective

          - scanning		progressive

          - synchronization		Internal

          - frequency

            - NTSC		H: 15.734KHz / V: 59.94Hz

            - PAL		H: 15.625KHz / V: 50Hz

          - Horizontal Resolution

            - Color 	 	1000TV lines

            - B/W  		1000TV lines

        - Minimum illumination

          - Color mode		0.05Lux (F1.4, 50IRE), 0.001Lux (1sec, F1.4, 50IRE),

0.02Lux (F1.4, 30IRE), 0.0006Lux (1sec, F1.4, 30IRE)

          - Black & white mode	0 0.005Lux (F1.4, 50IRE), 0.0001Lux (1sec, F1.4, 50IRE),

0.002Lux (F1.4, 30IRE), 0.00006Lux (1sec, F1.4, 30IRE)

        - Signal: Noise ratio		52 dB (AGC off, Weight on)

        - Video Output		CVBS: 1.0 Vpp / 75Ω composite

    - Lens					3 – 10 mm, varifocal

          - Optical zoom ratio			3.3x

          - Maximum aperture ratio   		F1.4

          - Angle of view				H: 82.0°(Wide) ~ 26.5°(Tele)

V: 59.7˚(Wide) ~ 19.9˚ (Tele)

          - Minimum Object Distance 		0.5 m (1.64ft.)

          - Focus control				Manual

          - Lens Type		DC auto iris

          - Mount Type		Board

    - Operational

          - On Screen Display			English, Chinese, Korean, Japanese, German,

Italian, French, Spanish, Russian,

Czech, Polish, Romanian, Serbian, Swedish, Danish, Turkish, Portuguese

          - Camera Title				On / Off (up to 15 characters)

          - Day / Night				Auto (ICR) / External / Color / BW

          - Backlight compensation (BLC)		Off / User BLC / HLC

          - Contrast Enhancement			SSDR (Off / On)

          - Digital Noise Reduction (DNR)		Off / On (Samsung Super Noise Reduction IV)

          - Defog					Off / Manual / Auto

          - Purple Fringe Reduction			Off / Low / Middle / High

          - Digital Image Stabilization		Off / On

          - Tampering				Off / On

          - Motion detection			Off / On

          - Privacy masking

a.)	24 zones with 4 sided polygon

b.)	selectable for On/Off

          - Gain control				Off / Low / Middle / High

          - White Balance				ATW / Outdoor / Indoor / Manual / AWC /

Mercury (1,800K° ~ 10,500K°)

          - Electronic shutter speed 		1 - 1/12,000 sec.

          - Digital Zoom				Off / On (1x ~ 16x)

          - Reverse				Off / H-Rev / V-Rev / HV-Rev

          - Profile					Basic, Day & Night, Backlight, ITS, Indoor, User

          - Communication				Coaxial control (SPC-300 compatible)

          - Protocol				Pelco-C (Coaxitron)

    - Electrical

        - Input Voltage and Current			Dual (24V AC & 12V DC ±10%)

        - Power Consumption			Maximum 5.6W

    - Mechanical And Environmental

      - Material				aluminum

      - Dimensions (W x H):  		137.0 x 106.1 mm (5.39 x 4.18 in.)

      - Weight:				730g (1.61 lb.)

      - Operating Temperature: 	-30° C to 55° C (-22° F to 131° F)

      - Humidity:			0 - 90%, non-condensing

      - Environmental Rating:

        - Ingress Protection				IP66

        - Mechanical (Vandal) Protection		IK10

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The design and configuration shall be verified for compatibility and performance with the camera(s).

    - Configuration shall be tested and qualified by the Contractor prior to camera installation.

    - All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA) or Digital Video Recorder (DVR).

  - INSTALLATION

    - The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

    - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

    - The camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

8