SCV-5083 1280H WDR VANDAL-RESISTANT DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer.  The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

D.	All systems and components shall be provided with a one-day turnaround repair express and 24-hour parts replacement. The repair and parts express shall be guaranteed by the manufacturer on warranty and nonwarranty items.

2.02	SCV-5083 1280H WDR VANDAL-RESISTANT  DOME CAMERA

A.	The camera shall be compatible with Board type with DC Auto Iris

B.	The camera shall support virtual progressive scan CMOS. The camera shall support privacy masking and shall provide a minimum of 24 programmable zones. Support for motion detection shall also be provided.

C.   The camera shall support standard communication protocols and it shall provide a

customizable on-screen display (OSD) which shall be available in English, Chinese, Korean,

Japanese, German, Italian, French, Spanish, Russian, Czech, Polish, Romanian, Serbian,

Swedish, Danish, Turkish, Portuguese

2.03	VIDEO

| A. Imaging Device |  | 1/3" 1.3M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,312(H) x 1,069(V) |
| C. Effective Pixels |  | 1,305(H) x 1,049(V) |
| D. Scanning System |  | Progressive scan |
| E. Synchronization |  | Internal |
| F. Frequency |  |  |
|  | 1. SCV-5083N | H : 15.734KHz / V : 59.94Hz |
|  | 2. SCV-5083P | H : 15.625KHz / V : 50Hz |
| G. Horizontal Resolution |  |  |
|  | 1. Color | 1000TV lines |
|  | 2. B/W | 1000TV lines |
| H. Min. Illumination |  |  |
|  | 1. Color | 0.03Lux (F1.2, 50IRE) 0.001Lux (1sec, F1.2, 50IRE), 0.01Lux (F1.2, 30IRE) 0.0003Lux (1sec, F1.2, 30IRE) |
|  | 2. B/W | 0.003Lux (F1.2, 50IRE) 0.0001Lux (1sec, F1.2, 50IRE) 0.001Lux (F1.2, 30IRE) 0.00003Lux (1sec, F1.2, 30IRE) |
| I. S/N Ratio |  | 52dB (AGC off, Weight on) |
| J. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite |

2.04	LENS

| A. Focal Length (Zoom Ratio) | 2.8 ~ 10.5mm (3.8x) varifocal |
| --- | --- |
| B. Max. Aperture Ratio | F1.2 |
| C. Angular Field of View | H : 91.5°(Wide) ~ 27.7°(Tele) V : 68.3°(Wide) ~ 20.8°(Tele) |
| D. Min. Object Distance | 0.5m (1.64ft) |
| E, Focus Control | Manual |
| F. Lens Type | DC auto iris |
| G. Mount Type | Board type |

2.05	OPERATIONAL

| A. On Screen Display | English, Chinese, Korean, Japanese, German, Italian, French, Spanish, Russian, Czech, Polish, Romanian, Serbian, Swedish, Danish, Turkish, Portuguese |
| --- | --- |
| B. Camera Title | Off / On (Displayed 15 characters) |
| C. Day & Night | Auto (ICR) / External / Color / B/W |
| D. Backlight Compensation | Off / User BLC / HLC / WDR |
| E. Wide Dynamic Range | 120dB |
| F. Contrast Enhancement | SSDR (Off / On) |
| G. Digital Noise Reduction | SSNR 4 (Off / On) |
| H. Defog | Off / Auto / Manual |
| I. Purple Fringe Reduction | Off / Low / Middle / High |
| J. Digital Image Stabilization | Off / On |
| K. Tampering | Off / On |
| L. Motion Detection | Off / On |
| M. Intelligent Video Analytics | Fence, (Dis)Appear, Counting (Off / On) |
| N. Privacy Masking | Off / On (24 programmable zones with 4 points polygonal masking) |
| O. Gain Control | Off / Low / Middle / High |
| P. White Balance | ATW / Outdoor / Indoor / Manual / AWC / Mercury (1,800K° ~ 10,500K°) |
| Q. Electronic Shutter Speed | 1sec ~ 1/12,000sec |
| R. Digital Zoom | Off / On (1x ~ 16x) |
| S. Reverse | Off / H-Rev / V-Rev / HV-Rev |
| T. Profile | Basic, Day & Night, Backlight, ITS, Indoor, User |
| U. Alarm | 1 Out |
| V. Communication | Coaxial control (SPC-300 compatible) |
| W. Protocol | Coax : Pelco-C (Coaxitron) |

2.06	ELECTRICAL

| A. Voltage | Dual (24V AC & 12V DC ±10%) |
| --- | --- |
| B. Power Consumption | Max. 5.6W |

2.07	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature | -30°C ~ +55°C (-22°F ~ +131°F) *Start up should be done at above -10°C (+14°F) |
| --- | --- |
| B. Operating Humidity | Less than 90% RH |

2.08	PHYSCIAL SPECIFICATIONS

| A. Dimension (WxH) | Ø137.0 x 106.1mm (Ø5.39" x 4.18") |
| --- | --- |
| B. Weight | 720g (1.59 lb) |
| C. Color/Material | Ivory / ALDC |

2.09	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.10	WARRANTY

A. 3 years, parts and labor.