SNB 6004 2MEGAPIXEL FULL HD NETWORK CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNB 6004 2MEGAPIXEL FULL HD NETWORK CAMERA

- The network camera shall offer 2M resolution (1,920 x 1,080) maximum.

- The cameras shall support multi-streaming technologies, transferring real-time high resolution images.  It shall support 60fps at all available resolution when using H.264.

- The camera shall provide video compression dual codec, H.264 and MJPEG.

- The camera shall support P-iris and Simple Focus.

- The camera shall provide Defog and WDR.

- The camera shall provide Audio, Face and Motion detection.

- The camera shall provide memory slot for SD, SDHC and SDXC.

- The camera shall provide Multi-crop streaming.

- The camera shall support Bi-directional audio.

- All operation and configuration menus shall be provided in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, and Finnish.

2.03	VIDEO

| A. Imaging Device |  | 1/2.8" 2.38M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.1Lux (F1.2, 50IRE) 0.001Lux (2sec, 50IRE) 0.03Lux (F1.2, 30IRE) |
|  | 2. B/W | 0.01Lux (F1.2, 50IRE) 0.002Lux (F1.2, 30IRE) |
| F. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |
| G. S/N Ratio |  | 50dB |

2.04	LENS

| A. Focus Control | Simple focus / Manual, Remote control via network, Button control (Manual, Simple focus, Day & Night) |
| --- | --- |
| B. Lens Type | Manual / DC auto iris, P-Iris |
| C. Mount Type | C / CS |

2.05	OPERATION

| A. Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| B. Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| C. Backlight Compensation | Off / BLC |
| D. Wide Dynamic Range | 120dB |
| E. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| F. Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| G. Digital Image Stabilization | Off / On |
| H. Defog | Off / Auto / Manual |
| I. Motion Detection | Off / On (4ea 4 points polygonal zones) |
| J. Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| K. Gain Control | Off / Low / Middle / High |
| L. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| M. Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| N. Flip / Mirror | Off / On |
| O. Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection |
| P. Alarm I/O | Input 1ea / Output 1ea |
| Q. Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Alarm input, Network disconnection |
| R. Alarm Events | File upload via FTP and E-mail Notification via E-mail Local storage (SD/SDHC/SDXC) or NAS recording at Event (Alarm triggers) External output |
| S. Remote Control Interface | RS-485 |
| T. RS-485 Protocol | Samsung-T/E, Pelco-P/D, Sungjin, Panasonic, Honeywell, AD, Vicon, GE, Bosch |

2.06	NETWORK

| A. Ethernet | RJ-45 (10/100BASE-T) |
| --- | --- |
| B. Video Compression Format | H.264 (MPEG-4 part 10/AVC), MJPEG |

| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| --- | --- | --- |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 60fps @ All resolutions |
|  | 2. MJPEG | Max. 30fps @ 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 Max. 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 |

| E. Video Quality Adjustment |  |  |
| --- | --- | --- |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| F. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| G. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| H. Audio In |  | Selectable (Mic in / Line in) Built-in mic. Max output level : 1 Vrms Supply voltage : 2.5V DC (4mA) Input impedance : approx. 2K Ohm |
| I. Audio Out |  | Line out (3.5mm stereo mini jack) |
| J. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| K. Audio Communication |  | Bi-directional audio |
| L. IP |  | IPv4, IPv6 |
| M. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| N. Security |  | HTTPS(SSL) login authentication Digest login authentication IP address filtering User access log 802.1x authentication |
| O. Streaming Method |  | Unicast / Multicast |
| P. Max. User Access |  | 15 users at unicast mode |
| Q. Edge Storage |  | SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded NAS (Network Attached Storage) |
| R. Application Programming interface |  | ONVIF profile S & G HTTP API (SUNAPI) 2.0 SVNP 1.2 |
| S. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| T. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 11 ~ 8) Mozilla Firefox (Ver. 19 ~9) * Windows only Google Chrome (Ver. 25 ~ 15) * Windows only Apple Safari (Ver. 5.1.7, 6.0.2(Mac OS X 10.8, 10.7 only)) * Mac OS X only |
| U Central Management Software |  | SmartViewer, SSM |

2.07	ELECTRICAL

| A. Input Voltage | 24V AC ±10%, 12V DC ±10%, PoE (IEEE802.3af, Class3) |
| --- | --- |
| B. Power Consumption | Max. 11.44W (24V AC, 50 ~ 60Hz) Max. 10.1W (12V DC) Max. 11.1W (PoE, Class3) |

2.08	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C (+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.09	PHYSCIAL SPECIFICATIONS

| A. Dimension (WxHxD) | 73.1 x 66.6 x 147.8mm (2.88" x 2.62" x 5.82") |
| --- | --- |
| B. Weight | 395g (0.87 lb) |
| C. Color/Material | Black, Ivory / Aluminum, Plastic |

2.10	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.11	WARRANTY

A. 3 years, parts and labor.