SNB 6004F 2Megapixel Full HD Fiber Optic Network Camera

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNB 6004F 2Megapixel Full HD Fiber Optic Network Camera

- The network camera shall offer maximum of 2M (1920 x 1080) resolution.

- The cameras shall support minimum illumination of 0.015Lux at F1.2 when in color mode and 0.0015Lux at F1.2 when in black and white mode.

- The camera shall support dual codec of H.264 and MJPEG.

- The camera shall support multiple streaming.

- The camera shall feature WDR (Wide Dynamic Range) with a gain of 120dB.

- The camera shall feature Simple Focus, the automatic motorized focus adjustment. This should be activated by a button or remotely through network. This should also be activated when the camera’s mode is changed from day to night.

- The camera shall feature P-Iris which improves image quality by finding the optimal iris position.

- The camera shall feature Defog mode to enhance color reproduction and contrast of the image in foggy weather.

- The camera shall feature video analysis including audio, face and motion detection.

- The camera shall have slots for micro SD, SDHC, and SDXC cards.

- The camera shall support Multi-crop streaming for an effective video surveillance for multiple users.

- The camera shall support SFP (Small Form-factor Pluggable) transceiver with default speed of 1000Mbps.

2.03	VIDEO

| A. Imaging Device |  | 1/2.8" 2.38M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.1Lux (F1.2, 50IRE), 0.03Lux (F1.2, 30IRE) |
|  | 2. B/W | 0.01Lux (F1.2, 50IRE), 0.002Lux (F1.2, 30IRE) |
| F. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |
| G. S/N Ratio |  | 50dB |

2.04	LENS

| A. Focus Control | Simple focus / Manual, Remote control via network (Manual, Simple focus, Day & Night) |
| --- | --- |
| B. Lens Type | Manual / DC auto iris, P-Iris |
| C. Mount Type | C / CS |

2.05	OPERATION

| A. Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| B. Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| C. Backlight Compensation | Off / BLC / WDR |
| D. Wide Dynamic Range | 120dB |
| E. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| F. Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| G. Digital Image Stabilization | Off / On |
| H. Defog | Off / Auto / Manual |
| I. Motion Detection | Off / On (4 zones with 4-sided polygon) |
| J. Privacy Masking | Off / On (32 zones with 4-sided polygon) |
| K. Gain Control | Off / Low / Middle / High |
| L. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| M. Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| N. Flip / Mirror | Off / On |
| O. Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection with Metadata |
| P. Alarm I/O | Input 1ea / Output 1ea |
| Q. Remote Control Interface | RS-485 |
| R. RS-485 Protocol | Samsung-T/E, Pelco-P/D, Sungjin, Panasonic, Honeywell, AD, Vicon, GE, Bosch |
| S. Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Alarm input, Network disconnection |
| T. Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at Event, External output |

2.06	NETWORK

| A. Ethernet: Fiber |  | SFP (LC type) |
| --- | --- | --- |
| B. Fiber : Ethernet Physical Layer |  | 100 Base-FX, 1000 Base-X |
| C. Fiber : Default Speed |  | 1000Mbps |
| D. Fiber : Ethernet Standard |  | IEEE 802.3u, IEEE 802.3z support |
| E. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| F. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| G. Max. Framerate |  |  |
|  | 1. H.264 | Max. 60fps at all resolutions |
|  | 2. MJPEG | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 : Max. 15fps 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 : Max. 30fps |
| H. Smart Codec |  | Manual mode (Area-based : 5ea), Face detection mode |
| I. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| J. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| K. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| L. Audio In |  | Selectable (Mic in / Line in), Built-in mic. Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| M. Audio Out |  | Line out (3.5mm stereo mini jack), Max output level : 1 Vrms |
| N. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| O. Audio Communication |  | Bi-directional audio |
| P. IP |  | IPv4, IPv6 |
| Q. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| R. Security |  | HTTPS(SSL) login authentication, Digest login authentication IP address filtering, User access log, 802.1x authentication |
| S. Streaming Method |  | Unicast / Multicast |
| T. Max. User Access |  | 15 users at unicast mode |
| U. Edge Storage |  | Micro SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| V. Application Programming interface |  | ONVIF profile S, HTTP API (SUNAPI) 2.0, SVNP 1.2 |
| W. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| X. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 ~ 10.10 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 8 ~ 11) Mozilla Firefox (Ver. 9 ~ 35), Google Chrome (Ver. 15 ~ 40) Apple Safari (Ver. 8.0.2 (Mac OS X 10.10), 7.0.6 (Mac OS X 10.9), 6.0.2 (Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| Y. Central Management Software |  | SmartViewer, SSM |

2.07	ELECTRICAL

| A. Input Voltage | 24V AC ±10%, 12V DC ±10% |
| --- | --- |
| B. Power Consumption | Max. 12.5W (24V AC, 50 ~ 60Hz), Max. 10.5W (12V DC) |

2.08	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C (+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.09	PHYSCIAL SPECIFICATIONS

| A. Dimension (WxHxD) | 73.1 x 66.6 x 147.8mm (2.88" x 2.62" x 5.82") |
| --- | --- |
| B. Weight | 385g (0.85 lb) |
| C. Color/Material | Black, Ivory / Aluminum, Plastic |

2.10	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.11	WARRANTY

A. 3 years, parts and labor.