SNB 6005 2MEGAPIXEL FULL HD NETWORK CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNB 6005 2MEGAPIXEL FULL HD NETWORK CAMERA

- The network camera shall offer Max. 2M (1920 x 1080) resolution.

- The cameras shall support Min. illumination of 0.015Lux at F1.2 when in color mode and 0.0015Lux at F1.2 when in black and white mode.

- The camera shall provide 1/1.9" 2.42M CMOS.

- The camera shall support H.264, MJPEG dual codec and multiple streaming.

- The camera shall feature WDR (120dB), Simple focus, P-Iris, and Defog.

- The camera shall feature Audio / Face / Motion detection.

- The camera shall support SD/SDHC/SDXC memory slot.

- The camera shall support multi-crop streaming.

- The camera shall support PoE and UPnP.

- The camera shall support Bi-directional audio.

2.03	VIDEO

| A. Imaging Device |  | 1/1.9" 2.42M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,241(V) approx 2.42M pixels |
| C. Effective Pixels |  | 1,937(H) x 1,097(V) approx 2.12M pixels |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.03Lux (1/30sec, F1.2, 50IRE), 0.015Lux (1/30sec, F1.2, 30IRE) |
|  | 2. B/W | 0.003Lux (1/30sec, F1.2, 50IRE), 0.0015Lux (1/30sec, F1.2, 30IRE) |
| F. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |
| G. S/N Ratio |  | 50dB |

2.04	LENS

| A. Focus Control | Simple focus (Motorized V/F) / Manual, Remote control via network, Button control (Manual, Simple focus) |
| --- | --- |
| B. Lens Type | Manual / DC auto iris, P-Iris |
| C. Mount Type | C / CS |

2.05	OPERATION

| A. Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| B. Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| C. Backlight Compensation | Off / BLC / WDR |
| D. Wide Dynamic Range | 120dB |
| E. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| F. Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| G. Digital Image Stabilization | Off / On |
| H. Defog | Off / Auto / Manual |
| I. Motion Detection | Off / On (4 zones with 4 points polygonal) |
| J. Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| K. Gain Control | Off / Low / Medium / High / Manual |
| L. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| M. Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| N. Flip / Mirror | Off / On |
| O. Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection - Support Metadata |
| P. Alarm I/O | Input 1ea / Output 1ea |
| Q. Remote Control Interface | RS-485 |
| R. RS-485 Protocol | Samsung-T/E, Pelco-P/D, Sungjin, Panasonic, Honeywell, AD, Vicon, GE, Bosch |
| S. Alarm Triggers | Motion detection, Tampering detection, Audio detection, Face detection, Network disconnection, Video analytics, Alarm input |
| T. Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at event triggers, External output |

2.06	NETWORK

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC) : Main / Baseline / High , MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 720 x 576, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 60fps@ all resolutions |
|  | 2. MJPEG | Max. 15fps@1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 Max. 30fps@800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Smart Codec |  | Manual mode (Area-based : 5ea), Face detection mode |
| F. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| G. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| H. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| I. Audio In |  | Selectable (Mic in / Line in), Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| J. Audio Out |  | Line out (3.5mm stereo mini jack), Max output level : 1 Vrms |
| K. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| L. Audio Communication |  | Bi-directional audio (2-way) |
| M. IP |  | IPv4, IPv6 |
| N. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| O. Security |  | HTTPS(SSL) login authentication Digest login authentication IP address filtering User access log 802.1x authentication |
| P. Streaming Method |  | Unicast / Multicast |
| Q. Max. User Access |  | 15 users at unicast mode |
| R. Edge Storage |  | SD/SDHC/SDXC - Motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| S. Application Programming interface |  | ONVIF profile S, SUNAPI (HTTP API), SVNP, Samsung camera application platform |
| T. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek |
| U. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1 or higher, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 8 ~ 11) Mozilla Firefox (Ver. 9 ~ 19), Google Chrome (Ver. 15 ~ 25) Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| V. Central Management Software |  | SmartViewer, SSM |

2.07	ELECTRICAL

| A. Input Voltage | 24V AC, 12V DC, PoE (IEEE802.3af, Class3) |
| --- | --- |
| B. Power Consumption | Max. 11.44W (24V AC, 50/60Hz), Max. 10.1W (12V DC), Max. 11.1W (PoE, Class3) |

2.08	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -30°C ~ +55°C (-22°F ~ +131°F) / Less than 90% RH * Start up should be done at above -10°C (+14°F) |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.09	PHYSCIAL SPECIFICATIONS

| A. Dimension (WxHxD) | 73.1 x 66.6 x 147.8mm (2.88" x 2.62" x 5.82") |
| --- | --- |
| B. Weight | 430g (0.95 lb) |
| C. Color/Material | Black, Ivory / Aluminum, Plastic |

2.10	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.11	WARRANTY

A. 3 years, parts and labor.