SNB 6011 2MEGAPIXEL FULL HD NETWORK REMOTE HEAD CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNB 6011 2MEGAPIXEL FULL HD NETWORK REMOTE HEAD CAMERA

- These cameras shall support multi-streaming technologies, transferring real-time high resolution images.  It shall support 30fps at all available resolution when using H.264.

- All operation and configuration menus shall be provided in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Denish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatia, Hungary, Greek, Finnish and Norwegian.

- The camera shall be of a dome type suitable for internal installation. The main unit shall be ivory and the camera unit shall be black in appearance.

- The camera should be capable of capturing an image size of 1920 x 1080 at 30 images per second.

- The camera shall feature Wide Dynamic Range with a gain of 120dB.

2.03	VIDEO

| A. Imaging Device |  | 1/2.8" PS 2.38M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1952(H) x 1116(V) |
| C. Effective Pixels |  | 1944(H) x 1104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | Color : 0.3Lux (1/30sec, F2.0, 50IRE) |
| F. Video Output |  | CVBS : 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P), for installation - DIP connector type |

2.04	LENS

| A. Focal Length (Zoom Ratio) | 2.4mm |
| --- | --- |
| B. Max. Aperture Ratio | 2.0 |
| C. Angular Field of View | H : 134.3˚ / V : 70.9˚ |
| D. Min. Object Distance | 0.3m |
| E. Mount Type | RJ-12 connected with main unit |

2.05	OPERATION

| A. Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| B. Day & Night | Auto(S/W) / Color / B/W / Schedule |
| C. Backlight Compensation | Off / BLC / WDR |
| D. Wide Dynamic Range | 120dB |
| D. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) |
| E. Digital Noise Reduction | SSNR3 (2D + 3D noise filter)(Off/On) |
| F. Digital Image Stabilization | Off / On |
| G. Defog | Off / Auto / Manual |
| H. Motion Detection | Off / On (4ea 4 Points Polygonal zones) |
| I. Privacy Masking | Off / On ( 32 Zones with 4 Points of Polygonal ) |
| J. Gain Control | Off / Low / Middle / High |
| K. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| L. Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| M. Flip / Mirror | Off / On |
| N. Intelligent Video Analytics | Tampering, Virtual Line, Enter/Exit, Appear / Disappear |
| O. Alarm I/O | Input 1ea / Output 1ea |
| P. Alarm Triggers | Motion detection, Alarm Input, Video Anaytics, Network Disconnection |
| Q. Alarm Events | "File upload via FTP and E-Mail |

2.06	NETWORK

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 Part 10/AVC), Motion JPEG |
| C. Resolution |  | 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768 |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 30fps @ all resolutions |
|  | 2. MJPEG | Max. 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 Max. 30fps @ 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression Level, Target Bitrate Level Control |
|  | 2. MJPEG | Quality Level Control |
| F. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| G. Streaming Capability |  | Multiple Streaming (Up to 10 Profiles) |
| H. IP |  | IPv4, IPv6 |
| I. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| J. Security |  | HTTPS(SSL) Login Authentication Digest Login Authentication IP Address Filtering User access Log 802.1x Authentication |
| K. Streaming Method |  | Unicast / Multicast |
| L. Max. User Access |  | 15 users at Unicast Mode |
| M. Edge Storage |  | SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (network attached storage |
| N. Application Programming interface |  | ONVIF Profile S HTTP API(SUNAPI) 2.0 SVNP 1.2 |
| O. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Denish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatia, Hungary, Greek, Finnish, Norwegian |
| P. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 11 ~ 8) Mozilla Firefox (Ver. 19 ~9) * Windows Only Google Chrome (Ver. 25 ~ 15) * Windows Only Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7only), 5.1.7) * Mac OS X only |
| Q Central Management Software |  | SmartViewer, SSM |

2.07	ELECTRICAL

| A. Voltage | DC12V, PoE(IEEE802.3af) |
| --- | --- |
| B. Power Consumption | PoE : 7.5W DC12V : 6.5W |

2.08	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C(+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.09	PHYSCIAL SPECIFICATIONS

| A. Dimension |  |  |
| --- | --- | --- |
|  | 1. Camera Unit | Φ24×50mm |
|  | 2. Main Unit | W121.2xH28xD76mm |
| B. Weight |  |  |
|  | 1. Camera Unit | 315g (Include Cable) |
|  | 2. Main Unit | 290g |
| C. Color/Material |  |  |
|  | 1. Camera Unit | Black/Plastic |
|  | 2. Main Unit | Ivory/Plastic |

2.10	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.11	WARRANTY

A. 3 years, parts and labor.