SNB 9000 12MEGAPIXEL NETWORK CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNB 9000 12MEGAPIXEL NETWORK CAMERA

- The network camera shall offer 12M resolution maximum.

- The cameras shall support multi-streaming technologies, transferring real-time high resolution images.  It shall support 20fps at all available resolution when using H.264.

- The camera shall provide video compression dual codec, H.264 and MJPEG.

- The camera shall support Simple focus.

- The camera shall provide Tampering, Virtual Line, Enter/Exit, Appear/Disappear, Face detection and Audio detection for intelligent video analytics.

- The camera shall provide memory slot for micro SD, SDHC and SDXC.

- The camera shall support Bi-directional audio.

- All operation and configuration menus shall be provided in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian and Finnish.

2.03	VIDEO

| A. Imaging Device |  | 1/2.3" 12M Progressive scan CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 12M(4,168x3,062) |
| C. Effective Pixels |  | 4,072(H)x3,046(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.28Lux(1/30sec,F1.2); 0.01Lux(1sec,F1.2) |
|  | 2. B/W | 0.03Lux(1/30sec,F1.2); 0.001Lux(1sec,F1.2) |
| F. Video Output |  | CVBS : 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P), for installation - BNC Type |
| G. S/N Ratio |  | Greater than 50dB |

2.04	LENS

| A. Focus Control | Simple Focus / Manual , Remote control via network ,Button control (Manual, Simple Focus, Day&Night) |
| --- | --- |
| B. Lens Type | Manual / DC auto iris |
| C. Mount Type | C / CS |

2.05	OPERATION

| A. Camera Title | Off / On (Displayed up to 20 characters) |
| --- | --- |
| B. Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| C. Backlight Compensation | Off / BLC / HLC |
| D. Contrast Enhancement(DWDR) | SSDR (Samsung Super Dynamic Range) (Off / On) |
| E. Digital Noise Reduction | SSNRIII (3D+2D Noise Filter) (Off / On) |
| F. Motion Detection | Off / On (4ea 4 Points Polygonal zones) |
| G. Privacy Masking | Off / On (32 Zones of Rectangular) |
| H. Gain Control | Off / Low / Middle / High |
| I. White Balance | ATW / AWC / Manual / Indoor / Outdoor / Mercury |
| J. Electronic Shutter Speed | Minimum / Maximum / Anti flicker (1 ~ 1/12,000sec) |
| K. Digital PTZ | 8x, Digital PTZ ( Preset, Group ) |
| L. Flip / Mirror | Off / On |
| M. Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection with metadata (to NVR / SSM recording server) |
| N. Alarm I/O | Input 2ea / Output 1ea |
| O. Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Alarm input, Network disconnection |
| P. Alarm Events | File upload via FTP and E-mail, Notification via E-mail, TCP and HTTP Local storage (SD/SDHC/SDXC) recording at Network disconnected & Event (Alarm triggers) or NAS recording at event (Alarm triggers), External output |
| Q. Remote Control Interface | RS-485 (11ea) |
| R. RS-485 Protocol | Samsung-T/E, Pelco-P/D, Sungjin, Panasonic, Honeywell, AD, Vicon, GE, Bosch |

2.06	NETWORK

| A. Ethernet | RJ-45 (10/100/1000BASE-T) |
| --- | --- |
| B. Video Compression Format | H.264 (MPEG-4 Part 10/AVC), MJPEG |

| C. Resolution |  | 4000 x 3000, 4096 x 2160, 3840 x 2160, 2592 x 1944, 2048 x 1536, 1920 x 1080, 1600 x 1200, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 720 x 576, 720 x 480, 704 x 576, 640 x 480, 640 x 360, 320 x 240 |
| --- | --- | --- |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 20fps @ 4000x3000 Max. 25fps @ 4096x2160 Max. 30fps @ 3840x2160, 2592x1944, 2048x1536, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 720x576, 720x480, 704x576, 640x480, 640x360, 320x240 |
|  | 2. MJPEG | Max. 5fps @ 4000x3000, 4096x2160, 3840x2160 Max. 10fps @ 2592x1944, 2048x1536 Max. 15fps @ 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768 Max. 20fps @ 800x600, 720x576, 720x480, 704x576, 640x480, 640x360, 320x240 |
| E. Smart Codec |  | Yes (Area Based Method) |

| F. Video Quality Adjustment |  |  |
| --- | --- | --- |
|  | 1. H.264 | Target Bitrate Level Control |
|  | 2. MJPEG | Quality Level Control |
| G. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR, |
|  | 2. MJPEG | VBR |
| H. Streaming Capability |  | Multiple Streaming (Up to 3 Profiles per each resolution mode) |
| I. Audio In |  | Selectable (Mic in/Line in), Max output level: 1 Vrms |
| J. Audio Out |  | Line out (3.5mm mono mini jack) |
| K. Audio Compression Format |  | G.711 u-law /G.726 Selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| L. Audio Communication |  | Bi-directional |
| M. IP |  | IPv4, IPv6 |
| N. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| O. Security |  | HTTPS(SSL) Login Authentication Digest Login Authentication IP Address Filtering User access Log 802.1x Authentication |
| P. Streaming Method |  | Unicast / Multicast |
| Q. Max. User Access |  | Max. 15 users access(Max. 5 users each profile) |
| R. Edge Storage |  | micro SD/SDHC/SDXC (128GB) - Motion Images recorded in the SDX/SDHC/SD memory card can be downloaded. |
| S. Application Programming interface |  | ONVIF Profile S SUNAPI(HTTP API) v2.0 SVNP 2.0 |
| T. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| U. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 8~11) Mozilla Firefox (Ver. 9~19) Google Chrome (Ver. 15~25) Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 Only), 5.1.7) * Mac OS X Only |
| V Central Management Software |  | SmartViewer, SSM |

2.07	ELECTRICAL

| A. Input Voltage | 24V AC ±10%, 12V DC ±10%, PoE (IEEE802.3af, Class3) |
| --- | --- |
| B. Power Consumption | Max. 12.5W (AC 24V, 50~60Hz) Max. 12.0W (DC 12V) Max. 12.0W (PoE, Class3) |

2.08	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C(+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 95% RH |

2.09	PHYSCIAL SPECIFICATIONS

| A. Dimension (WxHxD) | 78.3 x 67.3 x 130.4mm (3.08" x 2.65" x 5.13") |
| --- | --- |
| B. Weight | 493g (1.09 lb) |
| C. Color/Material | Black, Ivory / Aluminum |

2.10	WARRANTY

A. 3 years, parts and labor.