SND 6084 2MEGAPIXEL FULL HD NETWORK DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

- All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

- All systems and components shall have been thoroughly tested and proven in actual use.

- All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SND 6084 2MEGAPIXEL FULL HD NETWORK DOME CAMERA

- The camera shall be of a dome type suitable for interior installation. The camera shall be ivory in appearance with a plastic body.

- The camera shall provide video compression dual codec ( H.264 and MJPEG).

- The network camera shall feature up to 2 Megapixel resolution in a 16:9 format.

- The camera shall provide Defog and WDR.

- The camera shall provide Audio, Face and Motion detection.

- The camera shall provide memory slot for SD, SDHC and SDXC.

- The camera shall provide Multi-crop streaming.

- The camera shall support Bi-directional audio.

- All operation and configuration menus shall be provided in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, and Finnish.

2.03	CAMERA

| A. Imaging Device |  | 1/2.8" 2.38M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.1Lux (F1.2, 50IRE) 0.001Lux (2sec, 50IRE) 0.03Lux (F1.2, 30IRE) |
|  | 2. B/W | 0.01Lux (F1.2, 50IRE) 0.002Lux (F1.2, 30IRE) |
| F. S/N Ratio |  | 50dB |
| G. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |

2.04	LENS TYPE

| A. Focal Length (Zoom Ratio) | 3 ~ 8.5mm (2.8x) motorized varifocal |
| --- | --- |
| B. Max. Aperture Ratio | F1.2 |
| C. Angular Field of View | H : 105.5°(Wide) ~ 37.1°(Tele) V : 57.5°(Wide) ~ 21.0°(Tele) |
| D. Min. Object Distance | 0.5m (1.64ft) |
| E. Lens Type | DC auto iris, P-Iris |
| F. Mount Type | Board-in type |
| G. Focus Control | Simple focus (Motorized V/F) / Manual Remote control via network Button control (Manual, Simple focus) |

2.05	PAN/TILT/ROTATE

| A. Pan Range | 0° ~ 354° |
| --- | --- |
| B. Tilt Range | 0° ~ 67° |
| C. Rotate Range | 0° ~ 355° |

2.06	OPERATION

| - Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| - Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| - Backlight Compensation | Off / BLC |
| - Wide Dynamic Range | 120dB |
| - Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| - Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| - Digital Image Stabilization | Off / On |
| - Defog | Off / Auto / Manual |
| - Motion Detection | Off / On (4ea 4 points polygonal zones) |
| - Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| - Gain Control | Off / Low / Middle / High |
| - White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| - Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| - Flip / Mirror | Off / On |
| - Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection |
| - Alarm I/O | Input 1ea / Output 1ea |
| - Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Alarm input, Network disconnection |
| - Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at Event (Alarm triggers), External output |

2.07	NETWORK PROTOCOL

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 60fps @ all resolutions |
|  | 2. MJPEG | Max. 30fps @ 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 Max. 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 |
| E. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| F. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| G. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| H. Audio In |  | Selectable (Mic in / Line in), Built-in mic. Max output level : 1 Vrms Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| I. Audio Out |  | Line out (3.5mm stereo mini jack) |
| J. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| K. Audio Communication |  | Bi-directional audio |
| L. IP |  | IPv4, IPv6 |
| M. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| N. Security |  | HTTPS(SSL) login authentication Digest login authentication IP address filtering User access log 802.1x authentication |
| O. Streaming Method |  | Unicast / Multicast |
| P. Max. User Access |  | 15 users at unicast mode |
| Q. Edge Storage |  | SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| R. Application Programming interface |  | ONVIF profile S & G, HTTP API (SUNAPI) 2.0, SVNP 1.2 |
| S. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| T. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 11 ~ 8) Mozilla Firefox (Ver. 19 ~ 9) * Windows only Google Chrome (Ver. 25 ~ 15) * Windows only Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| U. Central Management Software |  | SmartViewer, SSM |

2.08	ELECTRICAL

| A. Voltage | 12V DC ±10%, PoE (IEEE802.3af, Class3) |
| --- | --- |
| B. Consumption | Max. 8.79W (12V DC), Max. 9.57W (PoE, Class3) |

2.09	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C (+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.10	PHYSCIAL SPECIFICATIONS

| A. Dimension | Ø132.1 x 107.6mm (Ø5.2" x 4.24") |
| --- | --- |
| B. Weight | 505g (1.11 lb) |
| C. Color/Material | Ivory / Plastic |

2.11	CERTIFICATIONS

- CE mark

- FCC mark

- UL mark

2.12	WARRANTY

A. 3 years, parts and labor.