SND 7082F 3 MEGAPIXEL FULL HD NETWORK DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SND 7082F 3 MEGAPIXEL FULL HD NETWORK DOME CAMERA

A. 	The network camera shall offer 3M resolution (2048 x 1536) with 16:9 full HD (1080p) resolution support.

B.	The camera shall provide video compression dual codec ( H.264 and MJPEG).

C.   The camera shall provide WDR and Smart codec (ROI encoding).

D.	The camera shall provide a customizable on-screen display (OSD) which shall be available in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, and Greek.

2.03	CAMERA

A. Imaging Device	1. 3” 3.1M PS CMOS

B. Total Pixels 	2,064(H) x 1,578(V)

C. Effective Pixels 	2,052(H) x 1,536(V)

D. Scanning System	Progressive

F. Min. Illumination

1. Color				0.7Lux (F1.2, 50IRE), 0.011Lux (Sens-up 60x)

2. B/W 	0.07Lux (F1.2, 50IRE), 0.001Lux (Sens-up 60x)

G. S / N Ratio	50dB

H. Video Output	CVBS:	1.0Vp-p / 75ohm composite, 704x480(N), 704x576(P), for installation

2.04	LENS TYPE

A. Lens Type	Manual, DC auto iris

B. Mount Type	C/CS

2.05	OPERATION

A. Camera Title 	Off, On (Displayed up to 15 characters)

B. Day & Night 	Auto (ICR), Color, B/W, External B/W

C. Backlight Compensation		Off, BLC, HLC, WDR

D. Wide Dynamic Range 	Off, / On (96dB) 3M mode : 20fps 2M mode : 30fps

E. Contrast Enhancement SSDR 	On, Off

F. Digital Noise Reduction SSNRIII 	On, Off

G. Motion Detection 	Off, On (4 programmable zones)

H. Privacy Masking 			Off, On (1 each 4 point-polygonal and 16 each

rectangular zone)

I. Sens-up (Frame Integration) 	Off, Auto (2x ~ 60x)

J. Gain Control 	Off, Low, Middle, High

K. White Balance 	ATW, AWC, Manual, Indoor, Outdoor

L. Electronic Shutter Speed 	Auto, A.FLK, Manual (1/30 ~ 12,000sec)

M. Flip / Mirror 	Off, On

N. Intelligent Video Analytics	Audio detection, Face detection, Tampering, Virtual line, Enter / Exit, Appear / Disappear

O. Alarm I/O 	Digital 1/1

P. Alarm Triggers 	Motion detection, Video analytics, Sensor input

Q. Alarm Events	File upload via FTP, E-mail / Notification via E-mail,

TCP / Local storage (SD/SDHC) recording at network disconnected, External output

2.06	NETWORK PROTOCOL

A. Ethernet 	RJ-45 (10/100Base-T)

B. Video Compression Format 	H.264(MPEG-4 part 10/AVC), MJPEG

C. Resolution

1. 3M Mode, H.264 		Max 20 fps at all resolutions

2. 3M Mode, MJPEG 		800 x 600 ~ 320 x 180, 20fps

1280 x 1024 ~1600 x 1200, 15fps

1920 x 1080 ~ 2048 x 1536, 10fps

3. 2M Mode, H.264 		Max. 30fps at all resolutions

4. 2M Mode, MJPEG 		800 x 600 ~ 320 x 180, 30fps

1280 x 1024 ~ 1920 x 1080, 15fps

D. Max. Framerate

1. 3M Mode	20fps

2. 2M Mode	30fps

E. Video Quality Adjustment

1. H.264 	Compression level, Target bit rate level control

2. MJPEG 	Quality level control

F. Bitrate Control Method

1, H.264	CBR or VBR

2. MJPEG 	VBR

G. Streaming Capability 	Multiple streaming (Up to 6 profiles)

H. Audio I/O	Mic/ Line in, Line out

I.  Audio Compression Format 	G.711 u-law, G.726 (16kbps, 24kbps, 32kbps, 40kbps)

J. Audio Communication 	Bi-directional audio

K. IP 		IPv4, IPv6

L. Protocol 				TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTSP, NTP,

HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP,

IGMP, SNMPv1, v2c, v3(MIB-2), ARP, DNS, DDNS, QOS, UPNP, ONVIF

M. Security 	HTTPS(SSL) login authentication, 
Digest login authentication, IP address filtering, 
User access log, 802.1x authentication

N. Streaming Method 	Unicast, Multicast

O. Max. User Access 	10 users at Unicast mode

P. Memory Slot 	SD/SDHC memory slot

Q. ONVIF Conformance 	Yes

R. Webpage Language 	English, French, German, Spanish, Italian, Chinese, 
		Korean, Russian, Japanese, Swedish, Danish, 
		Portuguese, Turkish, Polish, Czech, Rumanian, Serbian,

Dutch, Croatian, Hungarian, and Greek

S. Web Viewer

1. Supported OS 	Windows XP, Windows Vista, Windows 7,

Mac OS X 10.7 or higher

2. Supported Browser	Internet Explorer 7.0 or higher, Firefox, Chrome, Safari

T. Central Management Software 	NET-i viewer

2.07	ELECTRICAL

A. Voltage	12V DC, PoE (IEEE802.3af)

B. Consumption	Maximum 9W (12V DC, PoE(Class3) – Heater off) ,

Maximum 12.95W(PoE(Class3) – Heater on)

2.08	ENVIRONMENTAL SPECIFICATIONS

A. Operating Temperature	-10°C ~ +55°C (14°F ~ 131°F)

B. Operating Humidity	20% ~ 80% RH

2.09	PHYSCIAL SPECIFICATIONS

A. Dimension (WxH)	Φ159.9.0 x 125.0mm (Φ6.3” x 4.92”)

(Flush mounting type)

B. Weight		700g (1.54 lb)

C. Color	Ivory

D. Material	Plastic

2.10	CERTIFICATIONS

A. CE mark

B. FCC mark

2.11	WARRANTY

A. 3 years, parts and labor.