SND L6013R 2 MEGAPIXEL NETWORK IR DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SND L6013R 2 MEGAPIXEL NETWORK IR DOME CAMERA

- The camera shall be of a DOME type suitable for interior installation. The camera shall be ivory in appearance with a plastic body.

- The camera shall provide video compression dual codec ( H.264 and MJPEG).

- The network camera shall feature up to 2 Mega Pixel resolution in a 16:9 format. 4:3 format shall also be available in smaller resolutions.

- The camera shall feature an Adaptive Infrared illumination system with IR LEDs giving a viewable distance of 15m (49.21ft) in complete darkness.

2.03	CAMERA

| A. Imaging Device |  | 1/2.9" 2.19M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 2,000(H) x 1,121(V) |
| C. Effective Pixels |  | 1,984(H) x 1,105(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.15Lux (1/30sec, F1.8), 0.003Lux (2sec, F1.8) |
|  | 2. B/W | 0Lux (IR LED on) |

2.04	LENS TYPE

| A. Focal Length (Zoom Ratio) | 3.6mm fixed |
| --- | --- |
| B. Max. Aperture Ratio | F1.8 |
| C. Angular Field of View | H: 86.5˚ / V: 47.8˚ / D: 101.2˚ |
| D. Min. Object Distance | 0.5m (1.64ft) |
| E. Lens Type | Fixed |
| F. Mount Type | Board type |

2.05	PAN/TILT/ROTATE

| A. Pan Range | 0 ˚ ~ 350 ˚ |
| --- | --- |
| B. Tilt Range | 0 ˚ ~ 67 ˚ |
| C. Rotate Range | 0 ˚ ~ 355 ˚ |

2.06	OPERATION

| A. Camera Title | Off / On (Displayed up to 15 characters) |
| --- | --- |
| B. Day & Night | Electric Day & Night |
| C. Backlight Compensation | Off / BLC |
| D. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| E. Digital Noise Reduction | SSNR (Off / On) |
| F. Motion Detection | Off / On (4ea rectangular zones) |
| G. Privacy Masking | Off / On (6ea rectangular zones) |
| H. Gain Control | Off / Low / Middle / High |
| I. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| J. LDC(Lens Distortion Correction) | Off / On (5 levels with min / max) |
| K. Electronic Shutter Speed | Minimum / Maximum / Anti flicker |
| L. Flip / Mirror | Flip / Mirror / Hallway view |
| M. Intelligent Video Analytics | Motion detection with metadata, Tampering |
| N. Alarm Triggers | Motion detection, Tampering detection, SD card error, NAS error |
| O. Alarm Events | File upload via FTP and E-mail Local storage recording at event Notification via E-mail |
| P. IR Viewable Length | 15m(49.21ft) |

2.07	NETWORK PROTOCOL

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264, MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 720 x 576, 640 x 480, 320 x 240 |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 30fps at all resolutions |
|  | 2. MJPEG | Max. 1fps@1920 x 1080, 1280 x 960, 1280 x 720, 1024 x 768 Max. 15fps@other resolution |
| E. Video Quality Adjustment |  |  |
|  | 1. H.264 | Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| F. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| G. Streaming Capability |  | Multiple streaming (Up to 3 profiles) |
| H. Audio In |  | Built-in mic |
| I. Audio Compression Format |  | G.711 u-law / G.726 selectable G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| J. Audio Communication |  | Uni-directional audio |
| K. IP |  | IPv4, IPv6 |
| L. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| M. Security |  | HTTPS(SSL) login authentication Digest login authentication IP address filtering User access log 802.1x authentication |
| N. Streaming Method |  | Unicast / Multicast |
| O. ax. User Access |  | 6 users at unicast mode |
| P. Edge Storage |  | micro SD/SDHC max. 32G, NAS - Motion images recorded in the SD memory card can be downloaded - Manual recording at local PC |
| Q. Application Programming interface |  | ONVIF Profile S, G, SUNAPI (HTTP API) |
| R. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| S. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 ~ 10.10 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 8 ~ 11), Mozilla Firefox (Ver. 9 ~ 35), Google Chrome (Ver. 15 ~ 40), Apple Safari (Ver. 8.0.2(Mac OS X 10.10), 7.0.6(Mac OS X 10.9), 6.0.2 (Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| T. Central Management Software |  | SmartViewer |

2.08	ELECTRICAL

| A. Voltage | PoE (IEEE802.3af, Class2) |
| --- | --- |
| B. Consumption | Max. 5.9W |

2.09	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C (+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.10	PHYSCIAL SPECIFICATIONS

| A. Dimension | Ø110.0 x 86.0mm (Ø4.33" x 3.39") |
| --- | --- |
| B. Weight | 250g (0.55 lb) |
| C. Color/Material | Ivory / Plastic |

2.11	CERTIFICATIONS

A. CE mark

B. FCC mark

2.12	WARRANTY

A. 3 years, parts and labor.