SNV 6012M 2MEGAPIXEL FULL HD VANDAL-RESISTANT NETWORK MOBILE FLAT CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

- All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

- All systems and components shall have been thoroughly tested and proven in actual use.

- All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNV 6012M 2MEGAPIXEL FULL HD VANDAL-RESISTANT NETWORK MOBILE FLAT CAMERA

- The camera shall be of a flat dome type suitable for mobile installation.

- The camera shall satisfy the standard of vibration resistance, EN50155 and EN50121 for mobile installation.

- The camera shall be ivory in appearance with an aluminum body.

- The camera shall provide video compression dual codec ( H.264 and MJPEG).

- The network camera shall feature up to 2 Megapixel resolution in a 16:9 format.

- The camera shall provide Defog and WDR.

- The camera shall provide memory slot for SD, SDHC and SDXC.

- The camera shall provide M12 connector.

- All operation and configuration menus shall be provided in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, and Finnish.

2.03	CAMERA

| A. Imaging Device |  | 1/2.8" 2.38M CMOS (IMX136) |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.3Lux (1/30sec, F1.2, 50IRE) 0.005Lux (2sec, F1.2, 50IRE) 0.15Lux (1/30sec, F1.8, 50IRE) |
|  | 2. B/W | 0.3Lux (1/30sec, F1.8, 50IRE) 0.15Lux (F1.8, 30IRE) |
| F. S/N Ratio |  | 50dB |
| G. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |

2.04	LENS TYPE

| A. Focal Length (Zoom Ratio) | 3mm fixed |
| --- | --- |
| B. Max. Aperture Ratio | F2.0 |
| C. Angular Field of View | H : 103.1° / V : 53.9° / D : 124.5° |
| D. Lens Type | Fixed iris |
| E. Mount Type | Board-in type |

2.05	PAN/TILT/ROTATE

| A. Pan Range | -10° ~ 10° |
| --- | --- |
| B. Tilt Range | 0° ~ 90° |

2.06	OPERATION

| - Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| - Day & Night | Auto (Electrical) / Color / B/W / Schedule |
| - Backlight Compensation | Off / BLC |
| - Wide Dynamic Range | 120dB |
| - Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| - Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| - Digital Image Stabilization | Off / On |
| - Defog | Off / Auto / Manual |
| - Motion Detection | Off / On (4ea 4 points polygonal zones) |
| - Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| - Gain Control | Off / Low / Middle / High |
| - White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| - Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| - Flip / Mirror | Off / On |
| - Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear Face detection |
| - Alarm Triggers | Motion detection, Face detection, Video analytics, Network disconnection, Tampering |
| - Alarm Events | File upload via FTP and E-mail Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at Event (Alarm triggers) |

2.07	NETWORK PROTOCOL

| A. Ethernet |  | 10Base-T / 100Base-TX M12 connector (4poles, Female, D-coding) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 60fps @ all resolutions |
|  | 2. MJPEG | Max. 30fps @ 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 Max. 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 |
| E. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| F. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| G. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| H. IP |  | IPv4, IPv6 |
| I. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| J. Security |  | HTTPS(SSL) login authentication Digest login authentication IP address filtering User access log, 802.1x authentication |
| K. Streaming Method |  | Unicast / Multicast |
| L. Max. User Access |  | 15 users at unicast mode |
| M. Edge Storage |  | SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| N. Application Programming interface |  | ONVIF profile S, HTTP API (SUNAPI) 2.0, SVNP 1.2 |
| O. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| P. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 11 ~ 8) Mozilla Firefox (Ver. 19 ~ 9) * Windows only Google Chrome (Ver. 25 ~ 15) * Windows only Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| Q. Central Management Software |  | SmartViewer |
| R. Mobile Viewer |  | iPOLiS mobile |

2.08	ELECTRICAL

| A. Voltage | PoE (Class A type only) |
| --- | --- |
| B. Consumption | Max. 7.5W |

2.09	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -30°C ~ +55°C (-22°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |
| C. Ingress Protection | IEC 60529 IP66, NEMA 250 type 4X |
| D. Vandal Resistance | IEC 62262 class IK10 (20J) |
| E. Vibration Resistance | EN 50155 class T3 (Vibration, Shock, Bump, Temperature) EN 50121 ISO 16750-3 (Vibration) |
| F. UL | UL60590-1 |

2.10	PHYSCIAL SPECIFICATIONS

| A. Dimension | 135.2 x 54.3 x 124.5mm (5.32" x 2.14" x 4.9") |
| --- | --- |
| B. Weight | 530g (1.17 lb) |
| C. Color/Material | Ivory / Aluminum |

2.11	CERTIFICATIONS

- CE mark

- FCC mark

- UL mark

2.12	WARRANTY

A. 3 years, parts and labor.