SNV-6013 2MEGAPIXEL FULL HD VANDAL-RESISTANT NETWORK DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNV-6013 2MEGAPIXEL FULL HD VANDAL-RESISTANT NETWORK DOME CAMERA

- The network camera shall offer 2M resolution (1,920 x 1,080) maximum.

- The cameras shall provide 0.15 Lux at F1.8 in color and 0.15 Lux at F1.8 in black and white.

- The camera shall have built-in 2.8 fixed lens.

- The camera shall provide WDR up to 120 dB, DIS and Defog.

- The camera shall provide MD, FD and video analytics.

- The camera shall provide Audio, Face and Motion detection.

- The camera shall provide memory slot for SD, SDHC and SDXC.

- The camera shall provide PoE, IP66, IK10.

- All operation and configuration menus shall be provided in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Finnish and Norwegian.

2.03	VIDEO

| A. Imaging Device |  | 1/2.8" 2.38M CMOS (IMX136) |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.3Lux (1/30sec, F1.8, 50IRE) 0.15Lux (1/30sec, F1.8, 30IRE) |
|  | 2. B/W | 0.3Lux (1/30sec, F1.8, 50IRE) 0.15Lux (1/30sec, F1.8, 30IRE) |
| F. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite 706 x 480(N), 704 x 576(P), for installation DIP connector type |

2.04	LENS

| A. Focal Length (Zoom Ratio) | 2.8mm fixed |
| --- | --- |
| B. Max. Aperture Ratio F1.8 | F1.8 |
| C. Angular Field of View | H : 112° / V : 62° |
| D. Min. Object Distance 0.2m (0.66ft) | 0.2m (0.66ft) |
| E. Lens Type Fixed | Fixed |
| F. Mount Type Board-in type | Board-in type |

2.05	OPERATION

| A. Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| B. Day & Night | Auto (Electrical) / Color / B/W |
| C. Backlight Compensation | Off / BLC |
| D. Wide Dynamic Range | 120dB |
| E. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| F. Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| G. Digital Image Stabilization | Off / On |
| H. Defog | Off / Auto / Manual |
| I. Motion Detection | Off / On (4ea 4 points polygonal zones) |
| J. Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| K. Gain Control | Off / Low / Middle / High |
| L. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| M. Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| N. Digital Zoom | 16x |
| O. Flip / Mirror | Off / On |
| P. Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Face detection |
| Q. Alarm Triggers | Motion detection, Tampering, Face detection, Video analytics, Network disconnection |
| R. Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at Event (Alarm triggers) |

2.06	NETWORK

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Frame rate |  |  |
|  | 1. H.264 | Max. 60fps@ all resolutions |
|  | 2. MJPEG | Max. 15fps@ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 Max. 30fps@ 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Smart Codec |  | Yes (Area based method) |
| F. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| G. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| H. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| I. IP |  | IPv4, IPv6 |
| J. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| K. Security |  | HTTPS(SSL) login authentication, Digest login authentication, IP address filtering, User access log, 802.1x authentication |
| L. Streaming Method |  | Unicast / Multicast |
| M. Max. User Access |  | 15 users at unicast mode |
| N. Edge Storage |  | SD/SDHC/SDXC - Motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| O. Application Programming interface |  | ONVIF profile S, HTTP API (SUNAPI) 2.0, SVNP 1.2 |
| P. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Finnish, Norwegian |
| Q. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 11 ~ 8) Mozilla Firefox (Ver. 19 ~9) * Windows only Google Chrome (Ver. 25 ~ 15) * Windows only Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| R. Central Management Software |  | SmartViewer |

2.07	ENVIRONMENTAL

| A. Operating Temperature / Humidity | -30°C ~ +55°C (-22°F ~ +131°F) / Less than 90% RH * Start up should be done at above -10°C (+14°F) |
| --- | --- |
| B. Power Consumption | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |
| C. Ingress Protection | IP66 |
| D. Vandal Resistance | IK10 |

2.08	Electrical

| A. Input Voltage / Current | PoE (IEEE802.3af, Class3) |
| --- | --- |
| B. Power Consumption | Max. 7.5W |

2.09	Mechanical

| A. Color / Material | Ivory / Aluminum |
| --- | --- |
| B. Dimension (WxHxD) | Ø112.8 x 63.7mm (Ø4.44" x 2.51") |
| C. Weight | 550g (1.21 lb) |

2.10	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.11	WARRANTY

A. 3 years, parts and labor.