SNV 6084R 2MEGAPIXEL FULL HD VANDAL-RESISTANT NETWORK IR DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

- All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

- All systems and components shall have been thoroughly tested and proven in actual use.

- All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNV 6084R 2MEGAPIXEL FULL HD VANDAL-RESISTANT NETWORK IR DOME CAMERA

- The camera shall offer Max. 2M (1920 x 1080) resolution.

- The cameras shall support Min. illumination of 0.03Lux at F1.2 when in color mode and 0Lux at F1.2 when in black and white mode with IR LED on.

- The camera shall have 3 ~ 8.5mm (2.8x) motorized varifocal lens.

- The camera shall provide H.264, MJPEG dual codec and multiple streaming.

- The network camera shall feature WDR (120dB), Simple focus (Motorized V/F), P-Iris, and Defog.

- The camera shall feature Audio / Face / Motion detection.

- The camera shall provide memory slot for SD/SDHC/SDXC.

- The camera shall provide Multi-crop streaming

- The camera shall provide PoE, and UPnP.

- The camera shall have IR LED (12ea) and IP66/IK10 certifications.

- The camera shall support Bi-directional audio

2.03	CAMERA

| A. Imaging Device |  | 1/2.8" 2.38M CMOS (IMX136) |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.1Lux (1/30sec, F1.2, 50IRE), 0.0017Lux (2sec, F1.2, 50IRE), 0.03Lux (F1.2, 30IRE) |
|  | 2. B/W | 0Lux (F1.2, 50IRE, IR LED on), 0Lux (F1.2, 30IRE, IR LED on) |
| F. S/N Ratio |  | 50dB |
| G. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 704 x 480(N), 704 x 576(P), for installation DIP connector type |

2.04	LENS TYPE

| A. Focal Length (Zoom Ratio) | 3 ~ 8.5mm (2.8x) motorized varifocal |
| --- | --- |
| B. Max. Aperture Ratio | F1.2 |
| C. Angular Field of View | H : 105.5°(Wide) ~ 37.1°(Tele) V : 57.5°(Wide) ~ 21.0°(Tele) |
| D. Min. Object Distance | 0.5m (1.64ft) |
| E. Focus Control | Simple focus (Motorized V/F) Manual, Remote control via network (Manual, Simple focus) |
| F. Lens / Mount Type | DC auto iris, P-Iris / Board-in type |

2.05	PAN/TILT/ROTATE

| A. Pan Range | 0° ~ 354° |
| --- | --- |
| B. Tilt Range | 0° ~ 67° |
| C. Rotate Range | 0° ~ 355° |

2.06	OPERATION

| - IR LED | 12ea |
| --- | --- |
| - Viewable Length | 15m (49.21ft) |
| - Camera Title | Off / On (Displayed up to 45 characters) |
| - Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| - Backlight Compensation | Off / BLC |
| - Wide Dynamic Range | 120dB |
| - Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| - Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| - Digital Image Stabilization | Off / On |
| - Defog | Off / Auto / Manual |
| - Motion Detection | Off / On (4ea 4 points polygonal zones) |
| - Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| - Gain Control | Off / Low / Middle / High |
| - White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| - Electronic Shutter Speed | Minimum / Maximum / Anti flicker |
| - Flip / Mirror | Off / On |
| - Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection |
| - Alarm I/O | Input 1ea / Output 1ea |
| - Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Network disconnection, Alarm input |
| - Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at Event (Alarm triggers), External output |

2.07	NETWORK PROTOCOL

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Frame rate |  |  |
|  | 1. H.264 | Max. 60fps @ all resolutions |
|  | 2. MJPEG | Max. 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 Max 30fps at 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| F. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| G. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| H. Audio In |  | Selectable (Mic in / Line in), Max output level : 1 Vrms Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| I. Audio Out |  | Line out (3.5mm stereo mini jack) |
| J. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| K. Audio Communication |  | Bi-directional audio |
| H. IP |  | IPv4, IPv6 |
| I. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| J. Security |  | HTTPS(SSL) login authentication, Digest login authentication IP address filtering, User access log, 802.1x authentication |
| K. Streaming Method |  | Unicast / Multicast |
| L. Max. User Access |  | 15 users at unicast mode |
| M. Edge Storage |  | SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| N. Application Programming interface |  | ONVIF profile S & G, HTTP API (SUNAPI) 2.0, SVNP 1.2 |
| O. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| P. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 11 ~ 8) Mozilla Firefox (Ver. 19 ~ 9) * Windows only Google Chrome (Ver. 25 ~ 15) * Windows only Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| Q. Central Management Software |  | SmartViewer |

2.08	ELECTRICAL

| A. Input Voltage / Current | 12V DC ±10%, 24V AC ±10%, PoE (IEEE802.3af) |
| --- | --- |
| B. Power Consumpt | Max. 9.5W (Heater off), Max. 13.5W (24V AC - Heater on) |

2.09	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | 24V AC : -40°C ~ +55°C (-40°F ~ +131°F) / Less than 90% RH 12V DC, PoE : -20°C ~ +55°C (-4°F ~ +131°F) / Less than 90% RH * Start up should be done at above -10°C (+14°F) |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |
| C. Ingress Protection | IP66 |
| D. Vandal Resistance | IK10 |

2.10	PHYSCIAL SPECIFICATIONS

| A. Dimension | Ø160.0 x 118.5mm (Ø6.3" x 4.67") |
| --- | --- |
| B. Weight | 960g (2.12 lb.) |
| C. Color/Material | Ivory / Aluminum |

2.11	CERTIFICATIONS

- CE mark

- FCC mark

- UL mark

2.12	WARRANTY

A. 3 years, parts and labor.