SNV-6085R 2Megapixel Full HD Vandal-Resistant Network IR Dome Camera

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETIES AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNV-6085R 2Megapixel Full HD Vandal-Resistant Network IR Dome Camera

- The camera shall be of a vandal-resistant dome type with aluminum body. The camera shall be ivory in appearance.

- The camera shall feature up to 2 Megapixel Full HD resolution (1920x1080) in a 16:9 format. 4:3 format shall also be available in other resolutions.

- The camera shall offer dual codec, H.264 and MJPEG, with capability to stream up to 10 videos simultaneously.

- The camera shall offer maximum frame rate of 60 fps at all resolutions of videos encoded by H.264 codec. For videos encoded by MJPEG, maximum frame rate shall be 15 fps at resolutions from 1920x1080 to 1024x768 and 30 fps at resolutions from 800x600 to 320x180.

- When in color mode, the minimum illumination of the camera shall be 0.09 Lux at F1.6. When in black and white mode, it shall be 0 Lux with IR LED on.

- The network camera shall feature motorized varifocal lens that varies between10 mm and 23 mm with zoom ratio of 2.3.

- The camera shall feature Wide Dynamic Range with a gain of 120db. This feature will work at 30 images per second at 1920x1080 resolution. The WDR function shall feature an adaptive motion system to eliminate motion blur.

- The camera shall feature Simple Focus, an automatic motorized focus adjustment. This should be activated by a button on the camera and remotely through the network interface. This can also be activated when the camera changes from day to night mode.

- The camera shall feature P-Iris which automatically adjusts iris opening and makes adjustment using software to respond to changes in light. This feature shall deliver improved clarity and contrast.

- The camera shall feature a high performance 2D & 3D noise reduction that automatically adapts the type of technology used according to movement in the field of view. 2D noise reduction compares adjacent pixels while 3D technology compares the same pixel in the previous and subsequent images, giving a higher level of detail. When the camera detects movement in a region of the image it will apply 2D noise reduction to that area and 3D noise reduction to the remainder of the image.

- The camera shall feature an automatic back light compensation technology that detects and enhances dark areas in the field of view and increases the gain in those areas.

- The camera shall feature built-in license-free video analytics including line crossing, appear/disappear, audio detection, camera tamper (scene change) and enter/exit a predefined zone.

- The camera shall feature advanced motion detection with definable detection areas, minimum / maximum object size definition and a learning algorithm that ignores false alarms such as trees and waves on water.

- The camera shall support Multi Cropping technology to allow video profiles to be set up with a reduced area of the overall field of view of the camera.

- The camera shall feature a face detection technology that can be used to create an event whenever there is a face or multiple faces in the image. The technology should be able to detect 32 faces simultaneously.

- The camera shall feature a smart codec that can use a higher quality compression for regions of interest in the image, thereby prioritizing the encoding of the most important areas of the field of view.

- The camera shall feature a Digital Image Stabilization function.

- The camera shall support ONVIF Profile S for operations with 3rd party systems.

- The camera shall be capable of simultaneously transmitting multiple video streams of different resolution, compression, frame-rate and compression settings.

- The network camera should be configurable through a built in web server that can be accessed via standard browsers including Internet Explorer, Firefox, Chrome & Safari.

- The camera shall feature a line level audio input with an alarm function.

- The camera shall feature a line level audio input and output capable of duplex operation.

- The cameras shall support micro SD, SDHC & SDXC flash memory card for recording video footage on event, network loss or continuously.

- The camera shall provide a customizable on-screen display (OSD) which shall be available in English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Romanian, Serbian, Dutch, Croatian, Hungarian, Greek, Finnish, Norwegian

2.03	VIDEO

| A. Imaging Device |  | 1/2.8" 2.38M CMOS (IMX136) |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V) |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.25Lux (F1.6, 50IRE), 0.09Lux (F1.6, 30IRE) |
|  | 2. B/W | 0Lux (F1.6, 50IRE, IR LED on), 0Lux (F1.6, 30IRE, IR LED on) |
| F. S / N Ratio |  | 50dB |
| G. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |

2.04	LENS

| A. Focal Length (Zoom Ratio) | 10 ~ 23mm (2.3x) motorized varifocal |
| --- | --- |
| B. Max. Aperture Ratio | F1.6 |
| C. Angular Field of View | H : 31.3°(Wide) ~ 13.9°(Tele) V : 17.1°(Wide) ~ 7.8°(Tele) |
| D. Min. Object Distance | 0.8m (2.62ft) |
| E. Focus Control | Simple focus (Motorized V/F) Manual, Remote control via network (Manual, Simple focus) |
| F. Lens Type | P-Iris |
| G. Mount Type | Board-in type |

2.05      PAN / TILT / ROTATE

| A. Pan / Tilt / Rotate Range | 0° ~ 354° / 0° ~ 67° / 0° ~ 355° |
| --- | --- |

2.06	OPERATION

| A. IR LED | 14ea |
| --- | --- |
| B. Viewable Length | 20m (65.62ft) |
| C. Camera Title | Off / On (Displayed up to 45 characters) |
| D. Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| E. Backlight Compensation | Off / BLC |
| F. Wide Dynamic Range | 120dB |
| G. Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| H. Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| I. Digital Image Stabilization | Off / On |
| J. Defog | Off / Auto / Manual |
| K. Motion Detection | Off / On (4ea 4 points polygonal zones) |
| L. Privacy Masking | Off / On (32 zones with 4 points of polygonal) |
| M. Gain Control | Off / Low / Middle / High |
| N. White Balance | ATW / AWC / Manual / Indoor / Outdoor |
| O. Electronic Shutter Speed | Minimum / Maximum / Anti flicker |
| P. Flip / Mirror | Off / On |
| Q. Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection |
| R. Alarm I/O | Input 1ea / Output 1ea |
| S. Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Network disconnection, Alarm input |
| T. Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage (SD/SDHC/SDXC) or NAS recording at Event (Alarm triggers), External output |

2.07	NETWORK

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Frame rate |  |  |
|  | 1. H.264 | 60fps @ all resolutions |
|  | 2. MJPEG | 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 30fps @ 800 x 600, 800 x 450, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Smart Codec |  | Manual mode (Area-based : 5ea), Face detection mode |
| F. Video Quality Adjustment |  |  |
|  | 1. H.265/H.264 | Compression level, Target bitrate level control, |
|  | 2. MJPEG | Quality level control |
| G. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| H. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| I. Audio In |  | Selectable (Mic in / Line in), Max output level : 1 Vrms Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| J. Audio Out |  | Line out (3.5mm stereo mini jack) |
| K. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| L. Audio Communication |  | Bi-directional audio |
| M. IP |  | IPv4, IPv6 |
| N. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| O. Security |  | HTTPS(SSL) login authentication, Digest login authentication IP address filtering, User access log, 802.1x authentication |
| P. Streaming Method |  | Unicast / Multicast |
| Q. Max. User Access |  | 15 users at unicast mode |
| R. Edge Storage |  | micro SD/SDHC/SDXC Max. 64G, NAS - Motion images recorded in the SD/SDHC/SDXC memory card can be downloaded. Manual recording at local PC |
| S. Application Programming interface |  | ONVIF profile S & G, HTTP API (SUNAPI) |
| T. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Norwegian, Finnish |
| U. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 ~ 10.10 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 8 ~ 11), Mozilla Firefox (Ver. 9 ~ 35), Google Chrome (Ver. 15 ~ 44) Apple Safari (Ver. 8.0.2 (Mac OS X 10.10), 7.0.6 (Mac OS X 10.9), 6.0.2 (Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| V. Central Management Software |  | SmartViewer |

2.08	ENVIRONMENTAL

| A. Operating Temperature / Humidity | -40°C ~ +55°C (-40°F ~ +131°F) / Less than 90% RH * Start up should be done at above -35°C (-31°F) |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |
| C. Ingress Protection | IP66 |
| D. Vandal Resistance | IK10 |

2.09	Electrical

| A. Input Voltage / Current | 12V DC ±10%, 24V AC ±10%, PoE (IEEE802.3af) |
| --- | --- |
| B. Power Consumption | Max. 11W (Heater off), Max. 12.95W (Heater on) |

2.10	Mechanical

| A. Color / Material | Ivory / Aluminum |
| --- | --- |
| B. Dimension (WxHxD) | Ø160.0 x 118.5mm (Ø6.3" x 4.67") |
| C. Weight | 970g (2.14 lb) |

2.11	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.12	WARRANTY

A. 3 years, parts and labor.