SNZ 6320 2MEGAPIXEL FULL HD 32X NETWORK ZOOM CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

- All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

- All systems and components shall have been thoroughly tested and proven in actual use.

- All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNZ 6320 2MEGAPIXEL FULL HD 32X NETWORK ZOOM CAMERA

- The camera shall offer Max. 2M (1920 x 1080) resolution.

- The camera shall offer 16:9 Full HD (1080p) resolutions.

- The cameras shall support Min. illumination of 0.15Lux at F1.6 when in color mode and 0.015Lux at F1.6 when in black and white mode.

- The camera shall offer 4.44 ~ 142.6mm (32x) optical zoom and 16x digital zoom.

- The camera shall provide H.264, MJPEG dual codec, and multiple streaming.

- The network camera shall feature Day & Night (ICR) and WDR (120dB)

- The camera shall feature intelligent video analytics.

- The camera shall provide PoE.

- The camera shall provide SD/SDHC/SDXC memory slot.

- The camera shall provide Bi-directional audio.

2.03	CAMERA

| A. Imaging Device |  | 1/2.8" 2.38M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V), 2.18M |
| C. Effective Pixels |  | 1,944(H) x 1,104(V) , 2.14M |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.3Lux (1/30sec, F1.6, 50IRE), 0.15Lux (1/30sec, F1.6, 30IRE) |
|  | 2. B/W | 0.03Lux (1/30sec, F1.6, 50IRE), 0.015Lux (1/30sec, F1.6, 30IRE) |
| F. S/N Ratio |  | 50dB |
| G. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation DIP connector type |

2.04	LENS TYPE

| A. Focal Length (Zoom Ratio) | 4.44 ~ 142.6mm (Optical 32x) |
| --- | --- |
| B. Max. Aperture Ratio | F1.6(Wide) / F4.4(Tele) |
| C. Angular Field of View | H : 62.8˚(Wide) ~ 2.23˚(Tele) V : 36.80˚(Wide) ~ 1.26˚(Tele) |
| D. Min. Object Distance | 1.5m (4.92ft) |
| E. Focus Control | Auto / Manual / One push |
| F. Lens / Mount Type | DC auto iris / Board-in type |

2.05	OPERATION

| - Camera Title | Off / On (Displayed up to 45 characters) |
| --- | --- |
| - Day & Night | Auto (ICR) / Color / B/W / External / Schedule |
| - Backlight Compensation | Off / BLC |
| - Wide Dynamic Range | 120dB |
| - Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| - Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| - Digital Image Stabilization | Off / On |
| - Defog | Off / Auto / Manual |
| - Motion Detection | Off / On (4ea 4 points polygonal zones) |
| - Privacy Masking | Off / On (32 zones with 2 points of rectangle) |
| - Gain Control | Off / Low / Middle / High |
| - White Balance | ATW / AWC / Manual / Indoor / Outdoor / Mercury |
| - Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| - Digital Zoom | 16x |
| - Flip / Mirror | Off / On |
| - Intelligent Video Analytics | Tampering, Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection |
| - Alarm I/O | Input 1ea / Output 1ea (Relay) |
| - Remote Control Interface | RS-485 |
| - RS-485 Protocol | Samsung-T/E, Pelco-P/D, Panasonic, Honeywell, AD, Vicon, Bosch, GE |
| - Alarm Triggers | Motion detection, Tampering, Audio detection, Face detection, Video analytics, Alarm input, Network disconnection |
| - Alarm Events | File upload via FTP and E-mail, Notification via E-mail, Local storage(SD/SDHC/SDXC) or NAS recording at Network disconnected & Event (Alarm Triggers), External output |

2.06	NETWORK PROTOCOL

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 part 10/AVC), MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 800 x 450, 720 x 576, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Frame rate |  |  |
|  | 1. H.264 | Max. 60fps at all resolutions |
|  | 2. MJPEG | Max. 15fps at 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 Max. 30fps at 800 x 600, 800 x 450, 720 x 576, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Smart Codec |  | Manual mode (Area-based : 5ea), Face detection mode |
| F. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| G. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| H. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| I. Audio In |  | Selectable (Mic in / Line in), Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| J. Audio Out |  | Line out (3.5mm stereo mini jack), Max output level : 1 Vrms |
| K. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz, G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| L. Audio Communication |  | Bi-directional audio |
| M. IP |  | IPv4, IPv6 |
| N. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| O. Security |  | HTTPS(SSL) login authentication, Digest login authentication IP address filtering, User access log, 802.1x authentication |
| P. Streaming Method |  | Unicast / Multicast |
| Q. Max. User Access |  | 10 users at unicast mode |
| R. Edge Storage |  | SD/SDHC/SDXC - motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded. NAS (Network Attached Storage) |
| S. Application Programming interface |  | ONVIF profile S, SUNAPI 2.0, SVNP 1.2 |
| T. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Finnish, Norwegian |
| U. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8 / 8.1, MAC OS X 10.7 / 10.8 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver. 8 ~ 11), Mozilla Firefox (Ver. 9 ~ 19), Google Chrome (Ver. 15 ~ 32) Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7)* Mac OS X only |
| V. Central Management Software |  | SmartViewer, SSM, Mobile viewer |

2.07	ELECTRICAL

| A. Input Voltage / Current | 12V DC, PoE (IEEE802.3af, Class3) |
| --- | --- |
| B. Power Consumption | Max. 9W (12V DC) / Max. 10W (PoE) |

2.08	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -10°C ~ +55°C (+14°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |

2.09	PHYSCIAL SPECIFICATIONS

| A. Dimension | 72.0 x 59.9 x 135.4mm (2.83" x 2.36" x 5.33") |
| --- | --- |
| B. Weight | 540g (1.19 lb.) |
| C. Color/Material | Dark gray / Metal |

2.10	CERTIFICATIONS

- CE mark

- FCC mark

- UL mark

2.11	WARRANTY

A. 3 years, parts and labor.