Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

NETWORK CONTROLLER

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

# MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00 Data Communications

# 28 23 13 Video Surveillance Control and Management Systems

# 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19 Digital Video Recorders and Analog Recording Devices

# 28 23 23 Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

28 2x 00	Video Management System

NETWORK CONTROLLER

- GENERAL

  - SUMMARY

    - Section includes a network controller.

    - Product - A network controller with 3D joystick to control Techwin’s network products including network cameras and SSM.

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Back light compression

      - CBR – Constant Bit Rate

      - CVBS – Color, Video, Blanking, Sync

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - DSCP – Differentiated Services Code Point

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GOV – Group of Video

      - GUI – Graphical User Interface

      - HD – High Definition

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - MP - Megapixel

      - MPEG - Moving Pictures Experts Group

      - NAS – Network Attached Storage

      - NTP - Network Time Protocol

      - PIM-SM - Protocol Independent Multicast-Sparse Mode

      - PoE - Power over Ethernet

      - PPPoE – Point to Point Protocol over Ethernet

      - RTP - Real-Time Transport Protocol

      - RTCP – Real-Time Control Protocol

      - RTSP - Real-Time Streaming Protocol

      - SDK – Software Development Kit

      - SMTP - Simple Mail Transfer Protocol

      - SNMP – Simple Network Management Protocol

      - SSL – Secure Sockets Layer

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - UPnP – Universal Plug and Play

      - VBR – Variable Bit Rate

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

      - LDC – Lens Distortion Correction

      - TFT – Thin Film Transistor

      - LCD – Liquid Crystal Display

      - PTZ – Pan, Tilt, Zoom

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

      - Video

        - H.265

        - H.264

        - MJPEG

        - ONVIF – Profiles S and G

      - Emissions

        - FCC-47 CFR Part 15 Subpart B Class B

        - CE EN 55022:2010

      - Immunity - CE

        - EN 50130-4:2011

        - EN 61000-3-3:2014

        - EN 61000-4-2:2009

        - EN 61000-4-3:2006+A2:2010

        - EN 61000-4-4:2012

        - EN 61000-4-5:2014

        - EN 61000-4-6:2009

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment specified in section 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded camera video analytics free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://security.hanwhatechwin.com/

    - Model		SPC-7000

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - The controller shall have the following properties.

      - It shall have 3D joystick (3-axis twist zoom) to control Techwin’s network products including network cameras and SSM.

      - It shall have a jog shuttle to control playback of videos.

      - The joystick and jog shuttle shall be detachable.

      - It shall have 5 inch TFT touch LCD on which users can change settings and control PTZ.

      - It shall have a USB port to connect to a PC with SSM.

      - The controller shall provide camera-tile mapping.

      - The controller shall be able to import or export configuration.

      - Available camera control

        - PTZ

        - Focus

        - Auto Focus

        - Preset

        - Group

        - Tour

        - Swing

        - Trace

        - Auto Tracking

        - Alarm off/on

        - Freeze

      - Available video wall control

        - Camera-tile (Channel) mapping

        - Layout (Display division mode) change

      - The controller shall be able to take a snapshot in a jpeg format and store it to the storage device connected to its USB port.

      - The controller shall be able to upgrade firmware from the storage device connected to its USB port.

      - It shall be able to control up to 255 network cameras and 1 SSM.

  - CONTROLLER SOFTWARE

    - Installation

      - When the controller is connected to a PC, it shall automatically detect the device and start installation.

  - DETAILED SPECIFICATIONS

    - Communication

      - Control Equipment	Network camera / SSM

      - Interface		Ethernet (Network camera), USB (SSM) 1ea, USB 1ea (upgrade,

configuration import/export, image snapshot)

      - Interface protocol

        - Ethernet		SUNAPI

        - USB		dedicated

      - Units to control 		Max. 255 network cameras, 1 SSM.

      - Network Protocol	TCP, HTTP, DHCP, IPv4

    - Operational Mechanical

      - LCD Display		5 inch TFT LCD

      - Joystick			3 axis twist zoom

      - Jog Shuttle		SSM playback control

    - Control

      - Camera			Pan/Tilt/Zoom, Focus, Auto focus, Preset, Group, Tour,

Swing, Trace, Auto tracking, Alarm off/on, Freeze

      - Video Wall 		Camera-tile (channel) mapping, layout change (display division mode)

      - Recorder		Recording, playback control, alarm off, menu

      - Device management	Built-in device registration, modification on LCD

      - User management	admin + 9 operators

      - Import, Export		Configuration import/export

    - Miscellaneous

      - Video snapshot		Still image (network camera) export using USB memory

      - Input			Alphabet input (virtual keyboard)

      - Language		English, Spanish, French, Italian, German, Turkish,

Polish, Russian, Korean, Chinese, Japanese

    - Environmental

      - Operating Temperature	0°C ~ 40°C (+32°F ~ +104°F)

      - Operating Humidity	20 ~ 85%

    - Electrical

      - Input voltage		12V DC

      - Power consumption	Max. 6.4 W

    - Mechanical

      - Dimension (WxHxD)	459.1 x 59.1 x 177.9mm (18.07" x 2.33" x 7")

      - Weight			1.5Kg (3.3 lb)

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA) or Network Video Recorder (NVR).

    - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

    - The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

    - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

    - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

8