Hanwha Techwin is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/

16CH NETWORK VIDEO ENCODER

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00	Data Communications

# 28 23 13	Video Surveillance Control and Management Systems

# 28 23 16	Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19	Digital Video Recorders and Analog Recording Devices

# 28 23 23	Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

# 28 2x 00	Video Management System

16CH NETWORK VIDEO ENCODER

- GENERAL

  - SUMMARY

    - Section includes a network video encoder

    - Product – A network video encoder max 4MP resolution with 16ch analog camera

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC		Auto Gain Control

      - AES		Advanced Encryption Standard

      - API		Application Programming Interface

      - ARP		Address Resolution Protocol

      - AWB		Auto White Balance

      - BLC		Back light compensation

      - CBR		Constant Bit Rate

      - CVBS		Composite Video Blanking and Sync

      - DHCP		Dynamic Host Configuration Protocol

      - DNR		Digital Noise Reduction

      - DNS		Domain Name Server

      - DDNS		Dynamic Domain Name Server

      - DSCP		Differentiated Services Code Point

      - fps		frames per second

      - FTP		File Transfer Protocol

      - GOV		Group of Video

      - GUI		Graphical User Interface

      - HD		High Definition

      - HTTP		Hypertext Transfer Protocol

      - HTTPS		Secure HTTP

      - ICMP		Internet Control Message Protocol

      - IGMP		Internet Group Management Protocol

      - IP		Internet Protocol

      - IR		Infrared

      - JPEG		Joint Photographic Experts Group

      - LAN 		Local Area Network

      - LED		Light Emitting Diode

      - LDC		Lens Distortion Correction

      - LLDP		Link Layer Discovery Protocol

      - LPR		License Plate Recognition

      - MJPEG		Motion JPEG

      - MP		Megapixel

      - MPEG		Moving Pictures Experts Group

      - NTP		Network Time Protocol

      - NVR		Network Video Recorder

      - PIM-SM	Protocol Independent Multicast-Sparse Mode

      - PoE		Power over Ethernet

      - PPPoE		Point to Point Protocol over Ethernet

      - QoS		Quality of Service

      - RTP		Real-Time Transport Protocol

      - RTCP		Real-Time Control Protocol

      - RTSP		Real-Time Streaming Protocol

      - SDK		Software Development Kit

      - SFP		Small Form factor Pluggable

      - SMTP		Simple Mail Transfer Protocol

      - SNMP		Simple Network Management Protocol

      - SSDR		Super Smart Dynamic Range

      - SSNR		Super Smart Noise Reduction

      - SSL		Secure Sockets Layer

      - TCP		Transmission Control Protocol

      - UDP		User Datagram Protocol

      - UPnP		Universal Plug and Play

      - VBR		Variable Bit Rate

      - VMS		Video Management System

      - WDR		Wide Dynamic Range

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

        - IPv4 IP addressing version 4

        - IPv6 IP addressing version 6

        - QoS Quality of Service

      - Video

        - ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - EMC & Safety

        - FCC 47 CFR Part 15 Subpart B

          - ANSI C63.4-2014 Class A

        - IC Regulation ICES-003 Issue 7

          - ANSI C63.4-2017

        - CE EMC-Directive 2014/30/EU

          - EN 55032:2012/AC:2013 Class A

          - EN 50130-4:2011

          - EN 61000-3-2:2014

          - EN 61000-3-3:2013

        - CE EMC-Regulations 2016/1091

          - BS EN 55032:2012/A1:2013

          - BS EN 50130-4:2011

        - VCCI-CISPR 32:2016 Class A

        - AS/NZS CISPR32:2015 Class A

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

    - Definitions

      - GOV (Group of Video object planes) – A set of video frames for H.264 and H.265 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of two kinds of frames in video surveillance setup: I-Frame and P-Frame.

      - Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Multi-exposure wide dynamic range – Operation which automatically adjusts shutter speed to provide a wide range between dark and light areas visible at the same time, preventing backlighting issues. Long exposure is used for dark areas and a short exposure is used in bright areas.

      - Dynamic fps – Dynamic assignment of fps (frames per seconds) based on the movement of object(s) in the scene to efficiently manage bitrate of the video stream and reduce the storage required.

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment specified in section 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide at least a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded camera video analytics free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://www.hanwha-security.com/

    - Model		SPE-1620

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The encoder shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264 – maximum of 30fps/25fps(60Hz/50Hz) at all resolution

        - MJPEG – maximum of 30fps/25fps(60Hz/50Hz)

      - Input signal support CVBS, AHD(1, 2&4MP), CVI(1 & 2MP), TVI(1 & 2MP) format and detect automatically.

      - Video out support by 1 HDMI, up to 16 ch multiple images with 1920x1080 resolution.

      - The encoder shall be able to use below protocol.

        - RS-485 : SAMSUNG-T, PELCO-P/D with Half duplex

        - Coaxial :

NTSC/PAL : Pelco-C (Coaxitron)

AHD : ACP (AHD Coax protocol)

TVI

CVI

      - The encoder shall be able to configure various resolution selections.

        - 1920 x 1080, 1280 x 720

        - 928 x 480(N), 928 x 240(N), 704 x 480(N), 704 x 240(N), 352 x 240(N)

        - 928 x 576(P), 928 x 288(P), 704 x 576(P), 704 x 288(P), 352 x 288(P)

      - The encoder supports the following frame rate.

        - Main/Sub stream :

1080P input : 1080p/720p/WD1/4CIF/CIF : 15fps/CH(N), 12fps/CH(P)

720P input : 720p/WD1/4CIF/CIF : 30fps/CH(N), 25fps/CH(P)

SD input :928x480(576)/704x480(576)/928x240(288)/704x240(288)

/480x240(288)/352x240(288) : 30fps/CH(N), 25fps/CH(P)

        - MJPEG : 2fps

720P or above Input : 640x360/352x240(288)

SD input : 480x240(288)/352x240(288)

        - Max. Framerate

1080P input : 1920x1080 15fps + 640x360 15fps

720P input : 1280x720 30fps + 640x360 30fps

SD input :928x480 30fps + 480x240 30fps

      - The encoder shall support unicast video streaming up to 48 users(4 users / CH) on RTP/RTSP and 32 users on HTTP.

      - The encoder shall support multiple streaming up to 2 profiles per channel.

      - The encoder shall support multicast video streaming.

      - The encoder shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The encoder shall provide Dynamic GOV and Dynamic fps to efficiently manage bit rate of the video stream and reduce storage.

    - Privacy mask with 4 definable detection areas with rectangle zones, and minimum/maximum object size.

    - Interoperability – The encoder shall be ONVIF Profile S & SUNAPI(HTTP API) compliant.

    - The encoder shall possess the following further characteristics:

      - Built-in web server, accessed via non-plugin browsers including Google Chrome 54, MS Edge 38, IE11, Mozilla Firefox 49 (Windows 64bit only), Apple Safari 9 (Mac OS X only).

      - Alarms and notifications

        - alarm notification triggers:

          - Alarm input

          - Motion Detection

          - Video Loss

          - Tempering

          - Network disconnect

        - available notification means upon trigger:

          - File Upload via FTP and E-mail

          - Notification via E-mail

      - Pixel Counter available in the web viewer.

  - ENCODER SOFTWARE

    - The encoder shall have a built in web server which supports non-plugin browsers including Google Chrome, MS Edge, Mozilla Firefox and Apple Safari from a PC or Mac.

    - The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Access recorded data playback and camera configuration menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Recorded data search using date and time range

      - Recorded data search using event type

      - Play a recorded video by event triggering

      - Set resolution

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate and multicast parameters

      - User profile to include password, access level, authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - IP filtering

        - SSL, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

        - Auto IP configuration

      - Video setup

        - Flip / mirror mode

        - Video output type

        - Privacy zone.

      - Event detection setup to configure notification parameters, tamper protection, motion detection and event triggers

      - System function to control reboot, upgrade, check system and event logs and application (SDK) management

      - View profile information

    - Client requirements

      - Recommend Browser : Chrome

      - Acceptable Browser : Chrome, Safari, Firefox, MS Edge(chromium based)

      - Acceptable Operating Systems:	Windows, MAC, Android, iOS, Chrome

      - Verified Environment:

        - Windows 10 : Google chrome version 80 above, Firefox version 72 above, 
                               MS Edge version 83 above

        - Mac 10.13/14 : Safari version 11.0.1 above

- Decoding performance in web viewer depends on CPU/GPU performance of user

  - DETAILED SPECIFICATIONS

    - Video

      - Input Channel			16CH BNC, 1.0 Vp-p / 75Ω composite NTSC /PAL

      - Input Signal & Resolution	CVBS, AHD/CVI/TVI(1 & 2MP) Auto detection

      - Video Out			1 HDMI OUT (16 multi Image, 1920x1080)

    - Operational Functions

      - Motion Detection		Off/On

      - Privacy Masking		Off / On (4ea rectangular zones)

      - De-Interlacing			Support

      - Alarm I/O			16 Input / 4 Output

      - Alarm Triggers			Motion detection, Alarm input, Video loss, Tampering,

Network disconnect

      - Alarm Events			File upload via FTP

Notification via e-mail

      - Audio In/Out			4 Line Input / 1 Line Output (1 NO/COM/NC, 3 NO/COM)

      - OSD				Title

      - Remote Control Interface	1ea RS-485 (Half duplex)

      - RS-485 Protocol		SAMSUNG-T, PELCO-P/D

      - Coaxial Control			CVBS(NTSC/PAL) : Pelco-C (Coaxitron)

AHD : ACP (AHD Coax protocol)

TVI, CVI

    - Network

      - Ethernet			1 RJ-45 10/100/1000 BASE-T

      - Video Compression Format 	H.265, H.264 (MPEG-4 Part 10/AVC) : Main / Baseline / High,

MJPEG

      - Resolution			1920 x 1080, 1280 x 720,

928 x 480(N), 928 x 240(N), 704 x 480(N), 704 x 240(N), 352 x 240(N),

928 x 576(P), 928 x 288(P), 704 x 576(P), 704 x 288(P), 352 x 288(P)

      - Maximum Framerate

        - H.265 / H.264

          - Main/Sub stream

            - 1080P input		1080p/720p/WD1/4CIF/CIF : 15fps/CH(N), 12fps/CH(P)

            - 720P input 		720p/WD1/4CIF/CIF : 30fps/CH(N), 25fps/CH(P)

            - SD input		928x480(576)/704x480(576)/928x240(288)/704x240(288)

/480x240(288)/352x240(288) : 30fps/CH(N), 25fps/CH(P)

        - MJPEG : 2fps

            - 720P or above input
			720p/WD1/4CIF/CIF : 30fps/CH(N), 25fps/CH(P)

            - SD input		480x240(288)/352x240(288)

        - Maximum Framerate

            - 1080P input		1920x1080 15fps + 640x360 15fps

            - 720P input 		1280x720 30fps + 640x360 30fps

            - SD input		928x480 30fps + 480x240 30fps

- Max.Framerate can be dropped to 3fps in the 640x360 stream when using Main/Sub stream simultaneously

      - Bitrate Control Method		H.265 / H.264: CBR or VBR

MJPEG: VBR

      - Streaming Capability		Multiple streaming (Up to 2 profiles per channel)

      - Audio Compression		G.711 u-law

      - IP				IPv4, IPv6

      - Protocol			TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP,

HTTP, HTTPS, SSL/TLS, DHCP, PPPoE, FTP

SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, ONVIF, Bonjour

      - Streaming method		Unicast / Multicast

      - Max. User Access		RTP/RTSP Max. 48 users(4users/CH) at Unicast mode,

HTTP Max. 32 users

      - Interoperability			ONVIF Profile S, SUNAPI(HTTP API)

      - Compatible S/W&Rec.		Smartviewer, NVR(Hanwha Techwin, P/X/Q/L series)

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - User password protection

        - The device shall not provide a manufacture default password. Default password change shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses

        - HTTPS(SSL) login authentication

        - HTTPS(SSL) secured communication

        - Digest login authentication

        - User access log

        - 802.1x authentication (EAP-TLS, EAP-LEAP)

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Encoder backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Power

        - Input Voltage / Current	DC 12V/5A

        - Power Consumption	Max 16W(12V, 1.33A)

    - Mechanical And Environmental

      - Color/Material			Black/Metal

      - Dimensions			370.0x44.0x320.0mm(14.57x1.73x12.6")

      - Weight 				2.32 kg (5.12 lb)

      - Temperature

        - Operating 			-10°C ~ +45°C (14°F ~ +113°F)

-10°C ~ +40°C (14°F ~ +104°F) when installed in RACK

      - Humidity			20~80% RH

END OF SECTION

- EXECUTION

  - INSTALLERS

            - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

            - The network design and configuration shall be verified for compatibility and performance with the camera(s).

            - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

            - All firmware found in products shall be the latest and the most up-to-date provided by the manufacturer, or of a version as specified by the provider of the VMS or NVR.

            - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

            - The contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

            - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

            - Before permanent installation of the system, the contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

            - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the manufacturer.

END OF SECTION

10