Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com)

    - Model: SPS-A100M

    - Alternates: none

  - GENERAL DESCRIPTION

    - The device shall possess the following further characteristics:

      - AI-based sound classification included

        - Classified sound type: Gunshot, Crashing glass, Screaming, Vehicle horn, Tire screech

        - Best sound (audio file)

        - Event metadata support

        - Sound direction with Direction Finding algorithm

      - Five built-in MEMS microphones

      - Built-in speaker (Max. Sound Pressure Level(PoE+) 90dB)

      - Two way audio (Full-duplex mode)

      - Indication LED ring (Initial and post-duration action setting)

      - Milestone/Genetec integration support

    - Interoperability – The device shall be ONVIF Profile S compliant.

  - DETAILED SPECIFICATIONS

    - Video

      - Resolution: 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360

      - Max. Framerate

        - Static image video

        - H.265/H.264: Max. 10fps

        - MJPEG: Max. 5fps

      - Video Out: Micro USB Type C, 1280x720 for setting true north point using Wisenet installation

    - Operational

      - Analytics: AI Sound Classification

        - Classified sound type: Gunshot, Crashing glass, Screaming, Vehicle horn, Tire screech

        - Event metadata: Sound type, Sound direction, Confidence, Timestamp, Best sound (audio file), dB(±10dB)

        - Sound direction (Celling mounting: ±15 degrees)

      - Alarm I/O: 2 configurable I/O ports

      - Alarm Triggers: Analytics, Network disconnect, Alarm input, App event (Sound classification), Time schedule, MQTT subscription, Case tampering

      - Alarm Events: when alarm trigger occurred

        - File upload(audio): e-mail/FTP

        - Notification: e-mail

        - Alarm output

        - Handover: PTZ preset, send message by HTTP/HTTPS/TCP

        - Audio clip playback (6MB, WAVWAV in mono/stereo from 64 kbps to 320 kbps, sampling rate from 16 kHz up to 48 kHz)

        - MQTT: publication

        - LED

        - Speaker

      - Audio Streaming: Two-way/one-way selectable, Full duplex, Echo cancellation and noise reduction

      - Audio In: Five built-in MEMS MICs

      - Audio Out

        - Built-in speaker

        - Max. Sound Pressure Level 90dB at 1m(3.28ft)

        - Frequency Range: 144Hz~20kHz

      - Light Type: LED Ring(Green, Blue, Red, Pink, Sky blue, Purple), Initial and Post-duration action setting

      - Webpage Language: English, Korean, Simplified Chinese, Traditional Chinese, French, Italian, Spanish, German, Japanese, Russian, Portuguese, Czech, Polish, Turkish, Dutch, Hungarian, Greek

      - Memory: 4096MB RAM, 512MB Flash

    - Network

      - Ethernet: RJ-45 connector(10/100 Base-T

      - Audio Compression

        - G.711 u-law /G.726 Selectable

        - G.726(ADPCM) 8KHz, G.711 8KHz

        - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

        - AAC-LC: 48Kbps at 16KHz

      - Streaming: Unicast(6 users) / Multicast, Multiple streaming(Up to 6 profiles)

      - Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, SRTP (TCP, UDP Unicast), MQTT

      - Application Programming Interface

        - ONVIF Profile S

        - SUNAPI(HTTP API)

        - Wisenet open platform: AI Sound Classification pack included

      - Security Feature

        - OS / Firmware Protect: Encrypted Firmware, Secure boot, Signed Firmware

        - User authentication: Digest Authentication, Prevent brute-force attack

        - Network authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

        - Secure Communication: HTTPS, WSS (WebSocket Secure)

        - Access Control: IP-based access control

        - Data Protect: Encryption Credentials, Encrypt compress for live recording file export

        - Audit: User Access / System / Event Log management

        - Device ID: Device certificate (Hanwha Techwin Root CA)

        - Secure Storage: TPM

        - Security Certificate: TPM with FIPS 140-3 level 3

    - Environment and Electrical

      - Operating Temperature & Humidity: -20°C ~ +55°C (-4°F ~ +122°F) / Less than 90% RH

      - Storage Temperature & Humidity: -50°C ~ +60°C(-58°F ~ +140°F) / Less than 90% RH

      - Certification: IEC/EN 63000

      - Input Voltage: PoE+(IEEE802.3at, Class4)

      - Power Consumption: Max 18.5W, typical 8.2W

    - Mechanical

      - Color/Material: White / Plastic, Aluminum

      - RAL Code: RAL9003

      - Dimensions: 215.1(W)x135.1(D)x52.8(H)mm (8.47" x 5.32" x 2.08")

      - Weight: 973g(2.15 lb)

      - Compatible Conduit Hole / Gangbox

        - None / single, double, 4" octagon, 4" square

        - Sold separately conduit hole accessory: SBP-060BA

      - Hanging Mount(Dome): SBP-215HMWB

      - Conduit Adaptor: SBP-060BA

    - Certifications

      - EMC

        - FCC 47 CFR 15 Subpart B Class A

        - ICES-3(A)/NMB-3(A)

        - CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

        - VCCI CISPR 32 Class A

        - RCM AS/NZS CISPR 32 Class A

      - Safety

        - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

        - IEC/EN 62471

END OF SECTION

8