SRD-1654D 16 CHANNEL DIGITAL VIDEO RECORDER

TECHNICAL SPECIFICATIONS						SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with an explicit manufacturer warranty.

2.02	SRD-1654D 16 CHANNEL H.264 REAL TIME DIGITAL VIDEO RECORDER

- The digital video recorder (DVR) shall support 16 channels and provide recording rates of 480 frames per second (fps) on NTSC systems and 400 fps on PAL systems. The DVR shall display high-resolution images at a maximum of 928 x 480 pixels on NTSC systems and 928 x 576 on PAL systems.

- The DVR shall offer combined VGA/HDMI video output for main display; H.264 video compression and G.711 audio compression; up to 4 audio inputs and 1 audio output, and PTZ control using RS-485.

- The DVR shall support network monitoring and integration using Samsung central monitoring system.

- All operation and configuration menus shall be provided in several languages, including English, French, German, Spanish, Italian, Russian, Polish, Czech, Turkish, Netherlands, Portuguese, Swedish, Danish, Rumania, Serbia, Croatia, Hungary, Greek, Finnish, Norwegian, Korean, Japanese, Chinese (Traditional), Thai, Taiwanese.

2.03	DISPLAY

| A. Video |  |  |
| --- | --- | --- |
|  | 1. Inputs | 16 Composite video 0.5-1 Vpp, 75 ohm automatic termination |
|  | 2. Resolution | NTSC : 928 x 480, 704 x 480 PAL : 928 x 576, 704 x 576 |
| B. Live |  |  |
|  | 1. Frame Rate | NTSC : 480fps PAL : 400fps |
|  | 2. Resolution | NTSC : 928 x 480, 704 x 480 PAL : 928 x 576, 704 x 576 |
|  | 3. Multi Screen Display | 1 , 4 , 7 , 9 , 13 , 16 , 16A , Sequence , PIP |

2.04	PERFORMANCE

| A. Operating System |  | Embedded Linux |
| --- | --- | --- |
| B. Recording |  |  |
|  | 1. Compression | H.264 |
|  | 2. Record Rate | NTSC : Up to 120fps@928 x 480 / PAL : Up to 100fps@928 x 576 NTSC : Up to 120fps@704 x 480 / PAL : Up to 100fps@704 x 576 NTSC : Up to 240fps@704 x 240 / PAL : Up to 200fps@704 x 288 NTSC : Up to 480fps@352 x 240 / PAL : Up to 400fps@352 x 288 |
|  | 3. Mode | NTSC : Manual, Schedule (Continuous / Event), Event (Pre / Post), Time lapse (1~30 fps) PAL : Manual, Schedule (Continuous / Event), Event (Pre / Post), Time lapse (1 ~ 25fps) |
|  | 4. Event | Video loss, Motion (Level 1 ~ 10), Alarm |
|  | 5. Overwrite Modes | Selectable (Stop / Continuous) |
|  | 6. Pre-alarm | Up to 30sec (5, 10, 20, 30sec) |
|  | 7. Post-alarm | Up to 6hour (5,10, 20, 30sec, 1, 3, 5, 10, 20min, 1, 2, 3, 4, 5, 6hour) |
| C. Search & Playback |  |  |
|  | 1. Search Mode | Date/Time, Event, Back up, POS, Motion (*All search include preview function) |
|  | 2. Playback Function | Fast forward / backward (2x, 4x, 8x, 16x, 32x, 64x) Slow forward / backward (1/2x, 1/4x, 1/8x) Step forward / backward (2x, 4x, 8x, 16x, 32x, 64x) *In Fast and Step backward, backward play with i-frame only |
| D. Network |  |  |
|  | 1. Transmission Speed | 4CIF / 2CIF / CIF (NTSC : 120 / 240 / 480fps, PAL : 100 / 200 / 400fps) |
|  | 2. Bandwidth | Up to 32Mbps |
|  | 3. Bandwidth Control | Selectable |
|  | 4. Stream | H.264 (4CIF / 2CIF / CIF selectable) |
|  | 5. Max. Remote Users | Search 3 / Live unicast 10 / Live multicast 20 |
|  | 6. Protocol Support | TCP/IP, DHCP, PPPoE, SMTP, NTP, HTTP, DDNS, RTP, RTSP, SNMP |
|  | 7. Monitoring | SmartViewer, Webviewer, SSM (CMS) |
| E. Smartphone |  |  |
|  | 1. Platform | Android, iOS |
|  | 2. Protocol Support | RTP, RTSP, HTTP, CGI |
|  | 3. Monitoring | Live 2, Search 1 |
| F. Storage |  |  |
|  | 1. Internal HDD | Up to 4 SATA HDDs |
|  | 2. DVD Writer(Back up) | Yes |
|  | 3. USB (Back up) | 2 USB Ports |
|  | 4. File Format (Back up) | BU (DVR Plater), EXE(Include player), AVI |
| G. Security |  |  |
|  | 1. Password Protection | 1 Admin, 10 Group, 10 User per 1 group |
|  | 2. Data Authentication | Watermark |

2.05 	INTERFACE

| A. Monitors |  |  |
| --- | --- | --- |
|  | 1. VGA | 1 VGA (1280 x 720, 1280 x 1024, 1920 x 1080) |
|  | 2. HDMI | 1 HDMI (1280 x 720, 1280 x 1024, 1920 x 1080) |
|  | 3. Spot Composite | Digital spot (2CH) *Incldued OSD on screen, multi mode support (1CH) |
| B. Audio |  |  |
|  | 1. Inputs | 4CH line in |
|  | 2. Outputs | 1CH line out |
|  | 3. Compression | G.711 |
|  | 4. Sampling Rate | 8KHz |
| C. Alarm |  |  |
|  | 1. Inputs | Terminal 16 inputs (NO/NC) |
|  | 2. Outputs | Terminal 4 relay outputs (NO/NC) |
|  | 3. Rating | 30V DC/2A, 250V AC/0.25A |
|  | 4. Remote Notification | Notification via e-mail |
| D. Connections |  |  |
|  | 1. Ethernet | 1 RJ-45 10/100/1000 Base-T |
|  | 2. Serial Interface | RS-232/RS-485 for PTZ, Samsung system keyboard |
|  | 3. USB | USB 2.0, 2 ports |
|  | 4. Application Support | Mouse, Remote controller |
|  | 5. RS-485 Protocol | Samsung-T/E, Pelco-D/P, Panasonic, Phillips, AD, DIAMOND, ERNA, KALATEL, VCL TP, VICON, ELMO, GE |

2.06	GENERAL

| A. Electrical |  |  |
| --- | --- | --- |
|  | 1. Input Voltage/Current | 100 ~ 240V AC ±10%, 50/60Hz, Auto ranging |
|  | 2. Power Consumption | Max. 60W (With 1x HDD) |
| B. Environmental |  |  |
|  | 1. Operating Temperature | +0°C ~ +40°C (+32°F ~ +104°F) |
|  | 2. Operating Humidity | 20% ~ 85% RH |
| C. Mechanical |  |  |
|  | 1. Dimensions | W440.0 x H88.0 x D246.8mm (17.32" x 3.46" x 9.72") |
|  | 2. Weight(with HDDs) | Approx. 5.2Kg (11.46 lb) |
|  | 3. Rack Mount Kit | 2ea |
| B. Audio |  | English, French, German, Spanish, Italian, Russian, Polish, Czech, Turkish, Netherlands, Portuguese, Swedish, Danish, Rumania, Serbia, Croatia,Hungary, Greek, Finnish, Norwegian, Korean, Japanese, Chinese (Traditional), Thai, Taiwanese (25 Language) |

2.07	CERTIFICATIONS

- CE mark

- FCC mark

2.08	WARRANTY

3 years, parts and labor.