SRD-870D 8-CHANNEL DIGITAL VIDEO RECORDER

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 REMOTE VIDEO SURVEILLANCE SENSORS AND DEVICES

PART 2 – PRODUCTS

2.01	GENERAL

- All equipment and materials used must be standard components regularly manufactured and used in the manufacturer's system.

- All systems and components must have been thoroughly tested and proven to be in effective use.

- All systems and components must be supplied with an explicit manufacturer's warranty.

2.02	SRD-870D 8-CHANNEL DIGITAL VIDEO RECORDER

- The digital video recorder (DVR) shall support 8 camera channels, 4CIF recording, and provide recording speeds of 240 images per second (ips) of real-time digital video on NTSC systems and 200 fps on PAL systems. The DVR shall display high-resolution images with a maximum resolution of 704 x 480 pixels on NTSC systems and 704 x 576 on PAL systems.

- The DVR must be equipped with a maximum of 5 internal hard drives.

- The DVR should offer a combined BNC and VGA/HDMI video output for the main monitor; H.264 video compression and ADPCM audio compression; and up to 8 audio inputs and 1 audio output, as well as PTZ control via coaxial cable.

- The DVR is expected to have network and integration capabilities using Samsung's Smart Viewer central monitoring system.

E.	All operating and setup menus must be provided in multiple languages, including: English, Spanish, French, German, Italian, Portuguese, Polish, Russian, Turkish, Japanese, Korean, Czech, Swedish, Danish, Chinese, and Dutch.

2.03	VIEW

A.	Video

1. Tickets	8 x 0.5-1 Vpp composite video inputs, 75 ohm auto-termination

2. Resolution	NTSC: 704 x 480, PAL: 704 x 576

B.	Live

1. Frame Rate	NTSC: 240ips, PAL: 200ips

2. Resolution	NTSC: 704 x 480, PAL: 704 x 576

3. Multi-Screen Display	1, 4, 6, 8, 9, 13, PIP (Picture In Picture)

2.04	PERFORMANCE

A. Operating System	Linux, Built-in

B. Recording

1. Compression	H.264

2. Write Speed	NTSC: Up to 240ips@704 x 480; Up to 240ips@704 x 240; Up to 240ips@352 x 240
PAL: up to 25ips@704 x 576; up to 200ips@704 x 288; up to 200ips@352 x 288

3. Mode	NTSC: Manual, Scheduled (Continuous, Event), Event (Pre, Post), Time Lapse (0.9, 1.9, 3.8, 7.5, 15ips)
PAL: Manual, Programmed (Continuouso, Event), Event (Pre, Post), Time lapse (0.8, 1.6, 3.1, 6.2, 12.5ips)

4. Recording Overlay Mode	Ongoing

5. Pre-alarm	Up to 30 sec. (5, 10, 20, 30 sec.)

6. Post-alarm	Up to 6 hours (5, 10, 20, 30 sec, 1, 3, 5, 10, 20 min, 
1, 2, 3, 4, 5, 6 hours)

C. Search and Reproduction

1. Search Modes	Date/Time, Event, Backup, Position, Motion (All search modes include preview function)

2. Playback Function	Fast forward/backward, Slow forward/backward, Step-by-step forward/backward

D. Network

1. Transmission Speed	Up to 15 Mbps, Unlimited 4M, 2, 1.5, 1 Mbps, 800, 600, 500, 400, 300, 200, 100, 50 kbps

2. Bandwidth control	Automatic

3. Multiple transmission flow	H.264 (4CIF, 2CIF, CIF)

4. Maximum Remote Users	Quest 3, Unicast Live 10, Multicast Live 20

5. Supported Protocols	TCP/IP, DHCP, PPPoE, SMTP, NTP, HTTP, DDNS, RTP, RTSP

6. Monitoring	Net-i viewer, Smart viewer, Web viewer

E. Storage (Backup)

1. Internal Hard Drive	Up to 5 SATA hard drives

2. External Hard Drive	Up to 2 expansion bays (up to 4 Hard Drives each)

3. DVD Burner	Yes

4.USB 	3 USB ports

5. File Format	AVI, MDB (Playback DVR), EXE (including player)

F. Security

1. Password Protection	1 Admin, 10 Groups, 20 Users per Group. The user can have their authority assigned through a certain role

2. Data Authentication	Watermark

2.05 	INTERFACE

A. Monitors

1. VGA	Analog, 800 x 600 RGB resolution, 1024 x 768, 1280 x 1024

2. HDMI	480p, 720p, 1090p resolution

3. Main Compound	BNC, 1 Vp-p, 75 ohm

4. Auxiliary Compound	1 BNC, 1 Vp-p, 75 ohm, Supports multi-screen display and camera title

5. Circuit Outputs	8

B. Audio

1. Tickets	8 Line inputs (4 built-in channels, option: audio extension cable)

2. Exit	1 line

3. Compression	ADPCM

4. Sampling Rate	16 KHz

C. Alarm

1. Tickets	8-input terminal, NO/NC

2. Outputs	4-Output Terminal with Relay, NO/NC

3. Remote Notification	Notification via email

D. Connections

1. Ethernet	1 RJ-45 10/100/1000 Base-T

2. Interface serial	RS-232

3. PTZ Control Interface	RS-485 via cabo coaxial (Samsung, CCVC, Coaxtron)

4. PTZ Keyboard	RS-485

5. USB	USB 2.0, 3 ports

6. eSATA	2 external eSATA holders

7. App Support	Mouse, Remote Control

E. Supported Protocols	Samsung-E, Samsung-T, Pelco-D, Pelco-P, Panasonic, Philips, AD, Diamond, Ema, Kalatel, VCL TP, Vicon

2.06	GENERAL

A. Electrical Specifications

1. Input Voltage	100~240V AC  10%, 50/60 Hz, (auto-change)

2. Consumption	55W max (with 1 Hard Drive), 65W max (with 2 Hard Drives)

B. Environmental Specifications

1. Operating Temperature	+0°C ~ +40°C

2. Operating Humidity	Relative Humidity 20%~85%

C. Dimensions	440 x 88 x 430 mm

1. Mechanical Weight

(2 Hard Drives) 	Approximately 7.5 kg

2. Rack Mount Kit	2

D. Languages	English, Korean, Spanish, French, German, Italian, Portuguese, Polish, Russian, Turkish, Japanese, Czech, Swedish, Danish, Chinese and Dutch.

2.07	CERTIFICATIONS

- UL Certified

- UL certified to Canadian safety standards

- CE Mark

- FCC Brand

2.08	WARRANTY

- 3 years, parts and labor.