Hanwha Techwin is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/

8K NETWORK BOX CAMERA

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00	Data Communications

# 28 23 13	Video Surveillance Control and Management Systems

# 28 23 16	Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19	Digital Video Recorders and Analog Recording Devices

# 28 23 23	Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

# 28 2x 00	Video Management System

8K NETWORK BOX CAMERA

- GENERAL

  - SUMMARY

    - Section includes a 8K IP box camera

    - Product - A 8K resolution box type IP camera with multi-streaming (H.265, H.264 and MJPEG) capability

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC		Auto Gain Control

      - AES		Advanced Encryption Standard

      - API		Application Programming Interface

      - ARP		Address Resolution Protocol

      - AWB		Auto White Balance

      - BLC		Back light compensation

      - CBR		Constant Bit Rate

      - CVBS		Composite Video Blanking Sync

      - DHCP		Dynamic Host Configuration Protocol

      - DNR		Digital Noise Reduction

      - DNS		Domain Name Server

      - DDNS		Dynamic Domain Name Server

      - DSCP		Differentiated Services Code Point

      - fps		frames per second

      - FTP		File Transfer Protocol

      - GOV		Group of Video

      - GUI		Graphical User Interface

      - HD		High Definition

      - HTTP		Hypertext Transfer Protocol

      - HTTPS		Secure HTTP

      - ICMP		Internet Control Message Protocol

      - IGMP		Internet Group Management Protocol

      - IP		Internet Protocol

      - IR		Infrared

      - JPEG		Joint Photographic Experts Group

      - LAN 		Local Area Network

      - LED		Light Emitting Diode

      - LDC		Lens Distortion Correction

      - LPR		License Plate Recognition

      - MJPEG		Motion JPEG

      - MP		Megapixel

      - MPEG		Moving Pictures Experts Group

      - NAS		Network Attached Storage

      - NTP		Network Time Protocol

      - NVR		Network Video Recorder

      - PIM-SM	Protocol Independent Multicast-Sparse Mode

      - PoE		Power over Ethernet

      - PPPoE		Point to Point Protocol over Ethernet

      - QoS		Quality of Service

      - RTP		Real-Time Transport Protocol

      - RTCP		Real-Time Control Protocol

      - RTSP		Real-Time Streaming Protocol

      - SDK		Software Development Kit

      - SMTP		Simple Mail Transfer Protocol

      - SNMP		Simple Network Management Protocol

      - SSDR		Super Smart Dynamic Range

      - SSNR		Super Smart Noise Reduction

      - SSL		Secure Sockets Layer

      - TCP		Transmission Control Protocol

      - UDP		User Datagram Protocol

      - UPnP		Universal Plug and Play

      - VBR		Variable Bit Rate

      - VMS		Video Management System

      - WDR		Wide Dynamic Range

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

        - IPv4 IP addressing version 4

        - IPv6 IP addressing version 6

        - QoS Quality of Service

      - Video

        - ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S / G

      - Emission

        - FCC Part 15 Subpart B Class A

        - CE EN 55032:2015[Class A]

        - CE EN 61000-3-2:2014

        - CE EN 61000-3-3:2013

      - Immunity - CE

        - EN 50130-4:2011+A1:2014

        - EN 61000-4-2:2009

        - EN 61000-4-3:2006/A2:2010

        - EN 61000-4-4:2012

        - EN 61000-4-5:2014

        - EN 61000-4-6:2014

        - EN 61000-4-11:2004

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

    - Definitions

      - GOV (Group of Video object planes) - A set of video frames for H.264 and H.265 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of two kinds of frames in video surveillance setup: I-Frame and P-Frame.

      - Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Multi-exposure wide dynamic range - Operation which automatically adjusts shutter speed to provide a wide range between dark and light areas visible at the same time, preventing backlighting issues. Long exposure is used for dark areas and a short exposure is used in bright areas.

      - Dynamic fps – Dynamic assignment of fps (frames per seconds) based on the movement of object(s) in the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Smart Codec – Codec that controls quantization parameter, fps, and GOV length in H.265 and H.264 to efficiently manage bitrate of the video stream and reduce the storage required. Smart Codec may be referred to as WiseStream in this document.

      - DORI (Detect, Observe, Recognize, Identify) – A standard system (EN-62676-4) for defining the ability of a camera to distinguish persons or objects within a covered area.

        - Detect : 25PPM / 8PPF

        - Observe : 63PPM / 19PPF

        - Recognize : 125PPM / 38PPF

        - Identify : 250PPM / 76PPF

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment specified in section 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide at least a year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded camera video analytics free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://www.hanwha-security.com/

    - Model		TNB-9000

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264 – Max.15fps@8K, 20fps@24MP, 30fps@15MP, 60fps@4K

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - 7680x4320, 7360x4128, 6016x3384, 6016x4008, 5472x3648, 4768x3184,

4608x2592, 3840x2160, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768,

800x600, 800x448, 720x576, 720x480, 640x480, 640x360

      - The camera shall support multicast and unicast video streaming up to 10 users.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide smart codec (WiseStream Ⅱ, Dynamic GOV, and Dynamic fps) to efficiently manage bit rate of the video stream and reduce storage while producing video quality that is visually equal to the one without smart codec.

    - Camera – The camera device shall have the following physical and performance properties:

      - Automatic, manual, scheduled day and night operation with infrared cut filter. Images are available in color or black and white.

        - Low light level operation to 0.015Lux (F1.4) in color mode and 0.0015Lux (F1.4) in black and white mode.

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - Configurable 6 privacy masking regions utilizing rectangle

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with eight definable detection areas with eight point polygonal zones, and minimum/maximum object size.

      - Motion detection hand-over to PTZ cameras. The camera shall be able to call a preset of PTZ camera when motion event is triggered.

      - Detection of logical events of specified conditions from the camera’s video

        - Tampering

        - Loitering

        - Directional detection

        - Defocus detection

        - (Dis)Appear,

        - Audio detection

        - Motion detection

        - Classified object type : person/face/vehicle/license plate with attributes, Best shot per object

        - Analytics events based on AI engine : object detection, directional detection, Enter/Exit, loitering, virtual line

      - Detection and classification of the following sound.

        - Scream

        - Gunshot

        - Explosion

        - Crashing glass

    - Interoperability – The camera shall be ONVIF Profile S,G and T compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard HTML5 browsers including Internet Explorer, Firefox, Chrome & Safari

      - The camera shall provide streaming to multiple smart phones with DDNS provided freely from the manufacturer. In addition, the application shall be available for both iOS and Android, free of charge with search keyword, ‘Wisenet Mobile’.

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - Alarm input

          - Analytics

          - Network disconnect

        - available notification means upon trigger:

          - File Upload via FTP and E-mail

          - Notification via E-mail

          - Local storage (SD / SDHC / SDXC) or NAS recording at event triggers

          - Alarm output

          - Handover

      - Pixel Counter available in the web viewer.

      - HPoE capable

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunderstroke, communication interruption.

  - CAMERA SOFTWARE

    - The camera shall have a built in web server which supports HTML5 browser-based configuration using Internet Explorer, MS Edge, Google Chrome, Mozilla Firefox, and Apple Safari from a PC or Mac.

    - The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Manual recording to SD or NAS

      - Access recorded data playback and camera configuration menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Recorded data search using date and time range

      - Recorded data search using event type

      - Play a recorded video by event triggering

      - Set resolution

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, and crop encoding area

      - User profile to include password, access level, authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - SSL, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

        - Auto configuration

      - Video setup to include flip and mirror mode, video type and privacy zone

      - Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, sharpness, contrast, color level and lens distortion correction.

      - Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection and event triggers

      - System function to include reboot, upgrade, check system and event logs and application (SDK) management

      - View profile information

    - Client requirements

      - Recommend Browser : Chrome

      - Acceptable Browser : Chrome, Safari, Firefox, MS Edge(chromium based)

      - Acceptable Operating Systems:	Windows, MAC, Android, iOS, Chrome

      - Verified Environment:

        - Windows 10 : Google chrome version 80 above, Firefox version 72 above, 
                               MS Edge version 83 above

        - Mac 10.13/14 : Safari version 11.0.1 above

- Decoding performance in web viewer depends on CPU/GPU performance of user

  - DETAILED SPECIFICATIONS

    - Video

      - Imager

        - Sensor			43.3mm Full-frame CMOS

          - Effective Pixels		7680(H) x 4320(V)

        - Minimum Illumination

          - Color Mode 		0.015Lux (1/30sec, F1.4)

          - Black & White Mode 	0.0015Lux (1/30sec, F1.4)

        - Video Out (Installation)	HDMI: 1080p@30fps

        - The following features with control settings shall be available

          - Camera Title 				Off / On (Displayed up to 75 characters)

          - Day/Night Setting			Auto (ICR)

          - Backlight Compensation (BLC)		BLC / DWDR

          - Digital Noise Reduction (DNR)		SSNR5

          - Motion Detection			Off / On (8ea, 8 points polygonal zones in 8k

resolution only)

          - Privacy Masking			Off / On (6ea zones of rectangle)

- Color: Gray / Green / Red / Blue / Black / White

          - Gain Control				Low / Middle / High

          - White Balance 				ATW / AWC / Manual / Indoor / Outdoor

          - Electronic Shutter Speed

            - Settings				Min / Max / Anti-flicker (1/5 ~ 1/12,000sec)

          - Analytics

            - Classified object type		Person/Face/Vehicle/License plate with

attributes, BestShot per object

            - Analytics events based on AI            object detection, Directional detection, Enter/Exit,

Loitering, Virtual line

            - Analytics events 			Defocus detection, Motion detection,

Appear/Disappear, Tampering, Audio detection, Sound classification(*AI function will be upgraded later)

          - Serial Interface				RS-485(Samsung-T, Pelco-D/P)

          - Alarm I/O				Input 1ea / Output 1ea

          - Alarm Triggers				Alarm Input

Video & Audio Analytics,

Network Disconnection

          - Alarm Events				File Upload via FTP and E-mail,

Notification via E-mail,

Local storage (SD / SDHC / SDXC) or

NAS recording at event triggers,

Alarm output

Handover

          - Audio In				Mic in/Line in

(2.5VDC(4mA), Input impedance: 2K Ohm)

          - Audio Out				Line out(Max. output level 1Vrms)

        - Lens

          - Field of View		Canon 24mm f1.4L, Auto-Iris  (EF 24mm f/1.4L II USM) :

Horizontal field of view : 8K 62.1°

- DORI Distance

Detect		255.3m (837.7ft)

Observe	102.1m (335.1ft)

Recognize	51.1m (167.5ft)

Identify		25.5m (83.8ft)

Canon 35mm f1.4L, Auto-Iris  (EF 35mm f/1.4L II USM) : Horizontal field of view:8K 44.5°

- DORI Distance

Detect		372.3m (1221.6ft)

Observe	148.9m (488.6ft)

Recognize	74.5m (244.3ft)

Identify		37.2m (122.2ft)

Canon 50mm f1.4, Auto-Iris   (EF 50mm f/1.4 USM) :

Horizontal field of view: 8K 31.6°

- DORI Distance

Detect		531.9m (1745.1ft)

Observe	212.8m (698.1ft)

Recognize	106.4m (349.0ft)

Identify		53.2m (174.5ft)

Canon 85mm f1.2L, Auto-Iris  (EF 85mm f/1.2L II USM) : Horizontal field of view: 8K 19.2°

- DORI Distance

Detect		904.3m (2966.7ft)

Observe	361.7m (1186.7ft)

Recognize	180.9m (593.3ft)

Identify		90.4m (296.7ft)

Canon 100mm f2.0, Auto-Iris  (EF 100mm f/2 USM) :

Horizontal field of view: 8K 16.1°

- DORI Distance

Detect		1063.8m (3490.3ft)

Observe	425.5m (1396.1ft)

Recognize	212.8m (698.1ft)

Identify		106.4m (349.0ft)

Canon 70-200mm f2.8L, Auto-Iris, Vari Focal (EF 70?200mm f/2.8L USM) : Horizontal field of view: 8K 23.2° ~ 8.3°

* When using Canon 70-200mm f2.8L, Auto-Iris, Vari Focal (EF 70?200mm f/2.8L USM) mount lens, housing accessory components must be used.

- DORI Distance

Detect		744.7m-2127.7m (2443.2ft – 6980.5ft)

Observe	297.9m-851.1m (977.3ft – 2792.2ft)

Recognize	148.9m-425.5m (488.6ft – 1396.1ft)

Identify		74.5m-212.8m (244.3ft – 698.1ft)

          - Focus Control		Auto focus

          - Lens Type		Canon EF mount Lens

          - Mount Type		Canon EF mount

          - Optional Lens		Canon 24mm f1.4L, Auto-Iris  (EF 24mm f/1.4L II USM)

Canon 35mm f1.4L, Auto-Iris  (EF 35mm f/1.4L II USM)

Canon 50mm f1.4, Auto-Iris   (EF 50mm f/1.4 USM)

Canon 85mm f1.2L, Auto-Iris  (EF 85mm f/1.2L II USM)

Canon 100mm f2.0, Auto-Iris  (EF 100mm f/2 USM)

Canon 70-200mm f2.8L, Auto-Iris, Vari Focal (EF 70?200mm f/2.8L USM)

      - Video Streams

        - The camera shall be able to produce 10 video profiles, each of which may have the following properties

          - Encoding Type

            - H.265

            - H.264

            - MJPEG

          - Resolution		7680x4320, 7360x4128, 6016x3384, 6016x4008, 5472x3648,

4768x3184, 4608x2592, 3840x2160, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360

          - Maximum Framerate

            - H.265/H.264: 8K @ Max. 15fps (Mode 0) : Available in Dec. 2019

            - H.265/H.264: 24MP @ Max. 20fps (Mode 1) : Available in Mar. 2020

            - H.265/H.264: 15MP @ Max. 30fps (Mode 2) : Available in Mar. 2020

            - H.265/H.264: 4K @ Max. 60fps (Mode 3) : Available in Mar. 2020

          - Smart Codec			WiseStreamⅡ, Dynamic GOV, Dynamic fps

          - Bitrate Control Method		H.265 / H.264: CBR or VBR

MJPEG: VBR

      - Number of Multi-Streaming Profiles	3 maximum

      - Simultaneous Users (Total)		10 maximum (Unicast)

      - Storage and Recording

        - The camera shall have an onboard SD card storage

          - Card Type	Micro SD/SDHC/SDXC

          - Capacity	256GB (1slot)

          - Video or images content on the card shall have the ability to be downloaded to a selected destination

      - Audio Compression			G.711 u-law /G.726 Selectable

G.726(ADPCM) 8KHz, G.711 8KHz

G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

AAC-LC: 48Kbps at 16KHz

      - Interoperability – Video streams shall be capable of supporting ONVIF Profile S / G / T

      - Still Image – The camera shall support image screenshot and export

    - Network

      - Connectivity – Metal Shielded RJ-45(10/100/1000 BASE-T), SFP slot(100/1000Mbps)

      - Protocol

IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP,RTSP, NTP, HTTP, HTTPS,

SSL/TLS, DHCP, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS,

DDNS, QoS, UPnP, Bonjour, LLDP

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - User password protection

        - The device shall not provide a manufacture default password. Default password change shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses

        - HTTPS(SSL) login authentication

        - HTTPS(SSL) secured communication

        - Digest login authentication

        - User access log

        - 802.1x authentication

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Power

        - Input Voltage / Current	HPoE(IEEE802.3bt, Class5), 12VDC

        - Power Consumption	PoE: Max 30W, typical 20W, 12VDC: Max 26W, typical 18W

    - Mechanical And Environmental

      - Material 			Black, Aluminum

      - Dimensions (W x H x D)		120(W)x118.1(H)x179(D)mm

      - Weight 				2.1Kg(4.55 lb)

      - Temperature

        - Operating 			0°C ~ +45°C(32°F ~ +122°F) / Less than 90% RH

        - Storage / Humidity		-40°C ~ +65°C(-40°F ~ +149°F) / Less than 90% RH

      - Certification			EMC

EN 50130-4, EN 55032 Class A, EN 61000-6-3, EN 61000-3-2,

EN 61000-3-3, EN 61000-6-1, FCC Part 15 Subpart B Class A, IC ICES-003 Class A, IC ICES-003 Class A,

Safety

UL 60950-1

END OF SECTION

- EXECUTION

  - INSTALLERS

            - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

            - The network design and configuration shall be verified for compatibility and performance with the camera(s).

            - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

            - All firmware found in products shall be the latest and the most up-to-date provided by the manufacturer, or of a version as specified by the provider of the VMS or NVR.

            - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

            - The contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

            - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

            - Before permanent installation of the system, the contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

            - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the manufacturer.

END OF SECTION

10