Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com)

Model: TNP-A6550RW

Alternates:	TNP-A9430RW, TNP-A7430RW

GENERAL DESCRIPTION

Video Compression and Transmission – The 8MP camera shall have the following properties relating to the video signals it produces.

H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

H.265 / H.264: up to 60 fps at 60 Hz, up to 50 fps at 50 Hz

MJPEG: up to 30 fps (at 4MP resolution, the maximum frame rate is 5 fps)

The 8MP camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

Resolution selections

3840x2160, 2592x1944, 2592x1464, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

Simultaneous unicast access by up to 20 users

Multicast or unicast capable

Dynamic DNS (DDNS) support

WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream

Camera – The 8MP camera device shall have the following physical and performance properties:

The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range.

IP66/IP67, IK10, NEMA 4X, NEMA TS 2 (2.2.7, 2.2.8, 2.2.9) rated for protection against impacts.

Auto day/night operation with removable IR cut filter

Low light level operation to 0.02 Lux at F1.6, 1/30sec, 30 IRE in color mode and 0.002 Lux at F1.6 with IR LED off in black and white mode, achieving 0 Lux performance when IR LED is activated

2D and 3D digital noise reduction

Integral IR illumination, providing effective visibility of 500m(1,640.42ft) at 0 Lux when activated in black and white mode

32 privacy masking regions with quadrangle

Supports dynamic privacy masking

The camera shall provide video display on smartphone (iPhone, Android) to adjust viewing angle, rotation and focus.

Intelligence and Analytics – The 8MP camera shall have a suite of integral intelligent operations and analytic functions to include:

Motion detection with 8 definable detection areas with 8-point polygonal zones, and minimum / maximum object size.

Classified object analytics

Person / Face / Vehicle / License plate

Attributes

Person: Upper/Lower clothes color, Gender, Bag

Vehicle: Type(car/bus/truck/motorcycle/bicycle), Color

Face: Age, Gender, Face mask, Glasses

Support BestShot per object

Analytics events

Based on AI engine

Object detection, Virtual line (Crossing / Direction), Motion detection, Slip & Fall detection, Virtual area (Loitering / Intrusion / Enter / Exit / Appear / Disappear)

Normal

Tampering, Audio detection, Shock detection

Interoperability – The 8MP camera shall be ONVIF Profile S and G, T, M compliant.

The 8MP camera shall possess the following further characteristics:

Built-in web server, accessed via standard browsers including Internet Explorer, Firefox, Chrome & Safari

Local recording via microSD card (supporting SD, SDHC, and SDXC formats) and network recording via NAS, with configurable pre-alarm and post-alarm recording intervals

Bi-directional audio

Alarms and notifications

alarm notification triggers:

Analytics

Network disconnect

Alarm input with NW I/O Box

available notification means upon trigger:

File upload via FTP and email

Notification via email

Local storage (SD / SDHC / SDXC) or NAS recording at event triggers

Alarm output with NW I/O Box

PTZ preset

Handover

Pixel Counter available in the plug-in Web Viewer

This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

CAMERA SOFTWARE

The 8MP camera shall have a built-in web server which supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari, for which web viewer plug-ins are available, from a PC or Mac.

The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

Live view window size

Resolution setting

Image (snapshot) capture

Manual recording to SD or NAS

Audio and microphone control

Access Playback and Setup menus

Digital PTZ

The web viewer shall provide an auto-tracking feature which tracks a specific object using object detection analytics or targeted object tracking.

The web viewer shall provide a playback screen which provides access to the following functions:

Search date and time range

Search event type

Play an event video

Set resolution

Play audio if present

Generate a backup copy of saved video data

The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, crop encoding area

User profile to include password, access level, authentication

Date and time

Network settings and IP version

DDNS

SSL, including certificate management

802.1x authentication

Quality of Service settings

SNMP to include version selection and settings

Auto configuration

Video setup to include flip and mirror mode, hallway view mode, video type, privacy zone

Audio setup to include source, audio codec type, gain, and bit rate

Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, IR illumination, sharpness, contrast, and color level.

Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection, event triggers

System function to include reboot, upgrade, check system and event logs, application (SDK) management

View profile information

Client requirements

Acceptable Operating Systems:	Windows 7 / 8.1 / 10, MAC OS X 10.9~10.11

Acceptable browsers

Plug-in free web viewer:	Google Chrome 47, MS Edge 20

Plug-in Web Viewer:	Internet Explorer 11, Mozilla Firefox 43, Apple Safari 9 * Mac OS X only

DETAILED SPECIFICATIONS

Video

Imager

Sensor: 1/1.8" CMOS

Scanning: Progressive

Minimum illumination

Color mode: 0.02 Lux (1/30sec, F1.6, 30IRE)

Black and white mode: 0.002 Lux (F1.6, IR LED off), 0 Lux (IR LED on)

The following features with control settings shall be available:

Camera Title

Off / On (Displayed up to 85 characters per line)

W/W: English / Numeric / Special characters

China: English / Numeric / Special / Chinese characters

Common: Multi-line (Max. 5), Color (Grey / Green / Red / Blue / Black / White), Transparency, Auto scale by resolution

Direction Indicator

Supported

Day/night setting

Auto (ICR) / Color / Black and white / External / Schedule

Backlight compensation (BLC)

BLC, HLC, WDR, SSDR

WDR

Extreme WDR (120dB)

Contrast Enhancement

Off / On (SSDR: Samsung Super Dynamic Range)

Digital Noise Reduction (DNR)

Off / On (WiseNRⅡ - Based on AI engine)

Off / On (SSNR: Samsung Super Noise Reduction)

Digital Image Stabilization

Off / On (built-in gyro sensor)

Defog

Off / Auto / Manual

Motion Detection

Off / On (8 polygonal zones)

Privacy Masking

Off / On (32 zones, 4-point quadrangle)

Color: Grey/Green/Red/Blue/Black/White/Mosaic

Dynamic privacy masking

Color: Grey/Green/Red/Blue/Black/White/Purple/Yellow/Mosaic

Gain Control

Off / Manual / Max

White Balance

ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor / Mercury / Sodium

Electronic shutter speed

Minimum / Maximum / Anti-flicker / Prefer shutter control (Based on AI engine)

Speed Range: 2 ~ 1/30,000sec

Image flip

Off / On

Image mirror

Off / On

Installation Direction

Upright / Inverted / Canted

Alarm In / Out

Supports external devices via optional I/O box

Alarm Triggers

Analytics, Network disconnect, Alarm input, Time schedule, MQTT subscription

Alarm Events

File upload (image): Email/FTP

File upload (video clip): FTP/SFTP

Notification: Email

Recording: SD/SDHC/SDXC or NAS recording at event triggers, Alarm output

Handover: PTZ preset, send message by HTTP/HTTPS/TCP

Audio clip playback

MQTT: publication

Pixel Counter

Available in the plug-in Web Viewer.

Lens

Focal length

4.75 ~ 261.4mm with 55x optical zoom and 1760x digital zoom

Aperture Ratio

F1.6 (Wide) ~ F6.5 (Tele)

Angle of view

Horizontal: 58.6° (Wide) ~ 1.23° (Tele)

Vertical: 34.8° (Wide) ~ 0.71° (Tele)

Diagonal: 65.5° (Wide) ~ 1.39° (Tele)

Min. Object Distance

5m (16.4ft)

Focus control

Auto, Manual, One-Shot AF, Focus save

Lens Type

DC auto iris

IR Viewable Length: 500m (1,640.42ft)

IR Wavelength: 850nm

Water Removal

Supported with integrated wiper

Auto Tracking

Object auto tracking (Person/Vehicle)

Target lock tracking

PTR

PAN Range

360° Endless

PAN Speed

Preset: Max. 160°/sec

Manual: 0.024°/sec ~ 160°/sec

TILT Range

Upright: 140° (50° ~ 90°)

Canted: 165° (75° ~ -90°)

Inverted: 140° (90° ~ 50°)

TILT Speed

Preset: Max. 160°/sec

Manual: 0.024°/sec ~ 160°/sec

Sequence

Preset (300), Swing, Group(6), Trace, Tour, Auto Run, Schedule

Preset Accuracy

Up to ±0.07°

Video Streams

The 8MP camera shall be able to produce 10 video profiles, each of which may have the following properties:

Encoding type:

H.265

H.264

MJPEG

Resolution:

3840x2160, 2592x1944, 2592x1464, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

Maximum frame rate:

H.265 / H.264: up to 60 fps at 60 Hz, up to 50 fps at 50 Hz

MJPEG: up to 30 fps

Bit rate control method:

H.265 and H.264: target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

MJPEG: quality level control, variable bit rate (VBR)

Number of multi-streaming profiles

10 maximum

Simultaneous users (total)

20 maximum (unicast)

Storage and Recording

The 8MP camera shall have onboard SD card storage.

Card type: microSD (SD/SDHC/SDXC supported)

Capacity: 1 slot, maximum 1TB (Hanwha Vision branded microSD cards have been certified and recommended for OnCloud edge recording)

Image content on the card shall have the ability to be downloaded to a selected destination.

Interoperability

Video streams shall be capable of supporting ONVIF protocol, profiles S and G.

Single Images

The 8MP camera shall support jpg file image screenshot and export.

Memory

4GB RAM, 512MB Flash

Network

Ethernet: RJ-45 Push-pull Connector (10/100BASE-T)

Video compression

H.265/H.264: Main / Baseline / High

MJPEG

Smart Codec: WiseStream (Based on AI engine)

Bitrate Control

H.265 / H.264: CBR or VBR

MJPEG: VBR

Streaming

Unicast (up to 20 users) / Multicast

Multiple streaming (Up to 10 profiles)

Protocols supported:

Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

Configuration: Dynamic Host Configuration Protocol (DHCP), Dynamic Domain Name System (DDNS)

Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS), Secure Sockets Layer/Transport Layer Security (SSL/TLS)

Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Network Time Protocol (NTP), Simple Network Management Protocol (SNMP v1/v2c/v3 – MIB-2), Universal Plug and Play (UPnP), Quality of Service (QoS)

Network discovery: Link Layer Discovery Protocol (LLDP), Cisco Discovery Protocol (CDP)

Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol (RTCP), Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol (SRTP)

Multicast: Internet Group Management Protocol (IGMP), Protocol Independent Multicast-Sparse Mode (PIM-SM)

Notifications: File Transfer Protocol (FTP), Secure File Transfer Protocol (SFTP), Simple Mail Transfer Protocol (SMTP), Message Queuing Telemetry Transport (MQTT)

Remote Access: Point-to-Point Protocol over Ethernet (PPPoE)

Management: System Logging Protocol (Syslog)

Transportation Systems: National Transportation Communications for ITS Protocol (NTCIP) 1205

DDNS

The 8MP camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

Quality of Service (QoS)

Layer 3 DSCP

Security features

HTTPS (SSL) Login Authentication

Digest Login Authentication

IP Address Filtering

User access log

802.1X Authentication (EAP-TLS, EAP-LEAP)

Device certificate (Hanwha Vision Root CA)

Discovery

Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

Application Programming Interface

ONVIF Profile S / G / T / M

SUNAPI (HTTP API)

Hanwha Vision Open Platform

Direct Cloud Connection

Supported

Electrical

Power

Input Voltage

PoE++ (IEEE 802.3bt, Class8, Type4)

Power Consumption

Typical 33.2W, Max 71W

Mechanical And Environmental

Color

White, Black

RAL Code

RAL9003(white), RAL9005(Black)

Material

Aluminum, PC(Polycarbonate), Glass

Material Management

RoHS(EU RoHS directive 2011/65/EU and IEC/EN 63000:2018)

REACH(Regulation (EC) No. 1907/2006)

Conflict mineral policy(Section 1502 of US Dodd Frank Act)

Dimensions (W x H)

ø188 x 264.9 mm (7.40 x 10.47")

Weight

Total Weight (Incl. Mounting Hardware): 10.4 kg (22.92 lb)

Unit Weight (Excl. Mounting Hardware): 9.4 kg (20.72 lb)

Operating Condition

Temperature: -50°C ~ +60°C (-58°F ~ +140°F) / +74°C(+165°F) (MAX) based on NEMA-TS 2(2.2.7),

Start-up should be done at above -30°C(-22°F)

Humidity: 0~100% RH (condensing)

Storage Condition

Temperature: -50°C ~ +60°C (-58°F ~ +140°F)

Humidity: 0~90% RH (non-condensing)

Wind load: Sustained 257km/h (160mph)

EPA (Effective Projected Area): 0.095 m2

DORI

Detect (25PPM/ 8PPF): Wide: 68.4m(224.5ft) / Tele: 3577.4m(11736.7ft)

Observe (63PPM/ 19PPF): Wide: 27.4m(89.8ft) / Tele: 1430.9m(4694.7ft)

Recognize (125PPM/ 38PPF): Wide: 13.7m(44.9ft) / Tele: 715.5m(2347.3ft)

Identify (250PPM/ 76PPF): Wide: 6.8m(22.5ft) / Tele: 357.7m(1173.7ft)

Certification

EMC (Electromagnetic Compatibility)

North America

FCC 47 CFR 15 Subpart B Class A

ICES-3(A)/NMB-3(A)

Europe

CE/UKCA - EN 55032 Class A, EN 50121-4, EN 50130-4, EN 55035

Asia-Pacific

VCCI CISPR 32 Class A (Japan)

RCM AS/NZS CISPR 32 Class A (Australia/New Zealand)

Korea

KS C 9832 Class A, KS C 9835

Safety

UL 62368-1, CAN/CSA C22.2 NO. 62368-1

IEC 62471

IEC/EN/UL 62368-1 GS UL cUL CE FCC VCCI CB

Environment

Ingress Protection

IEC 60529 IP66, IEC 60529 IP68

IEC 62262 IK10

Transportation Standards

NEMA TS 2-2013 (2.2.7, 2.2.8, 2.2.9)

NEMA TS2 (2.1.9, 2.1.10, 2.2.4)

NEMA 250 Type 4X, NEMA 250, 5.6

Environmental Testing Standards

IEC 60068-2-1 (Cold), IEC 60068-2-2 (Dry heat), IEC 60068-2-6 (Vibration), IEC 60068-2-14 (Temperature change), IEC 60068-2-27 (Shock), IEC 60068-2-78 (Damp heat), IEC 60068-2-52:2017 (Salt mist cyclic)

MIL-STD-810H: 501.7 (High temperature), 502.7 (Low temperature), 503.7 (Temperature shock), 505.7 (Solar radiation), 506.6 (Rain), 507.6 (Humidity), 509.7 (Salt fog), 510.7 (Sand and dust), 521.4 (Icing/Freezing rain)

ASTM B117 (Salt spray corrosion)

Environmental Management

IEC/EN 63000

END OF SECTION

8