Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com)

Model: TNP-A7430RW

Alternates:	TNP-A9430RW, TNP-A6550RW, TNP-A9430RWB, TNP-A7430RWB, TNP-A6550RWB

GENERAL DESCRIPTION

The camera is a 4M AI Ruggedized PTZ Positioning Camera powered by Wisenet 9 SoC with Dual NPU architecture, dedicating one processor exclusively to video quality and the other to analytics, ensuring that AI tasks never compromise image quality.

Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

H.265 / H.264: up to 60 fps at 60 Hz, up to 50 fps at 50 Hz

MJPEG: up to 30 fps (at 4MP resolution, the maximum frame rate is 5 fps)

The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

The camera shall support multicast and unicast video streaming up to 20 users.

The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

The camera shall provide AI-based WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream and reduce storage while producing video quality that is visually equal to the one without smart codec.

Camera – The camera device shall have the following physical and performance properties:

The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 120dB.

IP66/IP68, IK10 (except window), NEMA 4X, NEMA TS 2 (2.2.7, 2.2.8, 2.2.9), MIL-STD-810H and ASTM B117 compliant, wind load resistance up to 257 km/h (160 mph).

Auto day/night operation with removable IR cut filter.

Low light level operation to 0.04 Lux at F1.69, 1/30sec, 30 IRE in color mode and 0.004 Lux at F1.69 with IR LED off in black and white mode, achieving 0 Lux performance when IR LED is activated.

2D and 3D digital noise reduction.

Optical image stabilization support.

Canted mode support, providing a tilt range of 165° to eliminate blind spots directly beneath the camera.

Integral IR illumination, providing effective visibility of 500m (1,640.42ft) at 0 Lux when activated in black and white mode.

32 privacy masking regions with quadrangle.

Supports dynamic privacy masking which leverages AI to mask sensitive objects in compliance with privacy regulations.

Integrated wiper for water removal and window/pan ice removal support.

Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

Motion detection with 8 definable detection areas with 8-point polygonal zones, and minimum / maximum object size.

Classified object analytics

Person / Face / Vehicle / License plate

Attributes

Person: Upper/Lower clothes color, Gender, Bag

Vehicle: Type (car/bus/truck/motorcycle/bicycle), Color

Face: Age, Gender, Face mask, Glasses

Support BestShot per object.

Analytics events

Based on AI engine: Object detection, Virtual line (Crossing / Direction), Motion detection, Slip & Fall detection, Virtual area (Loitering / Intrusion / Enter / Exit / Appear / Disappear)

Normal: Tampering, Audio detection, Shock detection

AI object auto-tracking (Person / Vehicle) with target lock tracking.

Interoperability – The camera shall be ONVIF Profile S, G, T and M compliant.

The camera shall possess the following further characteristics:

Built-in web server, accessed via standard browsers including Internet Explorer, Firefox, Chrome & Safari.

Local recording via microSD card (supporting SD, SDHC, and SDXC formats, up to 1TB) and network recording via NAS, with configurable pre-alarm and post-alarm recording intervals.

Bi-directional audio via optional I/O box.

Alarms and notifications

Alarm notification triggers: Analytics, Network disconnect, Alarm input, Time schedule, MQTT subscription, Case tampering, Day/Night, Storage disruption

Available notification means upon trigger: File upload (image) via FTP/email, File upload (video clip) via FTP/SFTP, Notification via email, Local storage (SD/SDHC/SDXC) or NAS recording at event triggers, Alarm output, PTZ preset, Handover, Audio clip playback, MQTT publication, Send message by HTTP/HTTPS/TCP

Pixel Counter available in the plug-in Web Viewer.

This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

This product is compliant with the National Defense Authorization Act (NDAA), Section 889, and does not contain any covered telecommunications equipment or services as defined by the Act.

The camera seamlessly integrates with popular video management software.

The camera integrates with OnCloud, HealthPro, and SightMind for remote device management and access.

CAMERA SOFTWARE

The camera shall have a built-in web server which supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari, for which web viewer plug-ins are available, from a PC or Mac.

The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

Live view window size

Resolution setting

Image (snapshot) capture

Manual recording to SD or NAS

Audio and microphone control

Access Playback and Setup menus

Digital PTZ

The web viewer shall provide an auto-tracking feature which tracks a specific object using object detection analytics or targeted object tracking.

The web viewer shall provide a playback screen which provides access to the following functions:

Search date and time range

Search event type

Play an event video

Set resolution

Play audio if present

Generate a backup copy of saved video data

The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, crop encoding area

User profile to include password, access level, authentication

Date and time

Network settings and IP version

DDNS

SSL, including certificate management

802.1x authentication

Quality of Service settings

SNMP to include version selection and settings

Auto configuration

Video setup to include flip and mirror mode, hallway view mode, video type, privacy zone

Audio setup to include source, audio codec type, gain, and bit rate

Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, IR illumination, sharpness, contrast, and color level.

Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection, event triggers

System function to include reboot, upgrade, check system and event logs, application (SDK) management

View profile information

DETAILED SPECIFICATIONS

Video

Imaging Device: 1/1.8" CMOS, Progressive scan

Resolution: 2592x1520, 2560x1440, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

Maximum Framerate

H.265/H.264: 60fps/50fps (60Hz/50Hz)

MJPEG: 30fps (at 4MP, Max. 5fps)

Minimum Illumination

Color: 0.04 Lux (F1.69, 1/30sec, 30IRE)

B/W: 0.004 Lux (F1.69, IR LED off), 0 Lux (IR LED on)

Lens

Focal Length: 6.1 ~ 262.4mm

Zoom Ratio

Optical: 43x

Digital: 1367x

Maximum Aperture Ratio (F number)

Wide: F1.69

Tele: F4.92

Angular Field of View

Horizontal: 62.5° (Wide) ~ 1.72° (Tele)

Vertical: 38.42° (Wide) ~ 1.00° (Tele)

Diagonal: 70.34° (Wide) ~ 2.02° (Tele)

Minimum Object Distance: 5m (16.4ft)

Focus Control: Auto, Manual, One-Shot AF, Focus save

Lens Type: P-iris (IR corrected)

Pan / Tilt / Rotate

Pan Range: 360° Endless

Pan Speed

Preset: Max. 160°/sec

Manual: 0.024°/sec ~ 160°/sec

Tilt Range

Upright: 140° (50° ~ -90°)

Canted: 165° (75° ~ -90°)

Inverted: 140° (90° ~ -50°)

Tilt Speed

Preset: Max. 160°/sec

Manual: 0.024°/sec ~ 160°/sec

Sequence: Preset (300ea), Swing, Group(6ea), Trace, Tour, Auto Run, Schedule

Preset Accuracy: Up to ±0.07°

Operational

Direction Indicator: Support

Day/Night: Auto (ICR) / Color / Black and white / External / Schedule

Backlight Compensation: BLC, HLC, WDR, SSDR

Wide Dynamic Range: Extreme WDR (120dB)

Digital Noise Reduction: WiseNRⅡ (Based on AI engine), SSNR V

Digital Image Stabilization: Support (built-in gyro sensor)

Optical Image Stabilization: Support

Defog: Off / Auto / Manual

Motion Detection

Quantity: 8ea

Shape: 8-point polygonal zones

Privacy Masking

Quantity: 32ea

Shape: 4-point quadrangle zones

Color / Option: Gray, Green, Red, Blue, Black, White, Mosaic

Dynamic Privacy Masking

Color / Option: Gray, Green, Red, Blue, Black, White, Mosaic, Purple, Yellow

Gain Control: Off / Manual / Max

White Balance: ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor / Mercury / Sodium

Electronic Shutter

Mode: Minimum / Maximum / Anti-flicker / Prefer shutter control (Based on AI engine)

Speed Range: 2 ~ 1/30,000sec

Video Rotation: Flip, Mirror

Installation Direction: Upright / Inverted / Canted

IR Viewable Length: Up to 500m (1,640.42ft)

IR Wavelength: 850nm

Water Removal: Support (with integrated wiper)

Ice Removal: Window, Pan

Alarm In / Out: Supports external devices via optional I/O box

Alarm Triggers: Analytics, Network disconnect, Alarm input, Time schedule, MQTT subscription, Case tampering, Day/Night, Storage disruption

Alarm Events When Alarm Trigger Occurred

File upload (image): Email/FTP

File upload (video clip): FTP/SFTP

Notification: Email

Recording: SD/SDHC/SDXC or NAS recording at event triggers, Alarm output

Handover: PTZ preset, Send message by HTTP/HTTPS/TCP, Audio clip playback

MQTT: publication

Audio In: Support via optional I/O box

Audio Out: Support via optional I/O box

Pixel Counter: Available in the plug-in Web Viewer

Analytics

Classified Object Type: Person, Face, Vehicle, License plate

Attributes

Person: Upper/Lower clothes color, Gender, Bag

Vehicle: Type (car, bus, truck, motorcycle, bicycle), Color

Face: Age, Gender, Face mask, Glasses

Thumbnail: BestShot

Auto Tracking: Object auto tracking (Person/Vehicle), Target lock tracking

Analytics Events

Based on AI engine: Object detection, Virtual line (Crossing/Direction), Motion detection, Slip & Fall detection, Virtual area (Loitering/Intrusion/Enter/Exit/Appear/Disappear), Auto tracking

Normal: Tampering, Audio detection, Shock detection

Network

Ethernet: Metal shielded RJ-45 (10/100BASE-T)

Video compression: H.265/H.264: Main / Baseline / High, MJPEG

Smart Codec: WiseStream (Based on AI engine)

Bitrate Control

H.265 / H.264: CBR or VBR

MJPEG: VBR

Streaming

Unicast: Up to 20 users

Multicast: Support

Multiple Streaming: Up to 10 profiles

Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP, SFTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, CDP, SRTP(TCP, UDP Unicast), MQTT, NTCIP 1205, PPPoE, Syslog

Application Programming Interface

ONVIF: Profile S, G, T, M

Others: SUNAPI (HTTP API), Hanwha Vision Open Platform

Direct Cloud Connection: Support

Security

OS / Firmware Protect: Encrypted firmware, Secure boot, Signed firmware

User Authentication: Digest authentication, Prevent brute-force attack

Network Authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

Secure Communication: HTTPS, WSS (WebSocket Secure), SRTP

Access Control: IP-based access control, MAC-based access control, Auto logout

Data Protect: Encryption credentials, Encrypt compress for live recording file export

Audit: User Access / System / Event Log management

Device ID: Device certificate (Hanwha Vision Root CA)

Secure Storage: SD card partition encrypt (AES-256), Secure element

Supply Chain Security: SBOM

Security Certificate: Secure element (FIPS 140-3 level3, CC EAL6+), Consumer IoT Security Standard (ETSI EN 303 645)

General

SoC

Type: Wisenet 9

Dual NPU: Support

Memory

RAM: 4GB

Flash: 8GB eMMC

Edge Storage

Micro SD/SDHC/SDXC: Max 1TB x 1slot

Edge Storage (OnCloud)

Micro SD/SDHC/SDXC: Max 1TB x 1slot

Hanwha Vision branded microSD cards have been certified and recommended for OnCloud edge recording.

Electrical

Input Voltage: HPoE (IEEE 802.3bt, Class8, Type4)

Power Consumption: Max 71W, Typical 33.2W

WisePower: Power mode, Power monitoring, Eco mode

Mechanical and Environmental

Color: Black

RAL Code: RAL9005(Black)

Material: Aluminum, PC(Polycarbonate), ITO Glass

Material Management

RoHS (EU RoHS directive 2011/65/EU and IEC/EN 63000:2018)

REACH (Regulation (EC) No. 1907/2006)

Conflict mineral policy (Section 1502 of US Dodd Frank Act)

Product Dimensions

Upright Normal/Inverted: 293.4(W) x 205.6(D) x 399.9(H)mm (11.55 x 8.09 x 15.74")

Upright Canted: 293.4(W) x 301.9(D) x 363.6(H)mm (11.55 x 11.89 x 14.31")

Weight

Total Weight (Incl. Mounting Hardware): 10.5 kg (23.14 lb)

Unit Weight (Excl. Mounting Hardware): 9.5 kg (20.94 lb)

Operating Condition

Temperature: -50°C ~ +60°C (-58°F ~ +140°F) / +74°C(+165°F) (MAX) based on NEMA-TS 2(2.2.7),

Start-up should be done at above -30°C(-22°F)

Humidity: 0~100% RH (condensing)

Storage Condition

Temperature: -50°C ~ +60°C (-58°F ~ +140°F)

Humidity: 0~90% RH (non-condensing)

Wind load: Sustained 257 km/h (160mph)

EPA (Effective Projected Area): 0.095 m2

DORI (EN62676-4 standard)

Detect (25PPM/ 8PPF):		Wide	85.4m(280.28ft) /	Tele	3454m(11330ft)

Observe (63PPM/ 19PPF):	Wide	34.2m(112.11ft) /	Tele	1381m(4532ft)

Recognize (125PPM/ 38PPF): 	Wide	17.1m(56.06ft) /		Tele	690.7m(2266ft)

Identify (250PPM/ 76PPF): 	Wide	8.5m(28.03ft) /		Tele	345.3m(1133ft)

Certifications & Standards

EMC

North America

FCC 47 CFR 15 Subpart B Class A

ICES-3(A)/NMB-3(A)

Europe

CE/UKCA: EN 55032 Class A, EN 50121-4, EN 50130-4, EN 55035

Asia-Pacific

VCCI CISPR 32 Class A (Japan)

RCM AS/NZS CISPR 32 Class A (Australia/New Zealand)

Korea

KS C 9832 Class A, KS C 9835

Safety

UL 62368-1, CAN/CSA C22.2 NO. 62368-1

IEC 62471

Environment

Ingress Protection: IEC 60529 IP66, IEC 60529 IP68, IEC 62262 IK10

Transportation Standards: NEMA TS 2-2013 (2.2.7, 2.2.8, 2.2.9), NEMA TS2 (2.1.9, 2.1.10, 2.2.4), NEMA 250 Type 4X, NEMA 250, 5.6

Environmental Testing: IEC 60068-2-1, IEC 60068-2-2, IEC 60068-2-6, IEC 60068-2-14, IEC 60068-2-27, IEC 60068-2-78, IEC 60068-2-52:2017

MIL-STD-810H: 501.7, 502.7, 503.7, 505.7, 506.6, 507.6, 509.7, 510.7, 521.4

ASTM B117 (Salt spray corrosion)

Environmental Management: IEC/EN 63000

END OF SECTION

8