Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

    - Model: TNS-9060IBC

    - Alternates: None

  - GENERAL DESCRIPTION

    - This camera is a barcode reading and video monitoring camera with dual global 4K image sensor.

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.264, H.265 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - Mono

          - 4096x2160 @ 15/18/20fps all codec

        - Color

          - 4096x2160 @ 15/18/20fps all codec

          - 640x360 @ 15/18/20fps all codec

      - The camera shall be able to configure various resolution.

        - Mono: 4096 x 2160

        - Color: 4096 x 2160, 640 x 360

      - The camera shall support unicast video streaming up to 6 users.

      - The camera shall support multicast video streaming.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - AI based Object detection (person/face, visible channel only)

    - AI barcode location search - Track barcodes on high speed conveyor belt moving at 2m/s(6.5ft/s)

    - 1D Code is available (Code128 / Codabar / Interleaved 2 of 5)

    - Reinforced durability

      - IP66, IP67, IK10, NEMA TS-2 2.2.9, M12 8pin Ethernet connect

      - Humidity control with AIR vent, EN 60068-2-6(vibration resistance)

      - EN 60068-2-27 (shock resistance), Operating temperature -40°C ~ + 55°C

    - Interoperability – The camera shall be SUNAPI(HTTP API) and Wisenet Open Platform compliant.

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device: Global shutter Mono 1” CMOS and Color 1” CMOS

      - Resolution

        - Mono: Max. 4096x2160

        - Color: Max. 4096x2160

      - Max. Framerate

        - Mono: Max. 20fps

        - Color: Max. 20fps

      - Pixel Size: 3.45μm(H)x3.45μm(V)

    - Lens

      - Focal length: 35mm

      - Max. Aperture Ratio

        - Mono: F8.0

        - Color: F8.0

      - Field of View

        - Mono: H(21.5°) / V(11.5°) / D(24.2°)

        - Color: H(21.5°) / V(11.5°) / D(24.2°)

      - Min. Object Distance: 0.2m(0.66ft)

      - Focus Control: Manual

      - Lens Type: Manual

      - Mount Type: C Mount

    - Operational Functions

      - Camera Title: Displayed up to 85 characters

      - Digital Noise Reduction: SSNRⅤ

      - Gain Control: Manual 0~48dB

      - White Balance: AWC / Manual

      - Electronic Shutter Speed: Manual 1/10,000~1/30s (100us ~ 33.33ms)

      - Video Rotation: Flip, Mirror

      - Light Type: White LED 32ea

      - Analytics

        - Barcode recognition

        - Barcode type: 1D Code, Code128 / Codabar / ITF(Interleaved 2 of 5)

      - Memory: 4GB RAM, 512MB Flash

    - Streaming

      - Video compression

        - H.265/H.264: Main/High

        - MJPEG

      - Bitrate Control Method

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming Capability: Unicast (6 users) / Multicast

        - Mono:

          - 4096x216 @ 15/18/20fps all codec

        - Color

          - 4096x2160 @ 15/18/20fps all codec

          - 640x360 @ 15/18/20fps all codec

      - Interoperability: SUNAPI (HTTP API), Wisenet Open Platform

    - Network

      - Connectivity: M12 8pin connector (10/100/1000BASE-T)

      - Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, SRTP (TCP, UDP Unicast)

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - OS / Firmware Protect: Encrypted Firmware, Secure boot, Signed Firmware

        - User authentication: Digest Authentication

        - Network authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

        - Secure Communication: HTTPS, SRTP, WSS (WebSocket Secure)

        - Access Control: IP address filtering

        - Data Protect: Authentication information encryption, ZIP compression encryption

        - Audit: User Access / System / Event Log

        - Device ID: Device certificate (Hanwha Private Root CA, pre-installed)

        - Secure Storage: HTPM (Hanwha Trusted Platform Module)

        - Security Certificate: Secure by default

        - The device shall not provide a manufacture default password. Default password change shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Environmental and Electrical

      - Operating Temperature / Humidity

        - -40°C to +55°C(-40°F to +131°F) / 0~95% RH *Start up should be done at above -30°C

        - NEMA TS-2: 74°C - /

      - Storage Temperature / Humidity: -40°C to +60°C (-40°F to +140°F) / 0~95% RH

      - Certification: IP67, IP66, IK10(Exclude Window), NAME 4X

      - Input Voltage: 24VDC

      - Power Consumption: Max. (LED on) 28.6W, Typical (LED off) 13.7W

    - Mechanical and Environmental

      - Color/Material: Black/Aluminum

      - RAL Code: RAL9005

      - Product Dimension

        - Camera (W x H x D): 218 x 104 x 127.1 mm (8.58 x 4.09 x 5.00 inch)

        - LED (W x H x D): 255.8 x 73.8 x 49.8 mm (10.03 x 2.91 x 1.96 inch)

        - Total (W x H x D): 255.8 x 181 x 127.1 mm (8.58 x 7.13 x 5.00 inch)

      - Weight

        - Camera: 1897g(4.182lb)

        - LED: 996g(2.196lb)

        - Total: 3438g(7.579lb) *with install bracket

    - Certification & Standards

      - EMC

        - FCC 47 CFR Part 15, Subpart B CLASS A

        - ICES-3(A)/NMB-3(A)

        - CE / UKCA: EN 55032 CLASS A, EN 50130-4, EN 61000-3-2, EN 61000-3-3 CLASS A

        - Ks c 9832 Class A, KS C 9635

      - Safety

        - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

        - IEC/EN 62471

      - Environment

        - EN 60068-2-6 (variation resistance)

        - EN 60068-2-27 (shock resistance)

        - NEMA TS-2 2.2.9 (shock resistance)

        - IEC/EN 60529 IP67, IP66

        - IEC/EN 62262 IK10

END OF SECTION

10