Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin (http://security.hanwhatechwin.com/)

    - Model : TNV-7011RC

    - Alternates :	None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 – frame rates Max. 30fps at all resolution

        - MJPEG – frame rates to Max. 30fps

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections

2048x1536, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IP6K9K, IK10+ rated for protection against impacts

      - Auto day/night operation with removable IR cut filter

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of 10m(32.8ft) at 0 Lux when activated in Black & White mode

      - 32 privacy masking regions utilizing polygonal

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with 8 definable detection areas, minimum / maximum object size definition that ignores false alarms such as trees and waves on water.

      - Detection of logical events of specified conditions from the camera’s video input

        - Analytics events : Tampering, Loitering, Directional detection, Defocus detection, Fog detection, Virtual line, Enter/Exit, Appear/Disappear, Audio detection, Face detection, Motion detection, Digital auto tracking, Sound classification, Shock detection

    - Interoperability – The camera shall be ONVIF Profile S / G / T compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including MS Edge, Firefox, Chrome and Safari

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Bi-directional audio

      - Alarms and notifications

        - alarm notification triggers:

          - analytics

          - network disconnect

          - alarm input

        - available notification means upon trigger:

          - file upload via FTP and e-mail

          - notification via e-mail

          - record to local storage (SD/SDHC/SDXC or NAS)

          - alarm output

          - DPTZ preset

      - Pixel Counter available in the plug-in web viewer.

      - POE capable

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : 1/2.8" 3M CMOS

          - pixels per sensor : 2048 (H) x 1536 (V)

          - scanning : progressive

        - Minimum illumination

          - Color mode : 0.3Lux(F2.0, 1/30sec)

          - Black & white mode (IR LED off) : 0.03Lux(F2.0, 1/30sec)

          - Black & white mode (IR LED on) : 0Lux

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 85 characters)

            - English / Numeric / Special characters

          - Day/night : Auto (ICR)

          - Backlight compensation : BLC, HLC, WiseDR

          - WDR : 120dB

          - Digital Noise Reduction (DNR) : SSNR V

          - Digital Image Stabilization : Off / On

          - Motion Detection : Off / On (8ea, 8 point polygonal zones)

          - Privacy Masking : Off / On (32ea, polygonal zones)

          - Gain Control : Low / Middle / High

          - White Balance : ATW / AWC / Manual / Indoor / Outdoor

          - Lens Distortion Compensation(LDC) : Support

          - Electronic shutter speed : Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec)

          - Image flip : Off / On

          - Image mirror : Off / On

          - Hallway view : Off / On (90° / 270°)

          - Analytics

            - Tampering, Loitering, Directional detection, Defocus detection, Fog detection, Virtual line, Enter/Exit, Appear/Disappear, Audio detection, Face detection, Motion detection, Digital auto tracking, Sound classification, Shock detection

          - Alarm I/O : Input 1ea / Output 1ea

          - Alarm Triggers : Analytics, Network disconnect, Alarm input

          - Alarm Events

            - File upload via FTP and e-mail

            - Notification via e-mail

            - SD/SDHC/SDXC or NAS recording at Event Triggers

            - Alarm output

            - DPTZ preset

          - Video Out for installation

            - CVBS: 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P)

        - Lens : 2.8mm fixed focal

          - Max. Aperture Ratio : F2.0

          - FoV

            - H : 102˚ / V : 75˚ / D : 129˚

          - Min. Object Distance : 0.4m (1.31ft)

          - Focus Control : Manual

          - Lens Type : Fixed

        - Pan /Tilt / Rotate

          - - / 30 ~ 60 ˚  / -

        - DORI (EN62676-4 standard)

          - Detect : 33.2m(108.82ft)

          - Observe : 13.3m(43.53ft)

          - Recognize : 6.6m(21.76ft)

          - Identify : 3.3m(10.88ft)

      - IR Viewable Length : 10m (32.8ft), 940nm wave length

      - Video Streams

        - The camera shall be able to produce 10 video profiles, each of which may have the following properties:

          - Encoding type : H.265, H.264, MJPEG

          - Resolution:

            - 2048x1536, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

          - Frame rate

            - H.265 and H.264 : Max. 30fps at all resolution

            - MJPEG : Max. 30fps

          - Smart Codec : Manual(5ea area), WiseStream Ⅱ

          - Bit rate control

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

      - Number of multi-streaming profiles : 10 profiles

      - Simultaneous users (total) : 20 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to  256GB (1 slot)

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S / G / T.

      - Single Images - The camera shall support jpg file image screenshot and export.

    - Network

      - Connectivity : RJ-45(10/100BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Link Layer Discovery Protocol(LLDP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol(SRTP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication (EAP-TLS, EAP-LEAP)

      - Application Programming Interface

        - ONVIF Profile S / G / T, SUNAPI(HTTP API), Wisenet open platform

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Audio

      - Audio In

        - Selectable (mic in / line in / built-in MIC)

        - Supply voltage: 2.5VDC(4mA), Input impedance: 2K Ohm

      - Audio Out

        - Line out, Max. output level: 1Vrms

    - Electrical

      - Input Voltage : PoE (IEEE802.3af, Class2), 12VDC

      - Power consumption

        - PoE : Max 12W, Typical 10.1W

        - 12VDC : Max. 11W, Typical 8.6W

    - Mechanical And Environmental

      - Material : Aluminum

      - Dimensions  : Ø 295.5 x 113.5mm (Ø 11.6 x 4.47”)

      - Weight : 2.1kg (4.63lb)

      - Temperature And Humidity

        - Operating : -10°C ~ +55°C(+14°F ~ +131°F) / Less than 90% RH (Non-condensing)

        - Storage : -50°C ~ +60°C(-58°F ~ +140°F) / Less than 90% RH (Non-condensing)

      - Environmental Rating

        - IP66

        - IP6K9K

        - IK10+

      - RAL Code

        - RAL9003

    - EMC & Safety

      - FCC 47 CFR Part 15 Subpart B

        - ANSI C63.4-2014 Class A

      - IC Regulation ICES-003 Issue 7

        - ANSI C63.4a-2017 Class A

      - CE EMC-Directive 2014/30/EU

        - EN 55032:2015/A11:2020

        - EN 50130-4:2011

      - UK EMC-Regulations 2016/1091

        - BS EN 55032:2015/A11:2020 Class A

        - BS EN 50130-4:2011/a1:2014

      - VCCI-CISPR 32:2016 Class A

      - AS/NZS CISPR32:2015 Class A

      - UL listed

      - CE EN 50581:2012 (RoHS)

      - BS EN 50581:2012 (RoHS)

END OF SECTION

8