Hanwha Techwin is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin(http://www.hanwha-security.com/)

    - Model : XNB-6002

    - Alternates : None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 / H.264 : Max. 60fps/50fps(60Hz/50Hz)

        - MJPEG : Max. 30fps/25fps(60Hz/50Hz)

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - 16:9 aspect ratio : 1920x1080, 1280x720, 800x448, 640x360

        - 4:3 aspect ratio : 1280x960, 1024x768, 800x600, 640x480, 320x240

        - 5:4 aspect ratio : 1280x1024, 720x576

        - 3:2 aspect ratio : 720x480

      - The camera shall support unicast video streaming up to 20 users.

      - The camera shall support multicast video streaming.

      - The camera shall support multiple video streaming up to 10 profiles.

      - The camera shall be able to configure Dynamic DNS(DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStream Ⅱ, Dynamic GOV and Dynamic FPS to efficiently manage bit rate of the video stream and reduce storage.

      - The camera shall provide WiseNR for reduce noise and blur on image.

    - Camera – The camera device shall have the following physical and performance properties:

      - Electrical day/night operation

        - Low light level operation shall depend on specifications of lens modules.

        - Available lens modules : SLA-T1080FA, SLA-T2480A, SLA-T2480VA, SLA-T4680A, SLA-T4680VA

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 120dB.

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 32 privacy masking areas with quadrangle zones.

      - The camera shall have the defog feature to remove fogginess of scene which can be triggered automatically from the fog detection event.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Analytics events : Defocus detection, Motion detection, Tampering, Fog detection, Audio detection, Sound classification, Face detection, Virtual area(Loitering/Intrusion/Enter/Exit/Appear/ Disappear), Virtual line(Crossing/Direction)

        - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Detection and classification of the following sound.

          - Scream

          - Gunshot

          - Explosion

          - Crashing glass

      - Business Intelligence

        - People counting

        - Queue management

        - Heatmap

    - Interoperability – The camera shall be ONVIF Profile S / G and T compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - Analytics

          - Network disconnect

          - Alarm input

          - App event

          - Time schedule

        - available notification means upon trigger:

          - File upload(image) : e-mail/FTP

          - Notification : e-mail

          - Recording : SD/SDHC/SDXC or NAS recording at event triggers

          - Alarm output

          - Handover(PTZ preset, Send message by HTTP/HTTPS/TCP)

          - Audio clip playback

      - Pixel Counter available in the web viewer.

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

      - NDAA Compliant

  - DETAILED SPECIFICATIONS

    - Video (Optional 5 different Lenses)

      - Lens Model

        - SLA-T1080FA

          - Type			Fisheye lens

          - Sensor			1/2.8" 2M CMOS

            - pixels per sensor:		1945 (H) x 1097 (V) effective

            - scanning			progressive

          - Minimum illumination

            - Color mode: 		0.03Lux (F1.6)

            - Black & white mode: 	0.03Lux (F1.6)

          - Focal Length		1.6mm fixed focal

          - Color/Material		Black / Plastic, Metal

        - SLA-T2480A

          - Type			Fixed lens

          - Sensor			1/2.8" 2M CMOS

            - pixels per sensor:		1945 (H) x 1097 (V) effective

            - scanning			progressive

          - Minimum illumination

            - Color mode: 		0.45Lux (F2.0)

            - Black & white mode: 	0.45Lux (F2.0)

          - Focal Length		2.4mm fixed focal

          - Color/Material		Black / Plastic

        - SLA-T2480VA

          - Type			Fixed lens

          - Sensor			1/2.8" 2M CMOS

            - pixels per sensor:		1945 (H) x 1097 (V) effective

            - scanning			progressive

          - Minimum illumination

            - Color mode: 		0.45Lux (F2.0)

            - Black & white mode: 	0.45Lux (F2.0)

          - Focal Length		2.4mm fixed focal

          - Color/Material		Black / Plastic, Metal

        - SLA-T4680A

          - Type			Pinhole lens

          - Sensor			1/2.8" 2M CMOS

            - pixels per sensor:		1945 (H) x 1097 (V) effective

            - scanning			progressive

          - Minimum illumination

            - Color mode: 		0.06Lux (F2.5)

            - Black & white mode: 	0.06Lux (F2.5)

          - Focal Length		4.6mm fixed focal

          - Color/Material		Black / Plastic

        - SLA-T4680VA

          - Type			Pinhole lens

          - Sensor			1/2.8" 2M CMOS

            - pixels per sensor:		1945 (H) x 1097 (V) effective

            - scanning			progressive

          - Minimum illumination

            - Color mode: 		0.06Lux (F2.5)

            - Black & white mode: 	0.06Lux (F2.5)

          - Focal Length		4.6mm fixed focal

          - Color/Material		Black / Plastic, Metal

    - Main Unit Specifications

      - Video out for installation

        - CVBS: 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P)

        - USB: micro USB type B, 1280 x 720

    - Operational Functions

      - Camera Title

        - Off / On (Displayed up to 85 characters)

        - English / Numeric / Special characters

        - Multi-line (Max. 5lines), Color (Grey / Green / Red / Blue / Black / White), Transparency, Auto scale by resolution

      - Day/Night Setting : Auto(Electrical)

      - Backlight Compensation : Off / BLC / HLC / extremeWDR(120dB)

      - Contrast Enhancement : SSDR

      - Digital Noise Reduction (DNR) : WiseNR, SSNRⅤ

      - Digital Image Stabilization (DIS) : Support

      - Defog : Support

      - Motion Detection : Off / On (8ea, 8-point polygonal)

      - Privacy Masking : Off / On (32ea, Quadrangle zones)

        - Color : Grey / Green / Red / Blue / Black / White

        - Mosaic

      - Gain Control : Off / Manual / Max Gain

      - White Balance : ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor

      - LDC : Support(Not support for fisheye lens)

      - Electronic Shutter Speed : Min / Max / Anti-flicker (2 ~ 1/12,000sec)

        - Prefer shutter control

      - Image Rotation

        - Flip : Off / On

        - Mirror : Off / On

        - Hallway view(90°/270°)

      - Digital PTZ : Support

      - Alarm I/O : Input 1ea / Output 1ea

      - Alarm Triggers

        - Analytics

        - Network disconnect

        - Alarm input

        - App event

        - Time schedule

      - Alarm Events

        - File upload(image) : e-mail/FTP

        - Notification : e-mail

        - Recording : SD/SDHC/SDXC or NAS recording at event triggers

        - Alarm output

        - Handover(PTZ preset, Send message by HTTP/HTTPS/TCP)

        - Audio clip playback

      - Pixel Counter : Support

      - Storage : Micro SD/SDHC/SDXC 1slot, Max. 1TB

      - Business Intelligence : Based on AI engine

        - People counting

        - Queue management

        - Heatmap

      - Analytics events : Defocus detection, Motion detection, Tampering, Fog detection, Audio detection, Sound classification, Face detection, Virtual area(Loitering/Intrusion/Enter/Exit/Appear/ Disappear), Virtual line(Crossing/Direction)

      - Memory : 2GB RAM, 512MB Flash

    - Video Streams

      - Video compression : H.265, H.264, MJPEG

      - Resolution : 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Maximum Framerate

        - H.265 / H.264 : Max. 60fps/50fps(60Hz/50Hz)

        - MJPEG : Max. 30fps/25fps(60Hz/50Hz)

      - Smart Codec

        - Manual(5ea area)

        - WiseStream Ⅱ

      - Bitrate Control Method

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming Capability : Unicast(20 users) / Multicast, Multiple streaming(Up to 10 profiles)

      - Streaming method : Unicast / Multicast

      - Simultaneous Users : 20 maximum (Unicast)

      - Profile set : Max. 10 ea

      - Interoperability

        - ONVIF Profile S / G / T

        - SUNAPI(HTTP API)

        - Wisenet Open Platform

    - Audio

      - Audio In

        - Selectable(mic in/line in)

        - Supply voltage: 2.5V DC(4mA)

        - Input impedance: 2K Ohm

      - Audio Out

        - Line out

        - Max. output level 1Vrms

      - Audio Compression

        - G.711 u-law /G.726 Selectable

        - G.726(ADPCM) 8KHz, G.711 8KHz

        - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

        - AAC-LC: 48Kbps at 16KHz

    - Network

      - Connectivity : RJ-45(10/100BASE-T)

      - Protocol

        - IP v4 / v6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, PIM-SM, SNMPv1/v2c/v3(MIB-2), UPnP

        - Media: RTP, RTCP, RTSP

        - Unicast: SRTP

        - Multicast: IGMP, PIM-SM

        - Notifications: FTP, SMTP

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - User password protection

        - The device shall not provide a manufacture default password. Default password change shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses

        - HTTPS(SSL/TLS) login authentication

        - HTTPS(SSL/TLS) secured communication

        - Digest login authentication

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP, EAP-PEAP MSCHAPv2)

        - TPM 2.0 (FIPS 140-2 level 2)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

        - Secure by default certificate

        - Secure OS/Boot/Storage, Verify firmware forgery

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Power

        - Input Voltage

          - PoE(IEEE802.3af, Class3)

          - 12VDC

        - Power Consumption

          - PoE : Max 10.5W, typical 6.9W

          - 12VDC : Max 8.6W, typical 5.5W

        - Power redundancy failover

    - Mechanical And Environmental

      - Color/Material : Black / Plastic, Metal

      - Dimensions (W x H) : 131.1(W)x33.5(H)x86(D)mm(5.16x1.32x3.39")

      - Weight : 420g(0.93 lb)

      - Temperature

        - Operating : -10 °C ~ 55 °C(-14°F ~ +131°F)

        - Storage : -40°C ~ +60°C(-33°F ~ +140°F)

      - Humidity

        - Operating : Less than 90% RH (non-condensing)

        - Storage : Less than 90% RH

    - EMC & Safety

      - FCC 47 CFR Part 15 Subpart B

        - ANSI C63.4-2017 Class A

      - IC Regulation ICES-003 Issue7

        - ANSI C63.4-2017 Class A

      - CE EMC-Directive 2014/30/EU

        - EN 55032:2015/A11:2020

        - EN 50130-4:2011

        - EN 610003-2:2014

        - EN 610003-3:2013

      - VCCI-CISPR 32:2016 Class A

      - AS/NZS CISPR32:2015 AMD 1:2020 Class A

      - UL/CSA 62368-1

      - EN IEC 63000:2018 (hazardous substances)

END OF SECTION

10