Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com, https://hanwhavisionamerica.com)

Model: XNF-A9014RV

Alternates: XNF-A8014RV, XNF-A9014R, XNF-A8014R

GENERAL DESCRIPTION

Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

H.265 and H.264

Original view: Max. 30fps(60Hz)/25fps(50Hz) @ 3008x3008, Max. 20fps in PoE mode

Double panorama: Max. 30fps(60Hz)/25fps(50Hz) @ 3584x1792, Max. 20fps in PoE mode

Single panorama: Max. 30fps(60Hz)/25fps(50Hz) @ 3584x896, Max. 20fps in PoE mode

Quad view: Max. 30fps(60Hz)/25fps(50Hz) @ 3584x2688, Max. 20fps in PoE mode

Q1/Q2/Q3/Q4(4:3): Max. 30fps(60Hz)/25fps(50Hz) @ 1792x1344, Max. 20fps in PoE mode

MJPEG – Max. 5fps

The camera shall be able to configure up to 5 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

The camera shall be able to configure various resolution selections.

The camera shall support multicast and unicast video streaming up to 20 users.

The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

The camera shall provide smart codec(WiseStream Ⅲ: Based on AI engine) to efficiently manage bit rate of the video stream and reduce storage while producing video quality that is visually equal to the one without smart codec.

Camera – The camera device shall have the following physical and performance properties:

The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 120dB.

Automatic, manual, scheduled day and night operation. Images are available in color or black and white.

Low light level operation to 0.02Lux (F1.8, 1/30sec) in color mode

Low light level operation to 0.001Lux (F1.8, 1/30sec) in b/w mode

The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

Configurable 32ea privacy masking regions utilizing 8 points polygonal zones.

Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

Classified object type: Person/Vehicle

Attributes: Person (Upper/lower clothes color), Vehicle(color)

Re-ID: Person

Support BestShot

Analytics events based on AI engine: Motion detection, Object detection, Virtual line (Crossing/Direction), Virtual area(Loitering/Intrusion/Enter/Exit/Appear/Disappear)

Analytics events: Defocus detection, Tampering, Audio detection, Shock detection

Business Intelligence (Based on AI engine): People counting, Queue management, Heatmap, Vehicle counting, Crowd counting

Interoperability – The camera shall be ONVIF Profile S, G, T and M compliant.

The camera shall possess the following further characteristics:

Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

NAS recording option with configurable pre-alarm and post-alarm recording intervals

Alarms and notifications

Alarm notification triggers: Video & Audio analytics, Network disconnect, Alarm input, Time schedule, MQTT subscription, Case tampering, Day/Night, Storage disruption

Available notification means upon trigger: File upload (image) via e-mail/FTP/SFTP, File upload (video clip) via FTP/SFTP, Notification via e-mail, Local storage (SD/SDHC/SDXC) or NAS recording at event triggers, Alarm output, PTZ preset, Send message by HTTP/HTTPS/TCP, Audio clip playback, MQTT publication

NDAA Compliant

PoE capable

Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings

Webpage Language: English, Korean, Simplified Chinese, Traditional Chinese, French, Italian, Spanish, German, Japanese, Russian, Portuguese, Czech, Polish, Turkish, Dutch, Hungarian, Greek

The camera features an impact-resistant dome constructed of ruggedized materials, certified to IK10 rating, providing protection against vandalism and physical attacks.

DETAILED SPECIFICATIONS

Video

Imaging Device: 1/1.6" 12MP CMOS

Resolution

3584x2688, 3584x1792, 3584x896, 3536x3536, 3008x3008, 2560x2560, 2560x1920,2560x1280, 2560x640, 1920x960, 1920x480, 1792x1344, 1600x1600, 1280x1280,1280x960, 1280x640, 1280x320, 1024x768, 1024x512, 1024x256, 800x800, 800x600,640x640, 640x480, 640x320, 640x160, 480x480, 320x240, 160x160

Maximum Framerate

H.265/H.264: 30fps/25fps (60Hz/50Hz)

MJPEG: Max.5fps

Others: Max 20fps in PoE mode

Minimum Illumination

Color: 0.02Lux (F1.8, 1/30sec)

B/W: 0.001Lux (F1.8, 1/30sec)

Video Out (Installation): Micro USB Type B, 1280x720 for installation

Lens

Focal Length: 1.76mm

Max. Aperture Ratio(F number)

Wide: F1.8

Tele: F1.8

Angular Field of View

Horizontal: 185˚

Vertical:  185°

Diagonal: 185°

Min. Object Distance: 0.5m (1.64ft)

Pan / Tilt / Rotate

Pan / Tilt / Rotate Range: Digital Rotation

Operational

Camera Title: Off / On (Displayed up to 85 characters)

Day/Night: Auto(ICR)

Backlight Compensation: BLC / HLC / WDR / SSDR

Wide Dynamic Range: 120dB

Digital Noise Reduction: SSNR V, WiseNRⅡ(Based on AI engine)

Digital Image Stabilization: Support

Defog: Support

Motion Detection

Quantity: 8ea

Shape: 8 points polygonal zones

Privacy Masking

Quantity: 32ea

Shape: 8 points polygonal zones

Color / Option: Gray, Green, Red, Blue, Black, White, Mosaic, Purple, Yellow

Gain control: Off / Max gain/ Manual

White Balance: ATW / narrow ATW / AWC / Manual / Indoor / Outdoor

Electronic Shutter

Mode: Minimum / Maximum / Anti-flicker / Prefer shutter control(based on AI engine)

Speed Range: 2 ~ 1/30,000sec

Digital PTZ: support

Video Rotation: Flip, Mirror

Alarm I/O

Input/Output: 2 configurable I/O ports

Others: Additional alarm I/O supported when connected to I/O box

Alarm Triggers: Analytics, Network disconnect, Alarm input, Time schedule, MQTT subscription, Case tampering, Day/Night, Storage disruption

Alarm Events When Alarm Trigger Occurred

File upload(image): e-mail/FTP/SFTP

File upload(video clip): FTP/SFTP

Notification: e-mail

Recording: SD/SDHC/SDXC or NAS recording at event triggers, Alarm output

Handover: PTZ preset, Send message by HTTP/HTTPS/TCP, Audio clip playback

MQTT: publication

Audio In: Selectable(mic in / line in), Built-in 3 mics

Audio Out: Line out

Light Type: IR LED

Light Viewable Length: WiseIR up to 20m(65.61ft)

IR Wavelength: 850nm

Analytics

Classified object type: Person/Vehicle

Attributes

Person: Upper/lower clothes color

Vehicle: Color

Re-ID: Person

Thumbnail: BestShot

Analytics events

Based on AI engine: Object detection, Virtual line(Crossing/Direction), Motion detection, Virtual area(Loitering/Intrusion/Enter/Exit/Appear/Disappear)

Normal: Defocus detection, Tampering, Audio detection, Shock detection

Business Intelligence

Based on AI engine: People counting, Queue management, Heatmap, Vehicle counting, Crowd counting

Network

Ethernet: Metal shielded RJ-45 (10/100 Base-T)

Video Compression: H.265 / H.264: Main / Baseline / High, MJPEG

Audio Compression

Opus 8/16kHz

G.711(PCM): 8kHz(64kbps)

G.726(ADPCM): 8kHz(16/24/32/40kbps)

OPUS: 16kHz(64kbps)

AAC-LC: 16/32/44.1/48kHz(48kbps)

OPUS: 48kHz(64kbps)

Smart Codec: WiseStream (Based on AI engine)

Bitrate Control:

H.265 / H.264: CBR or VBR

MJPEG: VBR

Streaming

Unicast: Up to 20 users

Multicast: Support

Multiple Streaming: Up to 5 profiles

Protocol: IPv4, IPv6, TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL/TLS, DHCP, FTP, SFTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, UPnP, Bonjour, LLDP, CDP, SRTP(TCP, UDP Unicast), MQTT, SMTP(StartTLS), Syslog

Application Programming Interface

ONVIF: Profile S, G, T, M

Others: SUNAPI (HTTP API), Hanwha Vision Open Platform

Direct Cloud Connection: Support

Security

OS / Firmware Protect: Secure boot, Signed Firmware, Firmware encryption

User authentication: Digest Authentication, Prevent brute-force attack

Network authentication: IEEE 802.1X Authentication (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

Secure Communication: HTTPS, WSS (WebSocket Secure), SRTP, 802.1AE(MACsec)

Access Control: IP-based access control, MAC-based access control, Auto logout

Data Protect: Encryption credentials, Encrypt compress for live recording file export

Audit: User Access / System / Event Log management

Device ID: Device certificate (Hanwha Vision Root CA)

Secure Storage: SD card partition encrypt, Secure element

Supply Chain Security: SBOM

Security Certificate: Secure element (FIPS 140-3 level3, CC EAL6+), Consumer IoT Security Standard (ETSIEN 303 645)

General

SoC

Type: Wisenet 9

Dual NPU: Support

Memory

RAM: 4GB

Flash: 8GB eMMC

Edge Storage

Micro SD/SDHC/SDXC: Max 1TB x 1slot

Edge Storage (OnCloud)

Micro SD/SDHC/SDXC: Max 1TB x 1slot

Hanwha Vision branded microSD cards have been certified and recommended for OnCloud edge recording

Environmental & Electrical

Operating Condition

Temperature: -40°C to 60°C (-40°F to 140°F)

Humidity: Less than 100% RH(condensing)

In PoE+ mode, IR illuminator will turn off if the temperature is higher than 50°C(122°F)

Storage Condition

Temperature: -50°C to 60°C (-58°F to 140°F)

Humidity: Less than 95% RH(non-condensing)

Input Voltage: PoE (IEEE 802.3af, class3), PoE+ (IEEE 802.3at, class4), 12V DC

Power Consumption

PoE: Max 19.8W, typical 10.5W

12V DC: Max 20W, typical 10.9W

WisePower: Power mode, Power monitoring

Power Redundance Failover: Support

Mechanical

Color: White

Material: Aluminum, Hard-coated dome

Material Management

Declaration: RoHS(EU RoHS directive 2011/65/EU and IEC/EN 63000:2018)

RAL Code: RAL9003

Product Dimensions (D x H): Ø160 x 75mm (ø6.23x2.95")

Product Weight: 1,020g(2.25lb)

Compatible Gang Box: Single, Double 4" Octagon, Square

Certifications & Standards

EMC

EN 50121-4, FCC 47 CFR 15 Subpart B Class A, ICES-3(A)/NMB-3(A), CE/UKCA - EN55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3, VCCI CISPR 32 Class A, RCM AS-NZS CISPR 32 Class A, KS C 9832 Class A, KS C 9835, EN 50121-3-2

Safety

UL 62368-1, CAN/CSA C22.2 NO. 62368-1, IEC 62471

Environment

IEC/EN 63000, IEC 60529 IP66, IEC 62262 IK10, NEMA TS 2-2013 (2.2.7, 2.2.8, 2.2.9), NEMA 250 type 4X, EN 50155, IEC 62236-4, EN 50121-4, EN 50498, IEC/EN 61373, EN 45545, IEC 62236-3-2, IEC 60068-2-1, IEC 60068-2-2, IEC 60068-2-6, IEC 60068-2-14, IEC 60068-2-27, IEC 60068-2-78

DORI Distance (EN62676-4 standard)

Detect (25PPM/ 8PPF):	Wide 31.4m(102.97ft)

Observe (63PPM/ 19PPF): Wide 17.3m (56.89ft)

Recognize (125PPM/ 38PPF): Wide 8.7m (28.45ft)

Identify (250PPM/ 76PPF): Wide 4.3m (14.22ft)

Compatible Devices

Hanging Adaptor: SBP-160HMW1

Back Box: SBV-180BW, SBV-180WW

Ceiling Mount (Assy): SBP-150CMI, SBP-300CMI, SBP-300CMW1, SBP-900CMW, SBP-300CMTW, SBP-300CMTS

Ceiling Mount (Single Unit): SBP-180CMB, SBP-140CMB, SBP-180CMS, SBP-900CMP, SBP-300CMP, SBP-150CMP, SBP-C35H, SBP-C15H, SBP-C15P, SBP-C15NP

Wall Mount: SBP-160WMW1, SBP-400WMW, SBP-250WMW, SBP-300WMW, SBP-300WMW1

Pole Mount: SBD-180PMW, SBP-300PMW2, SBP-150PMW

In-ceiling Mount: SHD-1600FPW, SHD-1600FW

Corner Mount: SBP-156KMW, SBP-300KMW1

Parapet Mount: SBP-156LMW1, SBP-300LMW

Tilt Mount: SBP-160TMW1

Cabinet: SBP-150NBW, SBP-300NBW

Skin Cover: SBC-160BF

Other Compatible Models: SPP-C7400, SBP-140CMT, SBP-115PFA, M12 cable (name TBD)

END OF SECTION

8