MODEL : 2M H.265 네트워크 VANDAL DOME 카메라(XNV-6010)

1. 개 요

XNV-6010은 자체개발칩 WN5를 내장하여 저조도 특성이 향상되었으며,

WDR 및 지능형 영상 분석이 새롭게 적용된 고화질 네트워크 VANDAL DOME 카메라입니다.

2. 제 원

1) 촬상소자    : 1/2.8형 2메가픽셀 CMOS (총화소1945x1109, 유효화소1945x1097)/SN비 50dB

2) 최저조도    : 칼라 0.055Lux(F2.0 1/30sec), 흑백 0.0055Lux(F2.0 1/30sec)

3) 렌즈        : 초점 2.4mm Fixed/최대구경비F2.0/화각H:139,V:73,D:167°/수동포커스

팬.틸트.회전 - 0° ~ 354° / 0° ~ 67° / 0° ~ 355°(수동조정)

4) 보정        : 역광보정 HLC, BLC, WDR(150dB), 노이즈 제거(SSDR,SSNR), DIS, Defog지원

5) LDC         : 지원 / Digital PTZ 24X 지원

6) 분석        : Tampering, Loitering, Directional Detection, Defocus Detection,

Fog&Dust Detection, Virtual Line, Enter/Exit, Appear / Disappear,

Audio Detection, Face Detection, Motion Detection,

Digital Auto Tracking, Sound Classification, People counting,

Heat map, Queue management

7) 알람        : 알람입출력(입력1개,출력1개), 알람트리거(동작탐지,비디오.오디오분석등)

8) 오디오      : G.711, G.726 / 양방향

9) 네트워크    : 프로토콜 - TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP,

HTTPS, SSL/TLS,DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3

(MIB-2), ARP, DNS, DDNS,QoS, PIM-SM, UPnP, Bonjour

IP-IPv4, IPv6(전송방식-유니캐스트/멀티캐스트)

10) 웹뷰어      : 지원 OS : Windows 7 / 8.1 / 10, MAC OS X 10.10, 10.11, 10.12

플러그인 프리 웹뷰어 지원 브라우저 : Google Chrome 54, MS Edge 38,

Mozilla Firefox 49(Window 64bit only), Apple Safari 9 (Mac OS X only)

플러그인 지원 브라우저 : MS Explore 11, Apple Safari 9 (Mac only)

11) 동작/온습도 : -30°C ~ +55°C / 90% RH 이하

12) 방진/방수   : IP67,IP66

13) 내충격 대응 : IK10

14) 전원        : DC12V, PoE(IEEE802.3af, Class3), 최대 5.5W(12VDC), 최대 6W(PoE)

15) 외관        : Ivory / Metal, Ø120 x H97.5mm, 365g

http://www.hanwha-security.com/