Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

PRODUCTS

EQUIPMENT

Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

Model: XNV-A9016R

Alternates: XNV-A8016R, XNV-A6016R

GENERAL DESCRIPTION

Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously. (See detailed specifications)

The camera shall be able to configure various resolution selections. (See detailed specifications)

The camera shall be able to configure independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings

The camera shall support unicast video streaming up to 20 users.

The camera shall support multiple streaming up to 5 profiles.

The camera shall support multicast video streaming.

The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

The camera shall provide WiseStream based on AI engine to efficiently manage bit rate of the video stream and reduce storage.

The camera shall provide WiseNRⅡ, an AI-based feature to reduce noise and blur on image.

Camera – The camera device shall have the following physical and performance properties.

True day/night operation with removable IR cut filter. The IR function shall provide 25m(82.02ft) viewable distance with 850nm IR LED.

The camera shall be able to produce clear images in high-contrast scenes with wide dynamic range up to 120dB.

The camera shall support digital noise reduction using WiseNRⅡ based on AI engine and SSNRⅤ.

The camera shall be able to configure privacy masking areas with polygonal zones.

The camera shall provide USB Type C video out for preview and installation, requiring a WiFi dongle.

The camera shall be powered by Wisenet 9 and shall provide Dual NPU for separated AI processing and image enhancement.

Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

Classified object type: Person / Face / Vehicle / License plate

Attributes: Person (Gender, Clothing top/bottom color, Bag), Face (Age, Gender, Mask, Glasses), Vehicle (Type: car / bus / truck / motorcycle / bicycle, Color)

BestShot supported

Stopped vehicle detection supported

Analytics events based on AI engine: Motion detection, Object detection, Virtual line(Crossing / Direction), Virtual area(Loitering / Intrusion / Enter / Exit / Appear / Disappear), Slip & Fall detection, Face mask detection, Social distancing detection

Analytics events: Defocus detection, Tampering, Audio detection

Business Intelligence: People / Vehicle / Crowd counting, Queue management, Heatmap based on AI engine

The camera shall provide Dynamic Privacy Masking with selectable mosaic or color masking options based on AI engine.

Interoperability

The camera shall be ONVIF Profile S, G, T and M compliant.

The camera shall support direct cloud connection and Hanwha Vision Open Platform.

The camera shall support Hanwha Vision Open Platform for extensibility.

Additional Features

Micro SD/SDHC/SDXC memory slot supporting up to 1TB x 1 slot

NAS recording option with configurable event recording support

Alarm triggers and notifications

Impact protection (IK10), Water & Dust protection (IP66)

Support WisePower (Power monitoring & Eco mode)

The camera uses recycled plastic in construction

Device certificate (Hanwha Vision Root CA), Secure storage (Secure element with FIPS 140-3 level 3/CC EAL6+ certified), SBOM

The camera shall feature an admin session timeout to prevent a webpage user from leaving access to the camera for longer than a specified period.

DETAILED SPECIFICATIONS

Video

Imaging device: 1/2.8" CMOS

Resolution: 3840x2160, 3328x1872, 3072x1728, 2560x1440, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240, 320x180

Maximum Framerate

H.265/H.264: Max. 30fps(60Hz) / 25fps(50Hz)

MJPEG: Max. 30fps (at 8MP Max. 5fps)

Minimum Illumination

Color: 0.1Lux (F1.6, 1/30sec, 30IRE)

B/W: 0.01Lux (F1.6, 1/30sec, 30IRE)

Video Out (for Installation): USB Type C, preview and installation only, requires WiFi dongle

Lens

Focal length: 3.0mm fixed lens

Max. Aperture Ratio: F1.6

Field of View

H:108°

V:59°

D:127°

Min. Object Distance: 0.4m (1.31ft)

Focus Control: Manual

Lens Type: Fixed iris

Pan & Tilt & Rotate

Range: 0°~351° / 0°~64° / ±90°

Operational Functions

Camera Title: Displayed up to 85 characters

Day/Night: Auto (ICR)

Backlight Compensation: BLC / HLC / WDR / SSDR

WDR(Wide Dynamic Range): extremeWDR 120dB

Digital Noise Reduction: SSNRⅤ, WiseNRⅡ(AI-based)

Digital Image Stabilization: Supported

Defog: Supported

Motion Detection: 8 zones, each with 8-point polygonal shape

Privacy Masking

32 zones, polygonal shape

Color: gray/green/red/blue/black/white/purple/yellow

Mosaic

Dynamic Privacy Mask(Based on AI engine)

Color: gray/green/red/blue/black/white/purple/yellow

Mosaic

Gain Control: Off / Max Gain / Manual

White Balance: ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor

Lens Distortion Correction: Supported (Fill / Stretch mode)

Electronic Shutter Speed

Min / Max / Anti-flicker (2 ~ 1/30,000sec)

Prefer shutter control (based on AI engine)

Video Rotation: Flip, Mirror, Hallway view (90° / 270°)

Analytics

Classified object type: Person / Face / Vehicle / License plate

Attributes: Person (Gender, Clothing top/bottom color, Bag), Face (Age, Gender, Mask, Glasses), Vehicle (Type: car / bus / truck / motorcycle / bicycle, Color)

BestShot supported

Stopped vehicle detection supported

AI-based analytics events: Motion detection, Object detection, Virtual line(Crossing / Direction), Virtual area(Loitering / Intrusion / Enter / Exit / Appear / Disappear), Slip & Fall detection, Face mask detection, Social distancing detection

Analytics events: Defocus detection, Tampering, Audio detection

Business Intelligence: People / Vehicle / Crowd counting, Queue management, Heatmap based on AI engine

Alarm I/O: Support alarm I/O via optional I/O box

Alarm Triggers: Analytics, Network disconnection, Alarm input, App event, Time schedule, MQTT subscription, Day/Night, Storage disruption

Alarm Events (When alarm trigger occurred)

File upload

image: e-mail/FTP/SFTP

video clip: FTP/SFTP

Notification: e-mail

Recording: SD/SDHC/SDXC or NAS recording at event triggers

Alarm output

Handover: PTZ preset, Send message by HTTP / HTTPS / TCP

Audio clip playback

MQTT: publication

Audio in: Support via optional I/O box

Audio out: Support via optional I/O box

Light Type: IR LED (850nm)

IR Viewable Length: 25m(82.02ft)

Edge Storage

Micro SD/SDHC/SDXC 1TB (1TB x 1 slot), NAS support

Memory: 4GB RAM, 8GB eMMC

Webpage Language: English, Korean, Simplified Chinese, Traditional Chinese, French, Italian, Spanish, German, Japanese, Russian, Portuguese, Czech, Polish, Turkish, Dutch, Hungarian, Greek

Network

Ethernet: Metal shielded RJ-45 (10/100 BASE-T)

Video compression

H.265/H.264: Main / Baseline / High

MJPEG

Smart Codec: Manual, WiseStream (Based on AI engine)

Bitrate Control

H.265 / H.264: CBR or VBR

MJPEG: VBR

Streaming

Unicast(20 users) / Multicast

Multiple streaming (Up to 5 profiles)

Profile set: Up to 5 profiles

Application Programming Interface

ONVIF Profiles S, G, T, M

SUNAPI (HTTP API)

Hanwha Vision Open Platform

Protocol

Network: IPv4, IPv6, TCP/IP, UDP/IP

Streaming: RTP(UDP), RTP(TCP), RTCP, RTSP, SRTP

Services: NTP, HTTP, HTTPS, SSL/TLS, DHCP, DNS, DDNS

File Transfer: FTP/SFTP, SMTP(StartTLS)

Management: SNMPv1/v2c/v3(MIB-2), IGMP, UPnP, Bonjour, LLDP

Other: ICMP, ARP, QoS, CDP, MQTT, Syslog

DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

QoS – Layer 3 DSCP

Security Feature

OS / Firmware Protect: Encrypted Firmware, Secure boot, Signed Firmware

User authentication: Digest Authentication, Prevent brute-force attack

Network authentication: IEEE 802.1X (EAP-TLS, EAP-LEAP, EAP-PEAP, MSCHAPv2)

Secure Communication: HTTPS, SRTP, WSS (WebSocket Secure), 802.1AE(MACsec)

Access Control: IP-based access control, MAC-based access control, Auto logout

Data Protect: Encryption Credentials, Encrypt compress for live recording file export

Audit: Access / System / Event Log management

Device ID: Device certificate (Hanwha Vision Root CA)

Secure Storage: Secure element, SD card partition encrypt

Security Certificate: ETSI EN 303 645, Secure element (FIPS 140-3 level 3, CC EAL6+)

The device shall not provide a manufacturer default password. Default password change shall be required to access the camera.

A minimal level of password complexity shall be required by the camera.

The camera shall not have a manufacturer back-door password.

The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

Network filtering – The camera shall allow a list of allowed or blocked IP addresses/subnet mask and/or MAC addresses to be entered to restrict access to the device.

Discovery – The manufacturer shall offer a discovery program to identify all their devices on the network.

Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

Environmental & Electrical

Operating Condition

Temperature

-40°C to +50°C (-40°F to +122°F)

+74°C (+165°F)(Max) based on NEMA-TS 2(2.2.7)

Startup should be done at above -30°C (-22°F)

Humidity

0 ~ 100% RH(Condensing)

Storage Condition

Temperature

-50°C to +60°C (-58°F to +140°F)

Humidity

0 ~ 90% RH(non-condensing)

Input Voltage: PoE (IEEE802.3af, class 3)

Power Consumption: Max. 10.0W, typical 7.7W

WisePower: Power monitoring, Eco mode

Mechanical

Color: White

Material: Aluminum

Bubble: Hard-coated dome

RAL Code: RAL9003

Dimensions (Width x Height): ø120 x 65.5mm (ø4.72 x 2.58")

Weight: 555g(1.22lb)

Certifications & Standards

EMC

ICES-3(A)/NMB-3(A)

CE/UKCA: EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

VCCI CISPR 32 Class A

RCM AS/NZS CISPR 32 Class A

KS C 9832 Class A, KS C9835

Environment

IEC/EN 63000

IEC 60529 IP66, IEC 62262 IK10, NEMA 250 Type 4X

IEC 60068-2-1, IEC 60068-2-2, IEC 60068-2-6, IEC 60068-2-14, IEC 60068-2-27, IEC 60068-2-78

Compatible Models

Hanging Adaptor: SBP-120HMW

Back Box: SBD-110GP1

Ceiling Mount (Assy): SBP-140CMB, SBP-180CMB, SBP-180CMS, SBP-150CMP, SBP-300CMP, SBP-900CMP

Ceiling Mount (Single Unit): SBP-C15H, SBP-C15P, SBP-150CMI, SBP-300CMI, SBP-300CMW1, SBP-900CMW, SBP-300CMTW, SBP-300CMTS

Wall Mount: SBP-125WMW1, SBP-200WMW, SBP-250WMW, SBP-300WMW, SBP-400WMW

Pole Mount: SBV-140TMW

In-ceiling Mount: SBP-050NBW

Corner Mount: SBV-140BW

Parapet Mount: SBV-140WW

Housing: SBD-140PMW, SBD-140PMB, SBD-140KMB

Power Module: SPB-VAN68W

Interface Box: SPM-4210

Other Compatible Models: SBP-115PFA, SBP-156KMW, SBP-156LMW1, SBV-122WCW

DORI Distance (EN62676-4 standard)

Detect (25PPM/ 8PPF): Wide: 55.8m(183.07ft)

Observe (63PPM/ 19PPF): Wide: 22.3m(73.23ft)

Recognize (125PPM/ 38PPF): Wide: 11.2m(36.61ft)

Identify (250PPM/ 76PPF): Wide: 5.6m(18.31ft)

END OF SECTION

10